3GPP TSG-RAN WG4 Meeting#98-bis-e                                 R4-2106599

E-meeting,12th April– 20th April, 2021

Agenda item:	5.3.2.3.2

Source: 	ZTE Corporation

Title: 	TP to TS 38.xxx-1:  RX IMD requirements

Document for:	Approval

Introduction

In the past RAN4#98e meetings, work split has been agreed among companies, therefore in this contribution, we want to share the draft TP for section 7.7 Rx IMD test for further discussion.

Reference

- [1] R4-2103856	WF on IAB conformance specification work split and drafting guidelines, approved.

Annex

- <Start of TP>

7.7	Receiver intermodulation

7.7.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for TAB connector for IAB type 1-H in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

7.7.2	Minimum requirement

The minimum requirements for IAB-DU type 1-H are in TS 38.174 [2], clause 7.7.2.

The minimum requirements for IAB-MT type 1-H are in TS 38.174 [2], clause 7.7.3.

7.7.3	Test purpose

The test purpose is to verify the ability of the IAB receiver to inhibit the generation of intermodulation products in its non-linear elements caused by the presence of two high-level interfering signals at frequencies with a specific relationship to the frequency of the wanted signal.

7.7.4	Method of test

7.7.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see clause 4.9.1

IAB RF Bandwidth positions to be tested for multi-carrier (MC) and/or CA:

-	MRFBW for single-band connector(s), see clause 4.9.1,

-	BRFBW_T'RFBW and B'RFBW_TRFBW for multi-band connector(s), see clause 4.9.1.

NOTE:	When testing in M (or MRFBW), if the interferer is fully or partially located outside the supported frequency range, then the test shall be done instead in B (or BRFBW) and T (or TRFBW), and only with the interferer located inside the supported frequency range.

7.7.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For IAB type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.4.6 for IAB type 1-H.

2)	Set the IAB-DU type 1-H to transmit:

-	For single carrier operation set the connector under test to transmit at manufacturers declared rated carrier output power (Prated,c,AC or Prated,c,TABC, D.21).

-	For a connector under test declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

2a)	Set the IAB-MT type 1-H to transmit:

-	For single carrier operation set the connector under test to transmit at manufacturers declared rated carrier output power (Prated,c,AC or Prated,c,TABC, D.21).

-	For a connector under test declared to be capable of multi-carrier and/or CA operation (D.15-D.16) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.x.x.

3)	Set the signal generator for the wanted signal to transmit as specified in table 7.7.5-1 and 7.7.5-3.

4)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.7.5-2 and 7.7.5-4.

5)	Measure the throughput according to annex A.1.

In addition, for a multi-band connector, the following steps shall apply:

6)	For multi-band connector and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

7.7.5	Test requirements

7.7.5.1 IAB-DU type 1-H

The Wide Area IAB-DU receiver intermodulation test requirement is specified the same as the Wide Area receiver intermodulation test requirement for BS type 1-H in TS 38.141-1[x], subclause 7.7.5, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The Medium Range IAB-DU receiver intermodulation test requirement is specified the same as the Medium Range BS  receiver intermodulation test requirement for BS type 1-H in TS 38.141-1[x], subclause 7.7.5, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

The Local Area IAB-DU receiver intermodulation test requirement is specified the same as the Local Area BS  receiver intermodulation test requirement for BS type 1-H in TS 38.141-1[x], subclause 7.7.5, where references to BS channel bandwidth apply to IAB-DU channel bandwidth.

Referenced requirements applying to NB-IoT are not applicable to the IAB-DU.

7.7.5.2 IAB-MT type 1-H

The Wide Aarea IAB-MT receiver intermodulation test requirement is specified the same as the Wide Area receiver intermodulation test requirement for BS type 1-H in TS 38.141-1[x], subclause 7.7.5, where references to BS channel bandwidth apply to IAB-MT channel bandwidth.

The Local Area IAB-MT receiver intermodulation test requirement is specified the same as theLocal Area BS receiver intermodulation test requirement for BS type 1-H in TS 38.141-1[x], subclause 7.7.5, where references to BS channel bandwidth apply to IAB-MT channel bandwidth.

- Interfering signal for IAB-MT type 1-H should be CP-OFDM.

- <End of TP>