3GPP TSG RAN WG4 Meeting #98-bis-e	R4-2106633

Electronic Meeting, April 12 – 20, 2021

Agenda item:			5.5.2.2.2.2

Source:	vivo

Title:	Discussion on PRS-RSRP accuracy requirements

Document for:	Discussion

- Introduction

In the RAN4#98-e meeting, there were extensive discussions on PRS-RSRP measurement accuracy requirements. Following agreements were made during the meeting as captured in [1].

- PRS-RSRP SINR side condition #1 is [-3] dB. Companies are encouraged to bring simulation results for -3dB and -6dB SINR conditions. - The full set of parameters and values used for PRS RSRP simulations were included in R4-21xxxxx. - In the next meeting, the exact accuracy requirements can be defined from the simulation results, based on the parameter sets listed in R4-2104046 - The simulation results can still be updated in the next meeting - Depending on the final required accuracy numbers, not all parameters from R4-2104046 need to be present in the final accuracy requirements

There are remaining open issues to be further resolved.

- RF calibration margin

- Structure for accuracy requirements

In this contribution, we provide our views on open issues of PRS-RSRP measurement accuracy requirements.

# Discussion

## 2.1 RF calibration margin

- RF calibration margin: FFS - Option 1: The RF margin for PRS-RSRP accuracy is defined as - 2.5dB for FR1 absolute accuracy requirements - 4dB for FR2 absolute accuracy requirements - 0dB for FR1 relative accuracy requirements - 0dB for FR2 relative accuracy requirements, provided that two PRS-RSRP are measured with the same Rx beam

In our view the propsed RF margin is reasonable to define absolute accuracy requirements and relative accuracy requirements for PRS-RSRP measurement.

Proposal 1: RF margin as in option 1 is used to define PRS-RSRP absolute and relative accuracy requirements.

## 2.2 Structure for accuracy requirements

Following parameters are considered to define accuracy requirements for PRS based measurement as agreed in [1].

  - PRS BW defined in number of PRBs

  - PRS SCS

  - dl-PRS-ResourceRepetitionFactor

  - dl-PRS-NumSymbols

  - dl-PRS-CombSizeN

  - Other parameters are FFS

Simulation assumptions in [2] are also based on above parameters. PRS-RSRP measurement accuracy are not much dependent on PRS BW, but there is accuracy improvement with lager PRS BW. So as agreed in the last meeting, all the PRS BW and PRS SCS as in [2] would be considered when defining accuracy requirements.

The parameters of dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN will determine how many repetions of a PRS resource of same number for {comb size, number of PRS symbols}. It can be catergorized as total number of repetitions of a PRS resource. Based on our simulation results in [3], the PRS-RSRP measurement accuracy will be improved if total number of repetitions is larger than 4. We don’t think it is meaningful to define accuracy requirements for different number of repetitions. It would be enough to define requirements based on total number of repetitions of 4.

Based on the above analysis, it is proposed to define PRS-RSRP measurement accuracy requirements as in Table 1 and Table 2 for FR1 and FR2 respectively.

Proposal 2: PRS-RSRP measurement accuracy rquirements are specified as in Table 1 and Table 2 for FR1 and FR2 respectively.

Table 1: PRS-RSRP measurement accuracy in FR1

| Accuracy | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | Total number of repetitions of PRS resource |
| --- | --- | --- | --- | --- |
| dB | dB | RB | kHz |  |
|  | -3 | 24 | 15kHz | 1 |
|  | -3 | 24 | 15kHz | 4 |
|  | -3 | 52 | 15kHz | 1 |
|  | -3 | 52 | 15kHz | 4 |
|  | -3 | 104 | 15kHz | 1 |
|  | -3 | 104 | 15kHz | 4 |
|  | -3 | 268 | 15kHz | 1 |
|  | -3 | 268 | 15kHz | 4 |
|  | -3 | 48 | 30kHz | 1 |
|  | -3 | 48 | 30kHz | 4 |
|  | -3 | 132 | 30kHz | 1 |
|  | -3 | 132 | 30kHz | 4 |
|  | -3 | 272 | 30kHz | 1 |
|  | -3 | 272 | 30kHz | 4 |
|  | -13 | 24 | 15kHz | 1 |
|  | -13 | 24 | 15kHz | 4 |
|  | -13 | 52 | 15kHz | 1 |
|  | -13 | 52 | 15kHz | 4 |
|  | -13 | 104 | 15kHz | 1 |
|  | -13 | 104 | 15kHz | 4 |
|  | -13 | 268 | 15kHz | 1 |
|  | -13 | 268 | 15kHz | 4 |
|  | -13 | 48 | 30kHz | 1 |
|  | -13 | 48 | 30kHz | 4 |
|  | -13 | 132 | 30kHz | 1 |
|  | -13 | 132 | 30kHz | 4 |
|  | -13 | 272 | 30kHz | 1 |
|  | -13 | 272 | 30kHz | 4 |
|  | -13 | 24 | 15kHz | 1 |

Table 2: PRS-RSRP measurement accuracy in FR2

| Accuracy | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | Total number of repetitions of PRS resource |
| --- | --- | --- | --- | --- |
| dB | dB | RB | kHz |  |
|  | -3 | 32 | 120kHz | 1 |
|  | -3 | 32 | 120kHz | 4 |
|  | -3 | 64 | 120kHz | 1 |
|  | -3 | 64 | 120kHz | 4 |
|  | -3 | 128 | 120kHz | 1 |
|  | -3 | 128 | 120kHz | 4 |
|  | -13 | 32 | 120kHz | 1 |
|  | -13 | 32 | 120kHz | 4 |
|  | -13 | 64 | 120kHz | 1 |
|  | -13 | 64 | 120kHz | 4 |
|  | -13 | 128 | 120kHz | 1 |
|  | -13 | 128 | 120kHz | 4 |

In addition, the parameters in Table 1 and Table 2 can be further simplified or categorized based on collection of simulation results from companies. Some parameter combinations would have similar accuracy requirement which can be grouped together.

Proposal 3: Parameters in Table 1 and Table 2 can be further simplified or grouped based on agreements on accuracy.

# Summary

In this contribution we provide our views on PRS-RSTD measurement accuracy requirements. Based on analysis following proposals are present.

Proposal 1: RAN4 defines one set of requirements for RSTD measurement accuracy by taking into account calibration of timing between antenna panel and baseband.

Proposal 1: RF margin as in option 1 is used to define PRS-RSRP absolute and relative accuracy requirements.

Table 1: PRS-RSRP measurement accuracy in FR1

| Accuracy | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | Total number of repetitions of PRS resource |
| --- | --- | --- | --- | --- |
| dB | dB | RB | kHz |  |
|  | -3 | 24 | 15kHz | 1 |
|  | -3 | 24 | 15kHz | 4 |
|  | -3 | 52 | 15kHz | 1 |
|  | -3 | 52 | 15kHz | 4 |
|  | -3 | 104 | 15kHz | 1 |
|  | -3 | 104 | 15kHz | 4 |
|  | -3 | 268 | 15kHz | 1 |
|  | -3 | 268 | 15kHz | 4 |
|  | -3 | 48 | 30kHz | 1 |
|  | -3 | 48 | 30kHz | 4 |
|  | -3 | 132 | 30kHz | 1 |
|  | -3 | 132 | 30kHz | 4 |
|  | -3 | 272 | 30kHz | 1 |
|  | -3 | 272 | 30kHz | 4 |
|  | -13 | 24 | 15kHz | 1 |
|  | -13 | 24 | 15kHz | 4 |
|  | -13 | 52 | 15kHz | 1 |
|  | -13 | 52 | 15kHz | 4 |
|  | -13 | 104 | 15kHz | 1 |
|  | -13 | 104 | 15kHz | 4 |
|  | -13 | 268 | 15kHz | 1 |
|  | -13 | 268 | 15kHz | 4 |
|  | -13 | 48 | 30kHz | 1 |
|  | -13 | 48 | 30kHz | 4 |
|  | -13 | 132 | 30kHz | 1 |
|  | -13 | 132 | 30kHz | 4 |
|  | -13 | 272 | 30kHz | 1 |
|  | -13 | 272 | 30kHz | 4 |
|  | -13 | 24 | 15kHz | 1 |

Table 2: PRS-RSRP measurement accuracy in FR2

| Accuracy | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | Total number of repetitions of PRS resource |
| --- | --- | --- | --- | --- |
| dB | dB | RB | kHz |  |
|  | -3 | 32 | 120kHz | 1 |
|  | -3 | 32 | 120kHz | 4 |
|  | -3 | 64 | 120kHz | 1 |
|  | -3 | 64 | 120kHz | 4 |
|  | -3 | 128 | 120kHz | 1 |
|  | -3 | 128 | 120kHz | 4 |
|  | -13 | 32 | 120kHz | 1 |
|  | -13 | 32 | 120kHz | 4 |
|  | -13 | 64 | 120kHz | 1 |
|  | -13 | 64 | 120kHz | 4 |
|  | -13 | 128 | 120kHz | 1 |
|  | -13 | 128 | 120kHz | 4 |

Proposal 3: Parameters in Table 1 and Table 2 can be further simplified or grouped based on agreements on accuracy.

- References

- R4-2103586	WF on NR positioning UE performance requirements, Intel Corporation

- R4-2104092	Summary of link level simulation result of RSTD, PRS RSRP and UE DToA, Intel Corporation

- R4-2106636	Link level simulation results for PRS-RSP, vivo