3GPP TSG-RAN WG4 Meeting # 98bis-e                            R4-2106681

Electronic Meeting, 12– 20 April, 2021

Source: 	Huawei, HiSilicon

Title: 	General discussion on support of full bandwidth combinations for inter-band EN-DC

Agenda Item:	7.36.1

Document for:	Approval

Introduction

The WID [1] on Support of full bandwidth combinations for inter-band EN-DC was approved in RAN#91. The major objective for this WI is defined as follow.

- Specify the band combination specific EN-DC RF requirements, e.g. MSD exceptions, for all channel bandwidth combinations including the new CBWs introduced from Rel-16.

In this paper, we’d like to general discuss this topic so that the work related to MSD requirements can be organised orderly and clearly.

Discussion

Requirements for Rel-17

Based on the justification in the WI [1], some inter-band ENDC combinations which have MSD due to UL harmonic interference, receiver harmonic mixing or cross band isolation in TS 38.101-3 may be introduced into the specification firstly. Later, new NR channel bandwidths were introduced for the existing bands. Since full bandwidth combinations have already been used for inter-band ENDC combinations, it may result that some MSD requirements which were specified depending on the specific channel bandwidth are missing.

Observation 1: Since full bandwidth combinations have already been supported for inter-band ENDC combinations, the introduction of new NR channel bandwidths may result that some MSD requirements which were specified depending on the specific channel bandwidths are missing.

Since there is no general principle or method to know how to derive the MSD requirements, it’s very hard to derive these requirements when new NR channel bandwidths are introduced. And some mistakes can be found in the MSD requirements table. For example, the 2nd harmonic interference from UL band 1, 2, 3, 4 and 66 may fall into band n77, n78 and n48. Based on the discussion [2] in Rel-15, the -71.7dBm exceptional REFSENS for CA_3-42 was reused for DC_3_n78. In addition, the ΔRIB,c of 0.2 dB was considered in the MSD calculation. So the final MSD requirement for DL 10MHz can be calculated by the following equation

MSD (DL 10MHz) = exceptional REFSENS – REFSENS (DL 10MHz)               (1)

Thus, we can get 23.9dB = -71.7dBm - (-95.8dBm+0.2) for DL 10MHz.

Since NF for band n77 is assumed as 10.5dB, the REFSENS is -95.3dBm for band n77 DL 10MHz. If we follow the similar mechanism, the MSD (DL 10MHz) for DC_3_n77 should be 23.4dB.

The NF is assumed as 10dB for band n48 which has the same REFSENS with band n78. Following the same -71.7dBm exceptional REFSENS, the MSD (DL 10MHz) for DC_66_n48 should be 23.9dB which is same with DC_3_n78.

Observation 2: Some inconsistencies for the 2nd harmonic MSD exception can be observed among DC_3_n78, DC_3_n77 and DC_66_n48.

Table 1 Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 2 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  | 18 | 17.1 | 16.3 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 3 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 4 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 66 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  | 18 | 17.1 | 16.3 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |

As we discussed in the contribution [3], the exceptional REFSENS may be analysed using link budget. If the harmonic interference >> the thermal noise, the exceptional REFSENSs are same for all the channel bandwidths. Otherwise, the exceptional REFSENSs should be considered by both the harmonic interference and the thermal noise.

In order to solve these issues, one efficient alternative is to reconstruct the relative MSD requirements due to harmonic interference using equation-based representation. If we do so, it is helpful to reduce some duplicated work once new channel bandwidths are introduced. Besides, delegates and industry can be aware of how the MSD requirements are generated or calculated. Finally, some mistakes can be corrected and avoided.

Proposal 1: It’s proposed to reconstruct the relative MSD requirements due to harmonic interference using equation-based representation in Rel-17.

How to handle the missing MSD Requirements for Rel-16

As maintenances for Rel-16, there is no need to make major changes to the Rel-16 specification. The identified missing MSD requirements for Rel-16 can be completed using official CR and Rel-16 TEI work item code.

In this WI, some technical issues about MSD requirements can be discussed in this WI for Rel-16. Companies are encouraged to provide the draft CR and some analysis for Rel-16 missing MSD requirements.

Proposal 2: Some technical issues about MSD requirements can be discussed in this WI for Rel-16. Companies are encouraged to provide the draft CR and some analysis for Rel-16 missing MSD requirements. The identified missing MSD requirements for Rel-16 can be completed using an official CR per meeting and Rel-16 TEI work item code.

Conclusion

Based on the analysis and discussion, the proposal and observations are shown below.

Observation 1: Since full bandwidth combinations have already been supported for inter-band ENDC combinations, the introduction of new NR channel bandwidths may result that some MSD requirements which were specified depending on the specific channel bandwidths are missing.

Observation 2: Some inconsistencies for the 2nd harmonic MSD exception can be observed among DC_3_n78, DC_3_n77 and DC_66_n48.

Proposal 1: It’s proposed to reconstruct the relative MSD requirements due to harmonic interference using equation-based representation in Rel-17.

Proposal 2: Some technical issues about MSD requirements can be discussed in this WI for Rel-16. Companies are encouraged to provide the draft CR and some analysis for Rel-16 missing MSD requirements. The identified missing MSD requirements for Rel-16 can be completed using an official CR per meeting and Rel-16 TEI work item code.

- References

[1] RP-210891, “New WID: Support of full bandwidth combinations for inter-band EN-DC”, Huawei, HiSilicon

[2] R4-1712952, “On harmonic requirements for DC_3-n78”, Huawei, Hisilicon

[3] R4-2101816, “Discussion on how to simplify MSD definition using bandwidth-agnostic approach”, Huawei, HiSilicon