3GPP TSG-RAN WG4 Meeting # 98bis-e                            R4-2106684

Electronic Meeting, 12– 20 April, 2021

Source: 	Huawei, HiSilicon

Title: 	Further discussion on NTN simulation assumptions

Agenda Item:	8.8.2.1

Document for:	Approval

Introduction

In RAN4#98e, RAN4 made some progress on the NTN simulation assumptions [1]. In this paper, we’d like to further discuss NTN simulation assumptions to address the remaining issues.

Discussion on satellite transmitter output power

The following payload characteristics for DL transmissions are proposed in paper [2].

Table 2-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

Table 2-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |

It can be found that the Satellite EIRP density was defined for both set-1 and set-2. Considering the worst case and the maximum output power, it is proposed to assume 30MHz channel bandwidth for 2GHz.

Proposal 1: It is proposed to assume 30MHz channel bandwidth for 2GHz, considering the worst case and the maximum output power.

The satellite max Tx power can be calculated by the equation as below:

maxTx power(dBm)=EIRP density (dBW/MHz)+30+10 log10NRB*SCS*12-Max Gain

Based on the equation, the Max Tx power in dBm can be assumed as below.

Table 2-3 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite max TX power in dBm |  | 52.6dBm | 54.6dBm | 48.6dBm |
| Channel bandwidth |  | 30MHz | 30MHz | 30MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

Table 2-4 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| Satellite max TX power in dBm |  | 52.6dBm | 54.6dBm | 48.6dBm |
| Channel bandwidth |  | 30MHz | 30MHz | 30MHz |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |

Proposal 2: It is proposed to assume Satellite max TX power in dBm for 2GHz as above.

Discussion on Network layout

In last meeting, the heterogeneous scenario between TN and NTN systems was widely discussed. Two options can be found in summary for NTN co-existence [3]. For option 1, more than one satellite cell was proposed for NTN systems in heterogeneous scenario between TN and NTN systems. But it’s unclear how to match the network layout between TN and NTN systems. Option 2 was proposed in contribution [4], assuming only one cell for NTN systems. Generally, it also can be extended for six cells NTN systems with larger numbers of BS sites and sectors. The heterogeneous scenario between TN and NTN systems has to be considered when we look into NTN UE/Satellite performance with NR legacy networks as aggressor.

Observation 1: The heterogeneous scenario between TN and NTN systems has to be considered when we look into NTN UE/Satellite performance with NR legacy networks as aggressor.

Based on the analysis in contribution [4], the numbers of NR TN layers, sites and sectors are summarized as below.

Table 3-1 the numbers of NR TN layers, sites and sectors for 2GHz different scenarios

| Scenarios | GEO (Set-1) | LEO-1200 (Set-1) | LEO-600 (Set-1) | GEO (Set-2) | LEO-1200 (Set-2) | LEO-600 (Set-2) |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite beam diameter (D-beam) | 250 km | 90 km | 50 km | 450 km | 190 km | 90 km |
| ISD | 500m | 500m | 500m | 500m | 500m | 500m |
| Layer = D-beam/ISD/2 + 1 | 251 | 91 | 51 | 451 | 191 | 91 |
| Site_total = 3*n*(n-1) + 1 | 188251 | 24571 | 7651 | 608851 | 108871 | 24571 |

Observation 2: Even if the minimum beam diameter was chosen in table 6.1.1.1-1 from TR 38.821, thousands of sites will be used. Thus, RAN4 need to further check whether there is a method to further decrease the complexity of simulations.

Proposal 3: The figure 1 can be used as heterogeneous network layout between NR legacy network and NTN network for one beam cell.

Figure 1 The heterogeneous network layout

Discussion on Antenna model

Referring to the contribution [2], the antenna pattern modelling of TN BS and UE could be referred to TR 38.803. However, TR 38.803 is targeted for FR2 and 2GHz is assumed for FR1 NTN. Thus, it’s necessary to review the antenna pattern of TN BS and UE.

According to clause 4.2 from TR 36.942, the pattern is identical as below:

,

is the 3dB beam width which corresponds to 65 degrees, and  is the maximum attenuation

Figure 2 Antenna Pattern for 3-Sector Cells

We can use this assumption as Antenna horizontal radiation pattern (dB). As for Antenna vertical radiation pattern, it’s proposed to assume that  is 30 degrees, and  is the maximum attenuation

Table 4-1 FR1 BS antenna pattern for 2GHz

| Parameter for BS | Values |
| --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=30°,SLAV=20dB |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |
| Combining method for 3D antenna pattern (dB) |  |
| Maximum directional gain of an antenna GE,max | 12 dBi |

Proposal 4: The passive antenna is assumed for 2GHz BS. The parameter in table 4-1 can be used for 2GHz BS antenna pattern in the NTN system simulation. For UE antenna, an omni-directional radiation pattern with antenna gain 0dBi is assumed

Discussion on NTN UL power control

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [7] could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel

Referring to TR 38.803, X = 200MHz, Y = 11dB (NF), that means CLx-ile = 88. Actually, CLx-ile can be calculated by the following equation.

CLx-ile = Pmax – (SNRtarget +10*log10(kT) +10*log10(B) + F)

For the simulation in TR 38.803, SNRtarget = 15 dB, Pmax=23dBm, B = 200MHz, F=11dB, 10*log10(kT)= -174 dBm/Hz. When we put these parameters into the equation, we can get:

CLx-ile = 23-(15-80) = 88dB.

It can be found that this model can also be used for NTN UE UL power control. Thus, we can use the following TPC model for NTN UL power control.

Proposal 5: It’s proposed to use the following TPC model for UL NTN power control.

TPC model specified in Section 9.1 TR 36.942 [7] could be applied for UL NTN power control with following parameters.

Pt= Pmax+min⁡(0,maxRmin, γ×(CL-CLx-ile)

Where, Pmax = 23dBm, Rmin = TBD dB, CL (dB) is the path coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}

CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax.

CLx-ile (dB) and γ are set as following:

-	CLx-ile = Pmax – (SNRtarget +10*log10(kT) +10*log10(B) + F),

Where:

Pmax is the maximum output power for NTN UE (23dBm)

SNRtarget is the target SNR for NTN system (dB)

10*log10(kT) = -174dBm/Hz

B is the channel bandwidth (Hz)

F is the noise figure for NTN system (dB)

-	0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel

Generally, we can assume  = 1, B = 20MHz. 15dB can be used for SNRtarget. The system cascading method can be used to evaluate the noise figure for NTN system. Based on the Satellite access network architecture principles in sub-clause 4.3 from TR 38.811, the architecture can be found as below.

Figure 3 Satellite access network architecture principles

F1 is the noise figure of satellite. F2 is the noise figure of Gateway. Gain_satellite is the system gain when retransmitting signal. CL_feederlink is the couple loss between satellite and gateway. By using the system cascading method, we can get the linear value of F (the noise figure for NTN system) as below.

linear value of F = F1 + F2-1Gainsatellite/CL_feederlink

All of the parameters for this equation are linear value. It’s noted that:

1) If Gain_satellite >> CL_feederlink, we can get F = F1

2) If Gain_satellite << CL_feederlink, we can get F=F2-1Gainsatellite/CL_feederlink

3) If Gain_satellite = CL_feederlink, we can get F=F1+ F2-1

Observation 3: the noise figure F for NTN system (dB) should be further evaluated based on the couple loss assumption between satellite and gateway.

Observation 4: Generally, 15 dB targeted SNR is not suitable for NTN system. This value may affect how many UEs need to transmit the maximum output power for simulation.

Discussion on Link level performance for NR NTN

For Throughput ~ SNR mapping, it’s proposed to use section 5.2.7 of TR 38.803 in paper [2]. Regarding to TR 38.803, the parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 5.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configurations

-	AWGN channel model

-	Link Adaptation (see table 5.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 5.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in table 5.2.7-1 are targeted for eMBB coexistence scenario.

It’s infeasible to achieve 30dB/22dB DL/UL maximum SNIR for NR NTN, so the following parameters need to be further studied and RAN4 need to check them with RAN1.

Attenuation factor, representing implementation losses
SNIRMIN  Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

Proposal 6: the following parameters , SNIRMIN, and SNIRMAX need to be further studied and decided for NR NTN.

Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: It is proposed to assume 20MHz channel bandwidth for 2GHz, considering the worst case and the maximum output power.

Proposal 2: It is proposed to assume Satellite max TX power in dBm for 2GHz as below.

Table 2-3 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| Satellite max TX power in dBm |  | 52.6dBm | 54.6dBm | 48.6dBm |
| Channel bandwidth |  | 30MHz | 30MHz | 30MHz |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |

Table 2-4 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| Satellite max TX power in dBm |  | 52.6dBm | 54.6dBm | 48.6dBm |
| Channel bandwidth |  | 30MHz | 30MHz | 30MHz |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |

Observation 1: The heterogeneous scenario between TN and NTN systems has to be considered when we look into NTN UE/Satellite performance with NR legacy networks as aggressor.

Observation 2: Even if the minimum beam diameter was chosen in table 6.1.1.1-1 from TR 38.821, thousands of sites will be used. Thus, RAN4 need to further check whether there is a method to further decrease the complexity of simulations.

Proposal 3: The figure 1 can be used as heterogeneous network layout between NR legacy network and NTN network for one beam cell.

Proposal 4: The passive antenna is assumed for 2GHz BS. The parameter in table 4-1 can be used for 2GHz BS antenna pattern in the NTN system simulation. For UE antenna, an omni-directional radiation pattern with antenna gain 0dBi is assumed

Table 4-1 FR1 BS antenna pattern for 2GHz

| Parameter for BS | Values |
| --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=30°,SLAV=20dB |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |
| Combining method for 3D antenna pattern (dB) |  |
| Maximum directional gain of an antenna GE,max | 12 dBi |

Proposal 5: It’s proposed to use the following TPC model for UL NTN power control.

TPC model specified in Section 9.1 TR 36.942 [7] could be applied for UL NTN power control with following parameters.

Pt= Pmax+min⁡(0,maxRmin, γ×(CL-CLx-ile)

Where, Pmax = 23dBm, Rmin = TBD dB, CL (dB) is the path coupling loss defined as max{path loss-G_Tx-G_Rx, MCL}

CLx-ile is the x-percentile CL value. With this power control equation, the x percent of UEs that have the highest coupling loss will transmit at Pmax.

CLx-ile (dB) and γ are set as following:

-	CLx-ile = Pmax – (SNRtarget +10*log10(kT) +10*log10(B) + F),

Where:

Pmax is the maximum output power for NTN UE (23dBm)

SNRtarget is the target SNR for NTN system (dB)

10*log10(kT) = -174dBm/Hz

B is the channel bandwidth (Hz)

F is the noise figure for NTN system (dB)

-	0<<=1 is the balancing factor for UEs with bad channel and UEs with good channel

Observation 3: the noise figure F for NTN system (dB) should be further evaluated based on the couple loss assumption between satellite and gateway.

Observation 4: Generally, 15 dB targeted SNR is not suitable for NTN system. This value may affect how many UEs need to transmit the maximum output power for simulation.

Proposal 6: the following parameters , SNIRMIN, and SNIRMAX need to be further studied and decided for NR NTN system.

- References

- R4-2103878, Way Forward for NTN Coexistence Study, Samsung

- R4-2103998, Simulation assumption for NTN co-existence study, CATT

- R4-2103949, Email discussion summary for [98e][311] NTN_Solutions_Part2, Moderator (Samsung)

- R4-2101812, General discussion on NTN simulation assumptions, Huawei, HiSilicon

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)