3GPP TSG-RAN WG4 Meeting #98bis-e                                              R4-2106690

# Electronic Meeting, 12 April – 20 April 2021

Source: 	Ericsson, Nokia, Nokia Shanghai Bell

Title:	TP to the TR 38.844: Terminology

Agenda Item:	9.2.1

Document for:	Agreement

# 1	Introduction

In this contribution we provide an update to the TP to the TR 38.844.  Based upon 2 WFs [1][2] a focus of both agreed documents provided terminology to facilitate discussions on different approaches which have been discussed thus far. The intention of this contribution is to gather the terminologies from the WFs.

# 2	Discussion

During the process of compiling the definitions agreed in [1][2] it was noticed that there is some conflicting aspects relating to the definition of irregular bandwidth.  The following are the definitions agreed:

In approved WF R4-2103263:

- Irregular bandwidth = IrregularBW

  - a bandwidth that is not a multiple of 5MHz and does not exist in release 17

  - Irregular BW may have other restrictions depending on further agreements:

    - For example be a multiple of 1MHz or less than a given BW

In approved WF R4-2103387:

Irregular BW
refers to a BW that is not an existing NR channel bandwidth (CBW) in Rel-17.

The main aspect to decide is whether “not a multiple of 5 MHz” shall be considered as part of the definition.  As the current discussions have been based upon the Rel-17 specifications which contain NR regular bandwidths which are a multiple of 5MHz.  As the definition stated with “not a multiple of 5 MHz” may lead to misleading interpretations that there is intention to define a bandwidth granularity of 5 MHz.

Observation: Current definition of irregular bandwidth is not aligned between agreed WFs [1][2].  Interested companies are encouraged to provide opinon/views on whether “not a multiple of 5 MHz” shall be part of the definition for irregular bandwidth study.

Irregular bandwidth: an NR bandwidth that is not a multiple of 5 MHz and is not defined in Rel-17

# 3	References

- R4-2103263, ”WF on Use of Larger CBW”, Skyworks Solutions Inc

- R4-2103387, ”WF on Overlapping CBW method”, Nokia

##### [Start of changes]

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Existing immediately lower regular channel bandwidth: the closest NR channel bandwidth defined in Rel-17 which is smaller/less than the irregular bandwidth

Existing immediately wider regular channel bandwidth: the closest NR channel bandwidth defined in Rel-17 which is larger/wider than the irregular bandwidth

Overlapping UE channel BW from network perspective: network supports the irregular bandwidth while each UE operates in an existing lower regular NR channel bandwidth

Overlapping UE channel BW from UE perspective: network supports the irregular bandwidth while some new UEs support two overlapping (RF) carriers

Overlapping CA: the irregular bandwidth is handled by two overlapping component carriers (CCs) with NR channel bandwidth defined in Rel-17.  One of them (PCell) or both CCs contain an SSB. It is network responsibility to prevent collisions between the different component carriers.

Single BB carrier: means that from baseband (RAN1) perspective, there is a single cell with a waveform according to a single carrier

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

BS	Base Station

BW	Bandwidth

CBW	Channel Bandwidth

SCBW	Smaller Channel Bandwidth (Existing immediate lower channel bandwidth)

FR1	Frequency Range 1

RF	Radio Frequency

WCBW	Wider Channel Bandwidth (Existing immediate wider channel bandwidth)

UE	User Equipment

# 4	General

## 4.1	UE channel bandwidth

The following text is copied from TS38.101-1 for information:

The UE channel bandwidth supports a single NR RF carrier in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

[Unchanged Sections]

Annex A (normative): 
Terminology Figures

# A.1	General

In this annex the associated figures are captured.

# A.2	Existing Immediately Wider Regular Channel Bandwidth

# A.3	Existing Immediately Lower Regular Channel Bandwidth

# A.4	Overlapping UE channel BW from network perspective

# A.5	Overlapping UE channel BW from UE perspective

# A.6	Overlapping CA

##### [End of changes]