3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106795

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Discussions and simulation results on NR-U PUCCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.1.4.4.3

Document for:	Discussion

Background

As per WF [1], RAN 4 has agreed to introduce NR-U PUCCH performance requirements and simulation assumptions have been determined, but there is still one open issue left about test metric of PF3. In this paper, we provide our simulation results and discussions about this issue.

Simulation results

Simulation results are shown in Figure 2-1~Figure 2-4 and related required SNR values are summarized in Table 2-1.The simulation assumptions derived from [1] are shown in appendix.

Figure 2-1: Simulation results PF0

Figure 2-2: Simulation results PF1

Figure 2-3: Simulation results PF2

Figure 2-4: Simulation results PF3

Table 2-1: Summary of simulation results for NR-U PUCCH.

| Test metric (SNR dB @ 1% of BLER or 1% of ACK miss or 0.1%of NACK) |  | PF0 | PF1 |  | PF2 | PF3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACK miss | ACK miss | NACK2ACK | UCI BLER | UCI BLER | ACK miss |
| TDLA30-10 | 15kHz | -4.6 | -17.2 | -16.7 | 1.3 | -3.89 | -0.11 |
|  | 30kHz | -3.9 | -17.1 | -16.3 | 1.9 | -4.47 | -0.68 |

Discussions

One open issue left at last meeting is that test metric of PF3.  From our understanding, the test metric depends on UCI type, ACK miss should be used if HARQ-NACK information is transmitted (Rel-16 PF2 performance test) and UCI block BLER should be used if CSI part 1/ CSI part 2 is transmitted (Rel-16 UCI multiplexed on PUSCH test). For PF3 test, UCI block BLER has been used in Rel-15, so we prefer to use ACK miss as test metric for NR-U PF3 test. i.e. Prob (DTX->ACK)<=1% and Prob (ACK miss)<=1%. It is noted that no metric for NACK to ACK is needed since SNR @ Prob (NACK->ACK)= 0.1 % is much lower than SNR @ Prob (ACK miss)= 1 %  based on our simulation results in Figure 2-4.

Proposal 1: For PF3 test, UCI bits only contain HARQ-ACK information and use following test metric:

- Prob (DTX->ACK)≤1%

- Prob (ACK miss)≤1%.

Conclusion

In this paper, we provide our simulation results and discussions for NR-U PUCCH performance requirements. The proposals are:

Proposal 1: For PF3 test, UCI bits only contain HARQ-ACK information and use following test metric:

- Prob (DTX->ACK)≤1%

- Prob (ACK miss)≤1%.

Reference

[1]   R4-2103989 Way Forward on NR-U PUCCH demodulation requirements. Ericsson

Appendix

Table 5-1: Simulation assumptions for PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | TDLA30-10 Low |
| Test metric | SNR@Prob(ACK missed)≤10-2 SNR@Prob(DTX->ACK)≤10-2 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 5-2: Simulation assumptions for PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | TDLA30-10 Low |
| Test metric | SNR@Prob(ACK miss)≤10^(−2) SNR @Prob(PUCCH DTX→Ack bits) ≤ 10^(−2) SNR@Prob(NACK→ACK)≤10^(−3) |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 5-3: Simulation assumptions for PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits | 22 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note 1 |
| Propagation conditions | TDLA30-10 Low |
| OCC-Length-r16 | Not configured |
| Test metric | SNR @ ProbUCI block BLER=10-2 |
| Note 1: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 5-4: Simulation assumptions for PF3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | 4 |
| The number of UCI information bits | 4 bits |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0Note 1 |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | Not configure |
| Length of OCC | Not configure |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 |
| Note 1: RBs 0, 10, 20,…,90 are allocated for 15kHz and RBs 0,5,10,…,45 are allocated for 30kHz. Note 2: The UCI information does not contain CSI part 2. |  |