3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106800

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation results on PSFCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.2.1.2.4

Document for:	Discussion

Background

At last meeting, simulation assumptions for PSFCH were discussed. In this paper, we provide our simulation results based on simulation results from [1].

Simulation results

Simulation results for PSFCH are shown in Figure 2-1 and Table 2-1. Corresponding simulation assumptions derived from [1] are shown in Table 5-1

.

Figure 2-1: Simulation results for PSFCH

Table 2-1 Summary of simulation results for PSFCH

| Propagation conditions | SNR@ Prob(NACK miss)=1% and Prob(DTX to NACK)=1% |  |
| --- | --- | --- |
|  | Ideal results | Impairment results |
| TDLA-30-180 | 6.86 dB | 8.36dB |

Conclusion

In this paper, we provide our simulation results for PSFCH performance test.

Reference

[1]   R4-2103991 Updated simulation assumptions for V2X single link test cases, Huawei, HiSilicon

Appendix

Table 5-1: Simulation assumption for PSFCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols(Note 1) |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH periodicity | Slot | 1 |
| Number of cyclic shift pair |  | 1 |
| Antenna configuration |  | 1x2 Low |
| Performance metric |  | SNR@ Pr(NACK miss) ≤1% and Pr(DTX to NACK)=1% |
| Note 1: The first OFDM symbol of a PSFCH is duplicated as described in clause 8.3.4.2.2 of TS 38.211 and not used for demodulation. |  |  |