3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106808

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Discussion on PDSCH CA scenarios for NR UE HST FR1 performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.6.3.2

Document for:	Discussion

Background

During RAN4#98-e meeting, way forward [1] on NR FR1 HST demodulation requirements was approved. In this contribution, we share our views about the demodulation requirements for NR UE HST FR1 CA scenarios.

Discussion

Transmission schemes

- Transmission schemes - Define HST CA requirements for HST-SFN joint transmission - FFS whether to define HST CA requirements for DPS transmission scheme (with one active TCI state and/or two active TCI states) - It’s RAN4 common understanding DPS scheme can achieve better performance compared to joint transmission schemes and no advanced received required for HST deployment. - Candidate option for further discussion: - It’s RAN4 recommendation to include DPS scheme into FR1 HST WI, and further discuss whether test cases need to specified or not, and if specified, test applicable rules will be specified to ensure only one scheme will be verified for CA cases for UE.

As per the revised WID [2] approved in RAN#91-e meeting, DPS transmission scheme has been added in to the WI.

- Carrier Aggregation (CA) scenario - Specify the UE demodulation requirements for CA scenario with the same target speed (up to 500km/h) and carrier frequency (up to 3.6 GHz) as Rel-16 NR HST for HST-SFN joint transmission scheme and DPS transmission scheme. - Requirements/tests applicability rules should be discussed in order to minimize the testing burden.

In Rel-16 NR HST WI, both DPS 1a and 1b requirements are defined. For DPS CA, it is reasonable to align with the single carrier requirements to define both requirements. For the applicability rule between DPS 1a and 1b for HST CA requirements, we can reuse it from single carrier.

For the applicability rule, considering the RAN4 common understanding above that “better performance and no advanced received required for DPS comparing to SFN”, we propose that UE has passed DPS CA requirements can skip SFN CA requirements to minimize the testing burden.

For HST-DPS, considering DPS with both one and two active TCI(s). Reuse the applicability rule between the two DPS schemes from single carrier.

Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements.

Scheduling on ‘S’ slots

In Rel-16 HST WI, requirements under HST-SFN propagation condition is assumed that PDSCH is not scheduled on ‘S’ slots considering that large Doppler spread. Same assumption can be considered for HST CA requirements.

PDSCH is not scheduled on ‘S’ slots under HST-SFN propagation condition for HST CA requirements.

SCS configuration

- SCS configuration - At least cover following cases: - FDD 15 kHz + FDD 15 kHz - TDD 30 kHz + TDD 30 kHz - FDD 15 kHz + TDD 30 kHz - FFS whether following need to be included or not: - FDD 15 kHz + TDD 15 kHz CA - TDD 15 kHz + TDD 30 kHz CA

In Rel-16 HST WI, TDD 15 kHz is not considered, we don’t think TDD 15 kHz is the typical scenario. Also, the performance of TDD 15 kHz has been verified in normal CA requirements. So we prefer not to consider TDD 15 kHz for HST CA scenario.

Do not consider TDD 15 kHz for HST CA requirements.

Applicability rule for SCS configuration

- Applicability rule for SCS configuration - FFS whether reuse CA CQI applicability rule on CA duplex modes for testing: If UE supports both FDD 15 kHz + TDD 30 kHz and FDD 15 kHz + FDD 15 kHz CA duplex modes, apply requirements only to the first one

Applicability rule for SCS configuration are defined in CA CQI to reduce the test effort. For our understanding, it is reasonable to reuse the applicability rule defined in CA CQI.

Reuse CA CQI applicability rule on CA duplex modes for testing.

Release independent

- Release independent - FFS whether HST PDSCH CA requirements can be release independent from Rel-15

In Rel-16 LTE HST enhancement WI, highSpeedEnhMeasFlagSCell is introduced for RRM SCell measurement enhancement. Currently in Rel-17 NR HST WI, whether RRM measurement enhancement for SCell to be specified is still under discussion, maybe stricter RRM measurement requirements will be specified that is a higher challenge to UE implementation. Also considering LTE HST CA requirements are not release independent, we prefer to not define release independent for NR HST CA requirements.

Not define release independent for HST CA requirements.

UE capability and network-assisted signalling

- UE capability and network-assisted signalling - FFS whether existing HST network assisted signaling and UE capability can cover CA scenario

For UE capability, as per TS 36.306, there is only two field related to demodulation in HST scenario, no additional UE capability defined to support LTE HST-SFN CA scenario for demodulation requirements:

4.3.33.2 demodulationEnhancements-r14 This field defines whether the UE supports advanced receiver in SFN scenario as specified in TS 36.101 [6]. 4.3.33.5 demodulationEnhancements2-r16 This field defines whether the UE supports further enhanced demodulation requirements to support 500km/h velocity in HST-SFN scenario as specified in TS 36.101 [6]. A UE indicating support of demodulationEnhancements2-r16 shall also indicate support of demodulationEnhancements-r14.

For Rel-16 NR HST, network-assisted signalling is defined as “ServingCellConfigCommon->highSpeedConfig-r16->highSpeedDemodFlag-r16” that is configured for a UE with a SCells or with an additional cell group (SCG). For our understanding, the Rel-16 defined signalling already cover the scenario for CA without any new signalling to be introduced.

Therefore, we don’t think it is necessary to introduce any new UE capability or new network-assisted signalling from demodulation performance requirements point of view.

Existing HST network assisted signaling and UE capability can cover CA scenario.

Do not introduce extra UE capability or network-assisted signaling for SCell for HST CA scenario.

Proposals

In this contribution, we discuss on demodulation performance for NR UE HST FR1 CA scenarios. Our observations and proposals are:

- Existing HST network assisted signaling and UE capability can cover CA scenario.

- For HST-DPS, considering DPS with both one and two active TCI(s). Reuse the applicability rule between the two DPS schemes from single carrier.

Define applicability rule that UE has passed DPS CA requirements can skip SFN CA requirements.

PDSCH is not scheduled on ‘S’ slots under HST-SFN propagation condition for HST CA requirements.

Do not consider TDD 15 kHz for HST CA requirements.

Reuse CA CQI applicability rule on CA duplex modes for testing.

Not define release independent for HST CA requirements.

Do not introduce extra UE capability or network-assisted signaling for SCell for HST CA scenario.

Reference

R4-2103876, WF on UE demodulation for FR1 HST, RAN4#98-e, CMCC

RP-210833, Revised WID on enhanced NR support for high speed train scenario for frequency range 1 (FR1), RAN#91-e, CMCC