3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106811

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Simulation results for evaluations of enhanced transmission schemes for NR UE HST FR1 performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.6.3.3

Document for:	Information

Background

During RAN4#98-e meeting, the way forward [1] for NR UE HST FR1 demodulation requirements was approved. In this contribution, we provide our evaluation of enhanced transmission schemes for NR UE HST FR1 performance requirements.

Simulation results

Comparison among three transmission schemes

For comparison among the three transmission schemes, we use the fixed RRH transmission power so the SNR is changed for different time and different RRH. The SNR in this clause is refer to the SNR calculated at the time point when UE is nearest to the RRH. MCS17/13 is used for Figure 2.1-1 and Figure 2.1-2 respectively.

Figure 2.1-1 Multi-DCI based multi-TRP vs HST DPS

Figure 2.1-2 Multi-DCI based multi-TRP vs HST SFN

Bad performance using multi-DCI based multi-TRP when UE near the RRH.

DPS is better than multi-DCI based multi-TRP all the time from the perspective of throughput.

Maximum supported frequency/time offset

In this clause we evaluate the non-overlapping case for multi-DCI based multi-TRP scenario, using the fixed SNR as other requirements did, i.e. the UE received power is normalized for each RRH.

Maximum supported frequency offset

Table 2.2-1 Maximum supported frequency offset for multi-DCI based multi-TRP

| Case | CBW/SCS | MCS | Rank | Frequency offset [Hz] | Timing offset [μs] | Antenna | SNR@70%maximum TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz15kHz | 13 | 2+2 | 870 | 0 | 2x2 | 6.62 |
| 2 | 10MHz/15kHz | 13 | 2+2 | 870 | 0 | 2x4 | 3.70 |
| 3 | 10MHz/15kHz | 13 | 2+2 | 1000 | 0 | 2x2 | N/A |
| 4 | 10MHz/15kHz | 13 | 2+2 | 1000 | 0 | 2x4 | N/A |
| 5 | 10MHz15kHz | 17 | 2+2 | 870 | 0 | 2x2 | 10.28 |
| 6 | 10MHz/15kHz | 17 | 2+2 | 870 | 0 | 2x4 | 7.45 |
| 7 | 10MHz/15kHz | 17 | 2+2 | 1000 | 0 | 2x2 | N/A |
| 8 | 10MHz/15kHz | 17 | 2+2 | 1000 | 0 | 2x4 | N/A |

Figure 2.2-1 Maximum supported frequency offset for multi-DCI based multi-TRP

Total frequency offset over than 870Hz for 15 kHz SCS will cause the “frequency wrap” then leads to the significant performance degradation.

Maximum supported timing offset

Table 2.2-1 Maximum supported frequency offset for multi-DCI based multi-TRP

| Case | CBW/SCS | MCS | Rank | Frequency offset [Hz] | Timing offset [μs] | Antenna | SNR@70 maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40MHz/30kHz | 13 | 2+2 | 1667 | 0 | 2x2 | 6.68 |
| 2 | 40MHz/30kHz | 13 | 2+2 | 1667 | 0 | 2x4 | 3.83 |
| 3 | 40MHz/30kHz | 13 | 2+2 | 1667 | -0.5 | 2x2 | 6.82 |
| 4 | 40MHz/30kHz | 13 | 2+2 | 1667 | -0.5 | 2x4 | 4.02 |
| 5 | 40MHz/30kHz | 17 | 2+2 | 1667 | 0 | 2x2 | 10.11 |
| 6 | 40MHz/30kHz | 17 | 2+2 | 1667 | 0 | 2x4 | 7.40 |
| 7 | 40MHz/30kHz | 17 | 2+2 | 1667 | -0.5 | 2x2 | 10.41 |
| 8 | 40MHz/30kHz | 17 | 2+2 | 1667 | -0.5 | 2x4 | 7.58 |

Figure 2.2-1 Maximum supported frequency offset for multi-DCI based multi-TRP

Maximum throughput cannot achieved for multi-DCI based multi-TRP with MCS 17 and TDD 30 kHz regardless the value of timing offset.

Conclusion

In this contribution, we provide our evaluation results about NR UE HST FR1 enhanced transmission schemes demodulation performance under propagation condition of HST-SFN and HST-DPS. Our observations are:

- Bad performance using multi-DCI based multi-TRP when UE near the RRH.

DPS is better than multi-DCI based multi-TRP all the time from the perspective of throughput.

Total frequency offset over than 870Hz for 15 kHz SCS will cause the “frequency wrap” then leads to the significant performance degradation.

Maximum throughput cannot achieved for multi-DCI based multi-TRP with MCS 17 and TDD 30 kHz regardless the value of timing offset.

Reference

R4-2103876, WF on UE demodulation for FR1 HST, RAN4#98-e, CMCC