3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106831

# Electronic Meeting, 12th - 20th Apr, 2021

Title: 	Discussion on BS demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	8.7.5.3

Document for:	Discussion

Background

During RAN#97-e meeting, Work plan [1] for NR support for high speed train scenario in FR2 was approved. In this contribution, we share our views about BS demodulation requirements for FR2 HST.

Discussion

PUSCH

Maximum Doppler

As per our contribution [2], 350km/h can be supported by using DMRS+PTRS. With the assumption of 29.5GHz carrier frequency, the maximum Doppler can be 19191Hz for 350km/h.

CBW

For CBW, we prefer to choose the typical one, such as 200 MHz.

DMRS

As per RAN1 feature list, DMRS 1+1+1 is the mandatory without capability signaling feature while DMRS 1+1+1+1 is the optional with capability signalling. So we prefer to use DMRS 1+1+1 for HST FR2 performance requirements definition.

Others

For other parameters, such as waveform, channel model and test metric, we prefer to reuse the same value from HST FR1 performance requirements.

Define PUSCH performance requirements for HST FR2 with the following parameters:

| Parameter | Value |
| --- | --- |
| Maximum Doppler | 19191Hz |
| Channel model | single-tap |
| CBW/SCS | 200MHz/120kHz |
| PUSCH mapping | Type B, start symbol 0, duration 10 |
| DMRS | 1+1+1 |
| PTRS | KPTRS=2, LPTRS=1 |
| Waveform | CP-OFDM |
| Antenna configuration | 1x2 |
| MCS | 16 |
| Test metric | 70% of maximum throughput |

PRACH

Here we analyze the feasibility for each format of preamble.

Doppler

With the assumption of 29.5GHz carrier frequency, the maximum Doppler can be 19191Hz for 350km/h.

The maximum supported Doppler is proportional to the SCS of the preamble. The following table 2.3-1 shows the maximum supported Doppler for each preamble format.

Table 2.3-1 Maximum supporting Doppler for different preamble formats

| Format | Maximum supported Doppler [kHz] |
| --- | --- |
| 0/1/2 | 0.625 |
| 0/1/2 (restricted set type A) | 1.25 |
| 0/1/2 (restricted set type B) | 2.5 |
| 3 | 2.5 |
| 3 (restricted set type A) | 5 |
| 3 (restricted set type B) | 10 |
| A1/A2/A3/B1/B2/B3/B4/C0/C2 | 60 |

Preamble format 0/1/2/3 cannot be used for HST FR2.

Range

For both Scenario A and Scenario B, the distance of two nearest RRH is 700m. The maximum supported range can be derived based on the CP and GP length. The following table 2.3-2 shows the maximum supported range for each preamble format, assuming 120 kHz SCS.

Table 2.3-2 Maximum supporting range for different preamble formats

| Format | Maximum supported range [m] |
| --- | --- |
| 0 | 14478 |
| 1 | 102278 |
| 2 | 22806 |
| 3 | 14478 |
| A1 | 175 |
| A2 | 350 |
| A3 | 525 |
| B1 | 44 |
| B2 | 131 |
| B3 | 219 |
| B4 | 482 |
| C0 | 666 |
| C2 | 1245 |

Preamble format A1/A2/A3/B1/B2/B3/B4/C0 cannot be used for HST FR2.

Above all, we propose to use C2 for HST FR2 performance requirements definition under AWGN channel with 19191 Hz frequency offset.

Use C2 for HST FR2 performance requirements definition under AWGN channel with 19191 Hz frequency offset.

UL TA

In NR Rel-16 HST FR1 WI, UL TA requirements are defined. For FR2, we propose to use the same value for MCS, SRS slots configuration, etc. For other parameters, such as the PUSCH and SRS resource allocation, that can be derived with the assumption of 200 MHz CBW.

Define UL TA requirements with the following parameters:

| Parameter | Value |
| --- | --- |
| Channel model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350 km/h |
| CP length | Normal |
| A | 1.25 μs |
| Δω | 1.04 s-1 |
| MCS | 16 |
| CBW | 200MHz |
| PUSCH resource allocation | 0 to 65 RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | last symbol in slot #3 in radio frames, CSRS = 33, BSRS =0, for 132 RB |

Proposals

In this contribution, we discuss on demodulation performance for NR BS HST FR2. Our observations and proposals are:

- Preamble format 0/1/2/3 cannot be used for HST FR2.

Preamble format A1/A2/A3/B1/B2/B3/B4/C0 cannot be used for HST FR2.

- Define PUSCH performance requirements for HST FR2 with the following parameters:

| Parameter | Value |
| --- | --- |
| Maximum Doppler | 19191Hz |
| Channel model | single-tap |
| CBW/SCS | 200MHz/120kHz |
| PUSCH mapping | Type B, start symbol 0, duration 10 |
| DMRS | 1+1+1 |
| PTRS | KPTRS=2, LPTRS=1 |
| Waveform | CP-OFDM |
| Antenna configuration | 1x2 |
| MCS | 16 |
| Test metric | 70% of maximum throughput |

Use C2 for HST FR2 performance requirements definition under AWGN channel with 19191 Hz frequency offset.

Define UL TA requirements with the following parameters:

| Parameter | Value |
| --- | --- |
| Channel model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350 km/h |
| CP length | Normal |
| A | 1.25 μs |
| Δω | 1.04 s-1 |
| MCS | 16 |
| CBW | 200MHz |
| PUSCH resource allocation | 0 to 65 RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | last symbol in slot #3 in radio frames, CSRS = 33, BSRS =0, for 132 RB |

Reference

R4-2014846, Work plan for NR support for high speed train scenario in FR2, RAN4#97-e, Samsung, Nokia, Nokia Shanghai Bell

R4-2106829, Discussion on general issues for NR FR2 HST demodulation requirements, RAN4#98-bis-e, Huawei, HiSilicon

R4-2103240, Way forward on Deployment Scenario and UE RF Requirement for FR2 HST, RAN4#98-e, Samsung