3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2106914

Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	On SCell activation in NR-U

Agenda item:	5.1.2.3

Document for:	Discussion

# 1	Introduction

In RAN4 98-e, the following topic has been discussed. Companies’ views were summarized in WF [1], the content copied below:

- This paper discusses the open issue listed above in the WF and provide our view.

2	Discussions

From several meetings ago, companies have been discussing the issues related to allowed interruptions for Scell activation under NR-U. In RAN4 98-e meeting, it was agreed that only the case when there is already an activated SCell needs further discussion. We analyze this issue below.

When there is already an activated SCell on the band where a new SCell is about to be activated, the UE will attempt an AGC adjustment. Prior to AGC adjustment the UE will need to first do RF tuning, which will cause an interruption. If AGC fails, the UE will tune back to the original BWP which will cause a second interruption. Thus, when there is already an activated cell in the same band as the serving cell, two interruptions can be expected.

As per the discussions in the RF session, even for intra-band CA scenario, RF tuning is needed. Thus, for intra- and inter-band CA, additional interruptions are needed. It was captured in the WF [1] that “For any active cell outside the band with the SCell being activated, the interruption requirements are FFS” and our view is that additional interruptions are needed for active cells outside the band with the SCell being activated.

Additional interruptions are needed for active cells outside the band with the SCell being activated.

As to the issue of whether requirements still apply when sCellDeactivationTimer is not configured, our view is that requirements shall apply. First of all, even if the time is not configured, the UE behavior shall still be clear. In TS 38.331 [2], the case when the timer sCellDeactivationTimer is not configured has been considered. The UE will apply the value inifity under this case.

Observation 1: The case when the timer sCellDeactivationTimer is not configured is specified in TS 38.331 and the UE behavior is clear.

Thus, we don’t see why not configuring the timer should cause any difference than the case when the timer is configured.

sCellDeactivationTimer SCell deactivation timer in TS 38.321 [3]. If the field is absent, the UE applies the value infinity.

- SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

- SCell activation delay requirements are applicable when sCellDeactivationTimer is not configured also in Scenarios B and C (EN-DC and SA) LBT types other than 2C.

3	Conclusion

Proposal 1: Additional interruptions are needed for active cells outside the band with the SCell being activated.

Observation 1: The case when the timer sCellDeactivationTimer is not configured is specified in TS 38.331 and the UE behavior is clear.

- Proposal 2: SCell activation delay requirements are applicable in Scenario A (CA with NR PCell and NR SCell) with any LBT type and in Scenario B and C (E-UTRAN-NR-U DC/SA NR-U) with LBT type 2C. Requirements are also applicable in all scenarios, if the UE does not experience any UL LBT failures during SCell activation/deactivation.

SCell activation delay requirements are applicable when sCellDeactivationTimer is not configured also in Scenarios B and C (EN-DC and SA) LBT types other than 2C.

References

- R4-2103512

- TS 38.331