3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2106924

Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	Discussion on handover with PSCell

Agenda item:	8.4.2.2

Document for:	Discussion

# 1	Introduction

In RAN4 98-e, the following topic has been discussed. Companies’ views were summarized in WF [1], the content copied below:

- This paper discusses the open issue listed above in the WF and provide our view.

2	Discussion

As pointed out in the WID, the procedures for handover with PSCell are already supported by RAN2 and captured in RAN2 specification TS 37.340 [2]. As captured in TS 37.340, the procedure for handover with PSCell change is depicted below.

During the handover, the master node is changed and a secondary node (the PSCell) can be added. The procedure is similar to handover plus PSCell addition followed right after.

Observation 1: Procedures for handover with PSCell addition is similar to handover followed by PSCell addition immediately after.

The mission for RAN4 within this WI is to specify related requirements (core requirement). We think the reasonable approach is to take core requirements for handover and PSCell addition as baseline and identify if there is any new issue in the new procedure.

Thus, for the starting point of the delay, we support to reuse the starting point definition from legacy HO, i.e., the end of the last TTI containing the RRC command implying handover with PSCell.

For the starting point of the delay, reuse the starting point definition from legacy HO, i.e., the end of the last TTI containing the RRC command implying handover with PSCell.

For interruption requirements, consider the following options:

- - Specify a total interruption for handover and PSCell addition

- - Specify separate interruptions for handover and PSCell addition.

When specifying requirements, we need to align with the current specification and the same applicability rule in TS 38.133 [3] shall apply. The current rule is copied below.

### 3.6.8 Applicability of 2-step RA and 4-step RA in RRM requirements Unless explicitly stated otherwise the requirements under the following clauses, where the UE transmits random acess to NR serving cell or NR target cell, are applicable for both 2-step RA and 4-step RA procedures [3]: - Handover requirements in clause 6.1, except for clause 6.1.2 - RRC connection mobility control requirements in clause 6.2, except for clause 6.2.2, - UE transmit timing requirements in clause 7.1, - PScell addition delay requirements in clause 8.9.2, - PSCell change requirements in clause 8.11 and - Conditional PSCell change requirements in clause 8.11B.

As reflected by the current applicability rule, both 2-step RA and 4-step RA are considered in handover requirements (except for handover to another RAT) and PSCell addition delay requirements. Thus, it’s reasonable to also include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell.

Some companies suggested to wait for the reply LS on issue 2-2-3 and then conclude on this, however we don’t think they are relevant issues. Again, we want to repeat our point that 2-step RACH is defined in a generic way and by default it applies to all relevant procedures involving RA.

Include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell.

3	Conclusion

Observation 1: Procedures for handover with PSCell addition is similar to handover followed by PSCell addition immediately after.

Proposal 1: For the starting point of the delay, reuse the starting point definition from legacy HO, i.e., the end of the last TTI containing the RRC command implying handover with PSCell.

Proposal 2: For interruption requirements, consider the following options:

- - Specify a total interruption for handover and PSCell addition

- - Specify separate interruptions for handover and PSCell addition.

Proposal 3: Include both 2-step RA and 4-step RA into the new requirements made for handover with PSCell.

References

- R4-2103673