3GPP TSG-RAN4 Meeting #98-bis-e	R4-2107007

# Electronic Meeting, April 12 –20, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to introduce accuracy requirements for RSTD measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-03-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR is based on Big draftCR R4-2103585 endorsed at RAN4#98-e. According to the work split in R4-2017158, this CR introduces RSTD measuremrent accuracy requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces RSTD measuremrent accuracy requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no RSTD measuremrent accuracy requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 10.1.23	RSTD Measurements

#### 10.1.23.1	Introduction

The requirements in Clause 10.1.23 shall apply, provided the UE has received nr-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the UE to report one or more DL RSTD measurements defined in TS 38.215 [4].

10.1.23.2	Measurement Accuracy Requirements

The RSTD measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.23.2-1 for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant PRS resource configured for measurement.

The RSTD measurement reported by the UE shall fulfil the accuracy requirements defined in Table 10.1.23.2-2 for FR2, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for RSTD measurements are fulfilled according to Annex B.2.z for a corresponding Band for each relevant PRS resource configured for measurement.

Table 10.1.23.2-1:  RSTD absolute accuracy in FR1

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| Tc Note 5 | dB | kHz | RB |  |  | dBm/SCS | dBm/BWChannel |
| TBD+ΔNote 7 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| TBD+Δ |  |  | ≥ [44] | ≥ [2] | Note 6 | Note 6 | Note 6 |
| TBD+Δ |  |  | ≥ [88] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| TBD+Δ |  |  | ≥ [172] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| TBD+ΔNote 7 |  | 30 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -117 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 | -50 |
| TBD+Δ |  |  | ≥ [44] | ≥ [2] | Note 6 | Note 6 | Note 6 |
| TBD+Δ |  |  | ≥ [88] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| TBD+Δ |  |  | ≥ [172] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| TBD+ΔNote 7 |  | 60 | ≥ [24] | ≥ [4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_FDD_FR1_B | -114.5 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_TDD_FR1_C | -114 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -114 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_FDD_FR1_F | -113.5 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_FDD_FR1_G | -113 | -50 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | NR_FDD_FR1_H | -111.5 | -50 |
| TBD+Δ |  |  |  |  |  |  |  |  |  |  |  | ≥ [44] | ≥ [2] | Note 6 | Note 6 | Note 6 |
| TBD+Δ |  |  | ≥ [88] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| TBD+Δ |  |  | ≥ [172] | ≥ [1] | Note 6 | Note 6 | Note 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. The number of repetitions for a PRS resource is dl-PRS-ResourceRepetitionFactor * dl-PRS-NumSymbols / dl-PRS-CombSizeN subject to UE capability N (duration of DL PRS symbols in ms). NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: Δ=0 if reference resource and the measured neighbour resource i are on the same PRS frequency layer, Δ=[32]*Tc otherwise. |  |  |  |  |  |  |  |

Table 10.1.23.2-2:  RSTD absolute accuracy in FR2

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| TBD+ΔNote 6 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ [24] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| TBD+Δ |  |  | ≥ [44] | ≥ [2] | Note 5 | Note 5 |
|  |  |  | ≥ [88] | ≥ [1] | Note 5 | Note 5 |
| TBD+Δ |  |  | ≥ [172] | ≥ [1] | Note 5 | Note 5 |
| TBD+ΔNote 6 |  | 120 | ≥ [24] | ≥ [4] | Same value as PRS_RP in Table B.2.z-2, according to UE Power class, operating band and angle of arrival | -50 |
| TBD+Δ |  |  | ≥ [44] | ≥ [2] | Note 5 | Note 5 |
| TBD+Δ |  |  | ≥ [88] | ≥ [1] | Note 5 | Note 5 |
| TBD+Δ |  |  | ≥ [172] | ≥ [1] | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. The number of repetitions for a PRS resource is the multiple of dl-PRS-ResourceRepetitionFactor * dl-PRS-NumSymbols / dl-PRS-CombSizeN subject to UE capability N (duration of DL PRS symbols in ms). NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δ=0 if reference resource and the measured neighbour resource i are on the same PRS frequency layer, Δ=[32]*Tc otherwise. |  |  |  |  |  |  |

<End of Change 1>