3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2107024

Electronic Meeting, 12 – 20 April, 2021

Title: 	Discussion on CSI-SINR accuracy requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.7.2.2.3

Document for:	Discussion

Introduction

CSI-SINR accuracy requirements have been discussed in RAN4#98-e. The outcomes are captured in the WF [1]. Most open issues in defining the requirements were closed, and the only remaining open issue is the combination of the timing offset side condition and the upper limit of Es/Iot:

- Issue 2-3-1: The upper limit of Es/Iot for CSI-SINR measurement with timing offset(T△) - Option 1: Es/Iot ≤ [10] dB for the case that timing offset is within CP. - Option 2: Es/Iot ≤ [18] dB for the case that timing offset is within CP/2.

To determine the side condition and upper limit, an updated simulation assumption was approved [2]. In this paper we will provide additional simulation results and our views on the timing offset condition and Es/Iot upper bound.

Discussion

Simulation results

The simulation results in this section are based on assumptions in [2]. The 5%- and 95%-tile of CSI-SINR error are shown in Table 1~3 for 15kHz, 30kHz and 120kHz, respectively.

Table 1: CSI-RSRP accuracy with 15kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  | ETU70 |  |
|  |  | 5% | 95% | 5% | 95% |
| 0 | 15 | -0.43 | 0.6 | -1.33 | 0.31 |
|  | 18 | -0.54 | 0.53 | -1.79 | 0.08 |
|  | 20 | -0.62 | 0.49 | -2.27 | -0.2 |
| -0.5*CP | 15 | -0.37 | 0.73 | -0.75 | 0.67 |
|  | 18 | -0.44 | 0.7 | -0.88 | 0.55 |
|  | 20 | -0.46 | 0.64 | -1.01 | 0.45 |
| 0.5*CP | 10 | -1.33 | 0.05 | -2.16 | -0.36 |
|  | 12 | -1.64 | -0.19 | -2.76 | -0.72 |
|  | 15 | -2.39 | -0.75 | -4.01 | -1.49 |
| -0.9*CP | 10 | -1.68 | -0.13 | -1.63 | 0.16 |
|  | 12 | -2.06 | -0.51 | -1.96 | -0.02 |
|  | 15 | -3.24 | -1.52 | -2.85 | -0.51 |
| 0.9*CP | 6 | -1.82 | -0.3 | -2.54 | -0.41 |
|  | 8 | -2.38 | -0.81 | -3.12 | -0.92 |
|  | 10 | -3.14 | -1.38 | -3.87 | -1.57 |
| -CP | 8 | -1.51 | 0 | -1.63 | 0.22 |
|  | 10 | -2.01 | -0.51 | -1.94 | 0 |
|  | 12 | -2.69 | -1.12 | -2.5 | -0.38 |
| CP | 6 | -2.71 | -1.22 | -3.07 | -0.63 |
|  | 8 | -3.65 | -2.16 | -3.97 | -1.24 |
|  | 10 | -4.9 | -3.33 | -5.05 | -2.01 |

Table 2: CSI-RSRP accuracy with 30kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  | EPA5 |  |
|  |  | 5% | 95% | 5% | 95% |
| 0 | 15 | -0.42 | 0.68 | -0.71 | 0.63 |
|  | 18 | -0.55 | 0.58 | -0.78 | 0.65 |
|  | 20 | -0.6 | 0.54 | -0.89 | 0.62 |
| -0.5*CP | 15 | -0.38 | 0.74 | -0.62 | 0.74 |
|  | 18 | -0.44 | 0.67 | -0.67 | 0.71 |
|  | 20 | -0.49 | 0.64 | -0.77 | 0.65 |
| 0.5*CP | 10 | -1.34 | 0.04 | -1.81 | 0.22 |
|  | 12 | -1.72 | -0.17 | -2.35 | -0.05 |
|  | 15 | -2.5 | -0.71 | -3.47 | -0.47 |
| -0.9*CP | 10 | -1.67 | -0.15 | -1.93 | 0.13 |
|  | 12 | -2.11 | -0.62 | -2.58 | -0.12 |
|  | 15 | -3.34 | -1.61 | -4.05 | -0.52 |
| 0.9*CP | 6 | -1.8 | -0.23 | -2.32 | 0.12 |
|  | 8 | -2.35 | -0.7 | -3.08 | -0.22 |
|  | 10 | -3.08 | -1.41 | -4.05 | -0.57 |
| -CP | 8 | -1.45 | -0.12 | -1.81 | 0.36 |
|  | 10 | -1.97 | -0.51 | -2.49 | 0.11 |
|  | 12 | -2.69 | -1.12 | -3.41 | -0.2 |
| CP | 6 | -2.78 | -1.25 | -3.84 | -0.25 |
|  | 8 | -3.69 | -2.16 | -5.06 | -0.74 |
|  | 10 | -4.89 | -3.36 | -6.53 | -1.28 |

Table 3: CSI-RSRP accuracy with 120kHz SCS

| Timing error | SNR | Channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDLA-30-3 |  | TDLC-30-30 |  |
|  |  | 5% | 95% | 5% | 95% |
| 0 | 15 | -0.72 | 0.65 | -0.79 | 0.65 |
|  | 18 | -0.77 | 0.66 | -0.79 | 0.73 |
|  | 20 | -0.78 | 0.67 | -0.77 | 0.68 |
| -0.5*CP | 15 | -0.67 | 0.64 | -0.7 | 0.76 |
|  | 18 | -0.75 | 0.6 | -0.81 | 0.73 |
|  | 20 | -0.84 | 0.59 | -0.83 | 0.65 |
| 0.5*CP | 10 | -1.91 | 0.12 | -1.82 | 0.05 |
|  | 12 | -2.54 | -0.11 | -2.31 | -0.21 |
|  | 15 | -3.94 | -0.67 | -3.39 | -0.86 |
| -0.9*CP | 10 | -1.67 | 0.41 | -1.62 | 0.18 |
|  | 12 | -2.26 | 0.19 | -2.07 | -0.14 |
|  | 15 | -3.72 | -0.16 | -3.18 | -0.75 |
| 0.9*CP | 6 | -2.34 | 0.16 | -2.1 | -0.15 |
|  | 8 | -3.21 | -0.17 | -2.72 | -0.53 |
|  | 10 | -4.38 | -0.59 | -3.54 | -1.01 |
| -CP | 8 | -1.54 | 0.57 | -1.45 | 0.37 |
|  | 10 | -2.26 | 0.35 | -1.92 | 0.07 |
|  | 12 | -3.13 | 0.08 | -2.6 | -0.32 |
| CP | 6 | -3.76 | -0.05 | -3.19 | -0.59 |
|  | 8 | -5.01 | -0.52 | -4.13 | -1.23 |
|  | 10 | -6.44 | -0.97 | -5.22 | -1.95 |

Timing offset side condition and Es/Iot upper bound

In RAN4#98-e, it is agreed that the CSI-SINR accuracy is same as existing SS-SINR accuracy, which is ±3dB for Es/Iot ≥-3dB. To derive the timing offset condition and Es/Iot upper bound, we summarized the maximum Es/Iot under which the delta SINR is ≤±3dB, for each timing offset condition. The results are listed in Table 4.

Table 4: Maximum Es/Iot for different timing offsets

| To | 0 | -0.5*CP | 0.5*CP | -0.9*CP | 0.9*CP | -1*CP | 1*CP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Max Es/Iot | 20 | 20 | 12 | 12 | 6 | 10 | <6 |

Based on the simulation results, we suggest to define the CSI-SINR accuracy based on timing offset condition 0.9*CP and the upper bound Es/Iot 6dB.

Proposal: Define the CSI-SINR accuracy based on timing offset condition 0.9*CP and the upper bound Es/Iot 6dB.

Conclusions

In this paper we provided we provided additional simulation results and our views on the timing offset condition and Es/Iot upper bound for CSI-SINR.

Proposal: Define the CSI-SINR accuracy based on timing offset condition 0.9*CP and the upper bound Es/Iot 6dB.

Reference

- R4-2104089, WF on performance requirements of CSI-RS based L3 measurement, CATT/OPPO

- R4-2103725, Updated link-level simulation assumption for CSI-RS based L3 measurement, CATT