3GPP TSG-RAN WG4 Meeting #98-Bis-e                 	R4-2107124

Electronic Meeting, 12nd – 20th Apr., 2021

Agenda item:		8.9.2

Source:	MediaTek Inc.

Title:	Evaluation on Rel-17 RLM/BFD measurement relaxation

Document for:		Discussion

1	Introduction

In RANP #91 meeting, a guidance to RAN4 has been approved:

RAN Plenary guidance to RAN4: For Rel-17 WI of UE power saving enhancements for NR, no specification impact to RRM measurement procedure requirements and measurement performance requirements is expected

Besides, in RAN4 #98e meeting [1], the agreed work plan are also listed as follows:

- 3GPP RAN4 #98b-e meeting (April, 2021, Study phase) - Discuss and conclude, if agreed, beneficial relaxation method and the corresponding criteria for UE to enter and exit the relaxation mode - A way forward to capture the RAN4 conclusion in study phase

In order to save power consumption for the connected-mode UE and in the meanwhile to guarantee the RLM/BFD performance, comprehensive evaluations are needed. In this paper, the RLM/BFD relaxed measurement is only considered in the scenarios that UE speed is low and SINR is much higher than Qout such that RLF will not be triggered. Based on provided RRM measurement performance analysis, power saving gain analysis, average increased latency analysis and SINR variation analysis, the benefit and feasibility of relaxed RLM/BFD measurement are identified. Furthermore, we also propose the criteria for UE to enter and exit the relaxation mode.

2	RRM measurement accuracy evaluation

In last meeting, it was agreed that corresponding RRM measurement performance should also be checked when companies evaluate the power saving gain [2]. Therefore, in this section we would like to firstly provide the measurement accuracy evaluation results for the cases X=2 and X=4.

Issue 2-2-1: Confirmation on beneficial Scenarios, from UE power saving gain perspective - Agreements - Further evaluate UE power saving gains for the following UE implementations: - UE meets Rel-15 RRM measurement period and accuracy requirements - Option 1: - UE uses all L1 samples for RRM measurements based on Rel-15 assumptions - Option 2: - How many L1 samples UE applies for RRM measurements is up to UE implementation (e.g. UE can use lower number of measurement samples for RRM measurements) - Further discuss how many samples to use for evaluations - Companies shall evaluate RRM measurements accuracy for the proposed number of samples. - FFS whether Option 2 can be considered for requirements definition Further assess impact on PDCCH monitoring due to relax UE measurements for RLM/BFD

As we mentioned, the goal of Rel-17 is to save UE power consumption when UE speed is low and SINR is high. Considering the fact that when SNR of serving cell is very high, the corresponding SNR of neighbouring cell that needs to be reported should also be high. It is possible for UE to skip some measurement samples within 1 PSS/SSS detection/time index detection/measurement period but still meet Rel-15 RRM measurement procedure requirements and accuracy requirements. Here, performance of measurement period is provided as the example to prove the feasibility and option 1 specified in Table II [3] is adopted, i.e., when RLM measurement interval is extended X times, time gap between 2 consecutive RRM measurement samples is also extended X times (given that the measurement period remains the same). The Rel-15 BB measurement accuracy and measurement period are listed as follows:

- BB measurement accuracy within ± 2.0 dB

- Basic measurement period:  max(200ms, ceil(1.5x 5) x max(SMTC period, DRX cycle))

We can see the sample number provided within 1 measurement period is 8 when DRX is configured. It means that 4 samples can be collected similar to the case where the corresponding RLM measurement interval is extended by 2 times; 2 samples can be collected similar to the case where the corresponding RLM measurement interval is extended by 4 times. Table 1 shows the measurement accuracies when SNR is ranged from 0 dB to 8 dB, and UE speed is ranged from 3 km/hr to 30 km/hr. We can see that all the measurement accuracies are within ±1dB, which is much lower than the specified Rel-15 requirement ±2dB. It means that RRM measurement period and accuracy requirements can be guaranteed.

Observation 1: UE can meet RRM measurement procedure requirements and measurement performance requirements when UE speed is lower than 30km/hr and SNR is higher than 0dB.

Table 1: Measurement accuracies for channel type TDL-C with delay spread 300ns

| UE speed (km/hr) | Corresponding RLM scaling factor (X) | Sample number for RRM measurement | Measurement accuracy (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SNR 0dB | SNR 4dB | SNR 8dB |
| 3 | 2 | 4 | ±0.711 | ±0.472 | ±0.339 |
| 3 | 4 | 2 | ±0.945 | ±0.601 | ±0.431 |
| 30 | 2 | 4 | ±0.753 | ±0.492 | ±0.361 |
| 30 | 4 | 2 | ±0.976 | ±0.632 | ±0.442 |

3	Averaged increased latency evaluation

We then discuss the increased latency in RLF triggering. As we shown in below Figure 1, UE would go back to the normal RLM measurement once OOS is indicated, so the worst case increased latency can be obtained by

(X-1)* Rel-15 evaluation period

If we take SSB based RLM as an example, the worst case increased latency seems to be a little bit large when X=4 and DRX cycle=20ms (3*200ms =600ms). However, there exists a SINR threshold SINRrelax for UE to determine whether the RLM measurement relaxation is allowable and that SINR threshold can provide sufficient SINR margin to prevent the miss detection or false alarm.

Figure 1: UE behavior of RLM measurement relaxation with corresponding SINR values

As we displayed in Figure 1, the appropriate SINR margin can be determined by the SLS evaluation results and it variates with the RS type, DRX cycle, evaluation period length, and confidence level that we take. Here, we assume that the criteria for UE to enter or exit the relaxation mode apply the same threshold value SINRrelax. And the probabilities that the averaged SINR is higher than SINRrelax but RLF is still triggered after UE enters the RLM measurement relaxation are counted and shown in Table 2. Even though the value of SINRmargin is only set as 5dB, which is much smaller than the theoretical values that we suggest in section 5	RLM/BFD performance impact, we can see that the probability of RLF triggering is very small and almost equal to zero. If we take 30km/hr as an example, the averaged increased latencies are only 0.0032 ms and 0.0097 ms when X=2 and X=4, respectively. Furthermore, when SINRmargin is further increased to 10dB, the probability of RLF triggering will be reduced more. So we have following observation

Observation 2: Probability of RLF triggering and the corresponding averaged increased latency when UE performs RLM/BFD measurement relaxation can be well controlled by setting a sufficient high SINR criterion.

Table 2: Averaged increased latency of RLF triggering

| UE speed (km/hr) | SINRrelax (dB) | SINRmargin (dB) | Probability of RLF triggering when UE enter the RLM measurement relaxation | Averaged increased latency in RLF triggering (ms) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | X=2 (worst case increased latency 200ms) | X=4 (worst case increased latency 600ms) |
| 3 | -5 | 5 | 0 | 0 | 0 |
| 30 | -5 | 5 | 0.00001619 | 0.0032 | 0.0097 |
| 70 | -5 | 5 | 0.0013 | 0.26 | 0.78 |
| 3 | 0 | 10 | 0 | 0 | 0 |
| 30 | 0 | 10 | 0 | 0 | 0 |
| 70 | 0 | 10 | 0.0004663 | 0.0933 | 0.2796 |

4	UE Power Saving Gain

The total UE power consumption, including recommended Rel-15 / 16 features listed in RAN1 LS [4] and L3 RRM measurement, is evaluated in this section. In addition to the basic agreed assumptions [3], we also assume that

- average UE power consumption is obtained with

  - timing distance between SMTC window and WUS occasion are randomly distributed

  - timing distance between SMTC window and DRX on occasion are randomly distributed

- for SSB based RLM relaxation,

  - In FR1, Psharing factor =1,

  - In FR2, RLM-RS resource is overlapped with the SSB symbols indicated by 
SSB-ToMeasure and Psharing factor =3.

  - SMTC periodicity = 20ms

- for CSI-RS based RLM relaxation,

  - CSI-RS RLM measurement is conducted within DRX on duration

  - CSI-RS RLM measurement resource is fully non-overlapped with RRM measurement resource

  - CSI-RS periodicity = 20ms

- for L3 RRM measurement,

  - meet Rel-15 RRM RRM measurement procedure requirements and measurement performance requirements

  - at least 2 samples will be collected within 1 measurement period

- for L1 RSRP

  - reporting periodicity is set as 160ms based on TR38.840

[UE power consumption]

In last meeting, we provided the evaluation results based on the baseline assumption that UE performs RRM/RLM measurement on each DRX cycle. However, the measurement/evaluation period are already extended with 1.5 times in Rel-15 when DRX is configured. So this time we modify the baseline power consumption as only 5 among 8 samples within 1 measurement period or 10 among 15 samples within 1 evaluation period are collected for UE to perform the RRM/RLM measurement. The updated power saving gain evaluation results are shown as follows:

In FR1, as shown in Figure 2 and Figure 3, when the scaling factor X=4 is applied, the UE power saving gain for SSB based RLM relaxation with DRX cycle (20, 4, 4) and DRX cycle (40, 4, 4) can be up to 7.35%~12.84% and 11.71%~16.68%, respectively. As shown in Figure 4 and Figure 5, when the scaling factor X=4 is applied, the UE power saving gain for CSI-RS based RLM relaxation with DRX cycle (20, 4, 4) and DRX cycle (40, 4, 4) can be up to 8.62%~15.86% and 13.17%~20.34%, respectively. Generally speaking, power saving gain will be more significant when PDCCH-WUS is configured. Furthermore, compare with SSB-based RLM relaxation, the power saving gain for CSI-RS based RLM relaxation will be more significant because we assume that PDCCH will be scheduled after the CSI-RS reporting, so the overall power consumption for CSI-RS based RLM during the DRX on duration will be a little bit more than that for SSB based RLM.

Figure 2: UE power saving gain of SSB based RLM relaxation in FR1 with DRX cycle (20, 4, 4)

Figure 3: UE power saving gain of SSB based RLM relaxation in FR1 with DRX cycle (40, 4, 4)

Figure 4: UE power saving gain of CSI-RS based RLM relaxation in FR1 with DRX cycle (20, 4, 4)

Figure 5: UE power saving gain of CSI-RS based RLM relaxation in FR1 with DRX cycle (40, 4, 4)

In FR2, considering that the SMTC occasions need to be shared between RRM and RLM for SSB based RLM measurement, it can be seen as there exists a scaling factor 3 already. So we do not think RLM measurement relaxation is possible for this scenario. For CSI-RS RLM measurement, as shown in Figure 6 and Figure 7, when the scaling factor X=4 is applied, the UE power saving gain with DRX cycle (20, 4, 4) and DRX cycle (40, 4, 4) can be up to 14.55%~27.44% and 15.97%~28.63%, respectively.

Figure 6: UE power saving gain of CSI-RS based RLM relaxation in FR2 with DRX cycle (20, 4, 4)

Figure 7: UE power saving gain of CSI-RS based RLM relaxation in FR2 with DRX cycle (40, 4, 4)

The detail power state assumptions and corresponding relaxed measurement scheduling can be found in Appendix. We have following observation and proposal.

Observation 3: Ranged from 8.62% to 20.34% UE power saving gain can be obtained for scenarios SSB-based and CSI-RS based RLM/BFD measurement in FR1

Observation 4: Ranged from 14.55% to 28.63% UE power saving gain can be obtained for scenario CSI-RS based RLM/BFD measurement in FR2

Proposal 1: RAN4 to confirm that from UE power saving gain perspective, it is beneficial to relax SSB-based and CSI-RS based RLM/BFD measurement in FR1, and it is beneficial to relax CSI-RS based RLM/BFD measurement in FR2

5	RLM/BFD performance impact

Based on the agreed SLS assumptions [3], we analyze the delta SINR ∆𝑆𝐼𝑁𝑅 (𝑖) to determine the minimum SINR threshold for UE to enter the power saving mode. The ∆𝑆𝐼𝑁𝑅 (𝑖) is the estimated SINR difference that UE obtains by adopting Rel-15 baseline measurement and Rel-17 relaxed measurement, detail can be found in Appendix. Other basic assumptions are:

- UE is only allowed to conduct relaxed RLM/BFD measurement when UE speed is low and SINR is high

- SINR should be larger than following minimum SINR threshold

max (|negative ΔSINR @1% cdf |, | positive ΔSINR @99% cdf |) + Qout (Qout,LR)+ 2dB

  - additional 2dB margin is introduced to make SINR higher than Qout and make RLF unlikely to be triggered.

  - Qout = -10 dB for RLM and Qout,LR = -6 dB for BFD

  - at least 98% confidence level is adopted to minimize the false alarm and/or miss detection rate

We simulate the SINR variation caused by large scaling fading through the system level simulation (SLS), and decide the corresponding maximal scaling factor X that can be applied.

[Maximal scaling factor X determined by SINR variation simulation results]

The evaluation results with 40ms DRX cycle length are shown in this section. The evaluation samples for different RS resources are listed in Table 3.

Table 3: Sample number for RLM/BFD measurement

|  | RLM | BFD |
| --- | --- | --- |
| SSB-based | 10 samples | 5 samples |
| CSI-RS-based | 20 samples | 10 samples |

Given particular UE speed and SINR value, the maximal scaling factor X that can be achieved is listed. In FR1, as shown in Figure 8 and Figure 9, UE is allowed to perform the layer 1 measurement with evaluation period extended at least by 4 times when SINR ≥ 4dB and UE speed ≤ 30km/hr.

Observation 5: In FR1, evaluation period for SSB based and CSI-RS based RLM/BFD measurement can be extended at least by 4 times when SINR ≥ 4dB and UE speed ≤ 30km/hr

Figure 8: Maximal scaling factor X that can be applied for SSB based RLM/BFD relaxation in FR1

Figure 9: Maximal scaling factor X that can be applied for CSI-RS based RLM/BFD relaxation in FR1

In FR2, for SSB based RLM/BFD measurement, relaxed measurement is not suggested with mobility because the number of RX beams, i.e. N=8, also needs to be considered in this scenario. UE can’t guarantee the ACG gain performance in this scenario even for the case X=2. For CSI-RS based RLM/BFD measurement, as shown in Figure 10, UE is allowed to perform the layer 1 measurement with evaluation period extended at least by 2 times when SINR ≥ 8dB and UE speed ≤ 30km/hr. Comparing with FR1, larger SINR variation is observed in FR2.

Observation 6: In FR2, evaluation period for CSI-RS based RLM/BFD measurement can be extended at least by 2 times when SINR ≥ 8dB and UE speed ≤ 30km/hr

Figure 10: Maximal scaling factor X that can be applied for CSI-RS based RLM/BFD relaxation in FR2

The SINR fluctuation in DRX cycle 20ms should be lower than that in DRX cycle 40ms, so we can determine the SINR threshold based on the worst case and only refer to the SLS evaluation results in DRX cycle 40ms. Based on the aforementioned results, we have following proposals

Proposal 2: RAN4 to confirm that from system impact perspective, SSB based and CSI-RS based RLM/BFD measurement relaxation in FR1 and CSI-RS based RLM/BFD measurement relaxation in FR2 are feasible for low mobility and high SINR UE.

Proposal 3: RAN4 to specify that requirement of Rel-17 connected mode power saving at least for the following scenarios: 1) SSB based and CSI-RS based RLM/BFD measurement relaxation in FR1, and 2) CSI-RS based RLM/BFD measurement relaxation in FR2

6	Criteria for UE to enter and exit the relaxation mode

In last meeting, companies agreed to discuss the criteria for UE to enter and exit the relaxation mode.

- Issue 2-3-3: How to consider serving cell’s quality as relaxation criteria - RAN4 to further discuss how to take serving cell’s quality into account for the relaxation criteria - FFS how to consider serving cell’s quality. E.g. Based on SINR or BLER. - FFS how to address different UE implantation issues. - FFS: When radio link quality > Qout + X (dB) for RLM and Qout,LR + Y (dB) for BFD relaxation - X and Y are FFS. - Issue 2-5-1: Reverting to the normal RLM operation The UE while performing relaxed RLM upon detecting certain number of out-of-sync indications or upon triggering T310 or upon observed link quality degradation or mobility state change reverts to the normal RLM operation (i.e. without relaxation). - FFS the following options - Option 1a: revert when the relaxation criterion is not met - Option 1b: revert when N310 starts to count, i.e. 1 out-of-sync indication. - Option 1c: revert when T310 is running, i.e. N310 out-of-sync indication. - Option 1d: revert when observed link quality degradation. - Option 1e: revert regarding observed mobility state change. - Other options are not precluded - Issue 2-3-2: How to consider UE mobility as relaxation criteria - FFS the following options - Option 1: R16 low-mobility criterion should not be directly reused in R17 SINR-based criterion for RLM/BFD relaxation. - Option 2: R16 RRM relaxation criterion can be used as baseline for RLM/BFD relaxation. - Option 3: “low mobility criteria” should consider both UE velocity and the channel quality variation. - Option 4: Consider time associated with a given condition when determining UE mobility state. - Option 5: Low mobility scenario under which the UE is allowed to apply the RLM/BFDBM requirements is determined and configured to UE by the network - Other options are not precluded

As we shown in the previous section, as long as there exists a SINR threshold for UE to determine whether the RLM measurement relaxation is allowable, it is possible for UE to prevent the miss detection or false alarm. Considering that for each UE, there exists a one-to-one mapping between BLER value and SINR value, it seems to us that these 2 options of the serving cell’s quality criteria are equivalent. So we suggest that RAN4 can agree on “applying particular BLER as the relaxation criteria” in the core part then determine the corresponding SINR value in the performance part.

Proposal 4: RAN4 to specify BLER (or SINR) threshold for UE to enter the relaxation mode

As for the mechanism for UE to revert to the normal RLM measurement, it is obviously that UE should back to normal measurement when N310 starts to count or when relaxation criterion is not met. Given with extended evaluation period, RAN4 had reached consensus on the evaluation assumption and some results were already provided to prove that conventional Qout and Qout,LR can still be applied. However, if evaluation period can’t be extended for the relaxation mode, RAN4 should start over the LLS evaluation to confirm the corresponding RLM/BFD accuracies for different sample number. RAN4 might not be able to make conclusion in this meeting. So we propose RAN4 to agree at least BLER (or SINR) threshold will be specified for UE to exit the relaxation mode. Whether evaluation period can be extended should be for future study and determined based on the evaluation results.

Proposal 5: RAN4 to specify BLER (or SINR) threshold for UE to exit the relaxation mode

Proposal 6: For future study whether to specify 2 different threshold values for UE to enter and exit the relaxation mode

Regarding the UE mobility relaxation criteria, the intension of Rel-16 low mobility criterion is to check whether the fluctuation of RSRP value is within certain level, which has no directly relationship with the SINR fluctuation. We prefer not to reuse Rel-16 low mobility criteria in Rel-17. A possible solution is to create a Rel-17 low mobility criterion that reflect whether the fluctuation of SINR value is within certain level, such as

(SINRref -SINRcurrent) < SINRthreshold

where

- SINRcurrent = current SINR value of serving beam

- SINRref = reference SINR value of serving beam, set as follows:

  - After selecting to a new beam, or

  - If (SINRcurrent -SINRref)>0, or

  - If the relaxed criteria has not been met for a period of time T

However, some simulation results in Rel-16 had already been provided to show that low mobility criterion is not precisely reflect the UE speed and serving cell’s quality criterion is sufficient enough for UE to determine whether relaxation is allowable. The advantage for applying only 1 criterion for Rel-17 power saving is that will simplify the relaxation rules a lot. Rather than indicates 2 different types of criteria, it is simpler that Network indicates different SINR threshold values for different scaling factor X. Therefore, we suggest that

Proposal 7: RAN4 to study the necessity of mobility criterion for Rel-17 power saving

7	Summary

In this contribution, we have the following observations:

Observation 1: UE can meet RRM measurement procedure requirements and measurement performance requirements when UE speed is lower than 30km/hr and SNR is higher than 0dB.

Observation 2: Probability of RLF triggering and the corresponding averaged increased latency when UE performs RLM/BFD measurement relaxation can be well controlled by setting a sufficient high SINR criterion.

Observation 3: Ranged from 8.62% to 20.34% UE power saving gain can be obtained for scenarios SSB-based and CSI-RS based RLM/BFD measurement in FR1

Observation 4: Ranged from 14.55% to 28.63% UE power saving gain can be obtained for scenario CSI-RS based RLM/BFD measurement in FR2

Observation 5: In FR1, evaluation period for SSB based and CSI-RS based RLM/BFD measurement can be extended at least by 4 times when SINR ≥ 4dB and UE speed ≤ 30km/hr

Observation 6: In FR2, evaluation period for CSI-RS based RLM/BFD measurement can be extended at least by 2 times when SINR ≥ 8dB and UE speed ≤ 30km/hr

And we propose

Proposal 1: RAN4 to confirm that from UE power saving gain perspective, it is beneficial to relax SSB-based and CSI-RS based RLM/BFD measurement in FR1, and it is beneficial to relax CSI-RS based RLM/BFD measurement in FR2

Proposal 2: RAN4 to confirm that from system impact perspective, SSB based and CSI-RS based RLM/BFD measurement relaxation in FR1 and CSI-RS based RLM/BFD measurement relaxation in FR2 are feasible for low mobility and high SINR UE.

Proposal 3: RAN4 to specify that requirement of Rel-17 connected mode power saving at least for the following scenarios: 1) SSB based and CSI-RS based RLM/BFD measurement relaxation in FR1, and 2) CSI-RS based RLM/BFD measurement relaxation in FR2

Proposal 4: RAN4 to specify BLER (or SINR) threshold for UE to enter the relaxation mode

Proposal 5: RAN4 to specify BLER (or SINR) threshold for UE to exit the relaxation mode

Proposal 6: For future study whether to specify 2 different threshold values for UE to enter and exit the relaxation mode

Proposal 7: RAN4 to study the necessity of mobility criterion for Rel-17 power saving

Reference

- R4-2103669, “Work Plan of R17 UE Power Saving Enhancements WI,” MediaTek Inc., Jan., 2021.

- R4-2103670, “WF on NR UE Power Saving Enhancements,” MediaTek, VIVO, Nov., 2020.

- R4-2104066, “Updated evaluation assumptions for R17 RLM/BFD relaxation,” VIVO, MediaTek, Nov., 2020.

- R1-2007419, “LS on evaluation methodology for connected mode UE power saving enhancements,” VIVO, Aug., 2020.

Appendix

[Power state assumption]

Corresponding power states are listed as follows:

Figure 11: Power state assumption in FR1

Figure 12: Power state assumption in FR2

[Evaluation assumption for power saving gain]

Scheduling of relaxed measurement schemes are shown as follows:

Figure 13: Power saving gain evaluation assumption in FR1

Figure 14: Power saving gain evaluation assumption in FR2

[SINR variation]

Delta SINR is the estimated SINR difference that UE obtains by adopting Rel-15 baseline measurement and Rel-17 relaxed measurement. If we take Rel-15 RLM as an example, during the overall evaluation period, UE averages the SINR of N samples. The similar way can also be applied in Rel-17, the only difference is that the time interval between each sample should be further multiplied with X, where X is the scaling factor that needs to be determined through the SLS evaluation. The detail formula is shown as follows, where SINRi is the SINR sample obtained at time i.

∆SINRi=1Nj=1NSINRi+X-j*X-1Nj=1NSINRi+1-j

Below we provide an example of Rel-17 relaxed method with N=10 and X=2. If we would like to obtain the averaged SINR value in the time index 19:

- for Rel-15 baseline method, samples with indices 10 to 19 will be used;

- for Rel-17 relaxed method, samples with indices 1, 3, 5… to 19 will be used.

Figure 15: Example of relaxed method with N=10 and X=2

How can we justify the performance of relaxed method based on this performance metric? The analysis of the consequences of under-estimation and over-estimation for Rel-17 relaxed method would be helpful. As shown in Figure 16, while Rel-17 relaxed method is adopted, there might exist under-estimation problem and over-estimation problem.

- Under-estimation (false alarm):

It happens when the averaged SINR value obtained from Rel-15 method is actually higher than Qout, but UE wrongly thinks that the averaged SINR is lower than Qout just because it applies the Rel-17 relaxed method. The delta SINR will be a negative value and it will cause a false alarm. UE might trigger the RLF too early.

- Over-estimation (miss detection):

It contrast, when the averaged SINR value obtained from Rel-15 method is actually lower than Qout, but UE wrongly thinks that the averaged SINR is higher than Qout just because it applies the Rel-17 relaxed method. The delta SINR will be a positive value and it will cause a miss detection. UE might trigger the RLF too late.

Figure 16: consequences of under-estimation and over-estimation for Rel-17 relaxed method

To avoid the false alarm and/or miss detection, one simple way that can be considered is to set an SINR threshold for UE to enter the power-saving mode. If UE is only allowed to conduct relaxed RLM/BFD measurement when its SINR is larger than an allowed margin plus Qout

max (|negative ΔSINR |, | positive ΔSINR |) + Qout,

we can theoretically guarantee that the Rel-17 method will have the same performance as the Rel-15 method. Besides, based on this assumption, there is no need to further consider the Qin. False alarm or mis-detection on Qin is not a problem if it is guaranteed that UE never has a chance to indicate Qout.