3GPP TSG-RAN WG4 Meeting #98bis-e														R4-2107189

Electronic meeting, April 12th – 20th February 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Bandplan for a NR band in the range 52.6GHz – 71GHz

Agenda item:	8.12.2

Document for:	Approval

# Introduction

In [1] the following texts have been captured in common part in the objectives:

According to the outcome of the study item on Supporting NR above 52.6GHz and leveraging FR2 design to the extent possible, this WI extends NR operation up to 71GHz considering, both, licensed and unlicensed operation, with the following objectives

In addition, in [1] the following objectives have been captured in RAN4 core part:

  - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range.

  - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1).

  - Specify RRM/RLM/BM core requirements.

Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives.

# Discussion

It is clear that the WI covers both licensed and unlicensed operation as feature while band(s) definition including its frequency range(s) and licensed/unlicensed are not clearly defined. The introduction of new bands is normal practice in RAN4 based on operator requests and the availability of regulations for deployments in specific region or country.

#### For unlicensed deployments

As was described in the current WID, in order to complete the WI, requirements for at least one band combination involving a new NR-U band need to be specified. It means that defining at least one band for unlicensed is essential for this end.

In [2] the following table have been captured:

Table 4.2.1-1: Current Licensing situation for various countries for frequency range between frequency 52.6GHz and 71GHz

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-711) |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U (Mobile) |  |  |  | U (Mobile) |  |  |
| ITU Region 2 | USA |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| NOTE 1: Access regime currently under discussion in CEPT |  |  |  |  |  |  |  |  |  |  |  |  |

Since the TR of 38.807 [2] was last updated in Dec RAN Plenary in 2019, some of the information is obsolete. In our understanding, there is now a greater likelihood that 66 -71 GHz can be used as unlicensed in EU and Canada. Provided that at least to cover US band, a band to cover 57 - 71 GHz is definitely required, the band would be also suitable for EU and Canada in the end and it can cover all the region in terms of frequency range, we believe that defining a band of 57 - 71 GHz is the most reasonable.

Proposal 1: Define a band (57 - 71 GHz) for unlicensed operation in countries where regulations are available.

According to Table 4.2.1-1 a band for the full range from 57 – 71 GHz will cover at least deployments in US and EU. Currently regulations are being developed for Europe by ETSI BRAN as a part of the ETSI EN 303 753 harmonized standard [3]. Previously when a new unlicensed band have been introduced to 3GPP RAN4 NR specification a starting point for the requirements have been the harmonized standard. Examples of this is the 5GHz (n46) and 6GHz (n96) band which is aligned to the EN 301 893 and EN 303 687 respectively. Additional regional requirements are indicated via NS signaling for the UE and on the basis of regional compliance responsibility of the deployer of a BS. It is proposed to follow same approach for an unlicensed 60 GHz band for simplicity and alignment to other technologies sharing the same spectrum which are also following same harmonized standard.

Proposal 2: For an unlicensed 60 GHz band (57 - 71 GHz) adopt requirements from the ETSI EN 303 753 harmonized standard where applicable.

#### For licensed deployments

At least we are not aware of surely available licensed spectrum anywhere in 52.6 - 71 GHz. Hence, it is premature to discuss a specific band definition for licensed band.

Proposal 3: Postpone discussing a band definition for licensed band until spectrum availability becomes clear enough.

# 3	Conclusions

In this contribution, we provided our views related to how to handle the new frequency range from 52.6GHz-71GHz, and we have discussed how to move forward with licensed and unlicensed bands for this same frequency range. We have made the following proposals:

Proposal 1: Define a band (57 - 71 GHz) for unlicensed operation in countries where regulations are available.

Proposal 2: For an unlicensed 60 GHz band (57 - 71 GHz) adopt requirements from the ETSI EN 303 753 harmonized standard where applicable.

Proposal 3: Postpone discussing a band definition for licensed band until spectrum availability becomes clear enough.

# 5	References

# [1] RP‑210862, Revised WID on Extending current NR operation to 71GHz

# [2] TR 38.807 v16.0.0, Study on requirements for NR beyond 52.6 GHz

[3] draft EN 303 753 v0.0.3, ETSI BRAN