3GPP TSG-RAN WG4 Meeting #98bis-e														R4-2107197

Electronic meeting, April 12th – 20th February 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On system parameters for the lower 6GHz NR unlicensed operation

Agenda item:	9.34.1

Document for:	Approval

# Introduction

During RAN#89-e a WID was approved for a lower 6GHz NR unlicensed operation intended for deployment in Europe. The start of the WID was delayed pending administrations in Europe have had unlicensed operation in the range 5925 to 6425 MHz for consultation. The result of this consultation is that at the November 2020 meeting the ECC with ECC Decision (20)01 “on the harmonized use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs)” approved unlicensed operation in the range 5945 MHz to 6425 MHz. Given the ECC decision the work was at RAN#90 approved to start in RAN4#98 [1].

NR-Unlicensed operation is standardized in Rel-16 [3] with the definition of band n96 covering the spectrum range 5925 MHz -7125 MHz which is currently applicable in the USA only subject to FCC Report and Order FCC 20-51. The ECC decision covers the range 5945 MHz to 6425 MHz which is therefor a sub-set of the already defined band n96.

The objectives for the core part from the WID [1] relevant for the UE are given as:

- Depending on the details of the European regulatory requirements, determine whether they are best handled by relevant updates (if any) of band n96 or whether a new band is needed.

  - If a new band is needed, determine the band plan for unlicensed operation in the range 5945-6425 MHz

- Define or update (if needed) system parameters such as channel bandwidths and channel arrangements

- Define or update (if needed) transmitter and receiver characteristics requirements for the UE

During RAN4#98 a WF was agreed in [4] including the following general agreements:

General - Band plan:

- Unlicensed operation in the range 5945-6425 MHz can be introduced by:

  - Option 1: Re-using already defined band n96

    - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

  - Option 2: Defining a new band n[xx]

    - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

- NR-ARFCN and GSCN already defined for n96 should be re-used in the frequency range applicable to 6GHz NR unlicensed operation in Europe. This is independent if n96 is reused or a new band is defined.

General - LPI and VLP deployment:

- LPI devices to be supported by 3GPP Specifications

- VLP devices can be supported by the 3GPP specification, VLP deployments and devices are subject to further checking of regulations

- 0 dBi antenna gain is assumed for the purpose of deriving 3GPP specifications on MOP, PSD, and ASE for LPI deployments and, if introduced, also VLP deployments.

- VLP deployment and device types are subject to further checking of regulations

The objectives of the WID together with the agreements so far are discussed further in this contribution.

# Discussion

#### Band Plan

As listed in the WF [4] two options are discussed for the introduction of a NR band for unlicensed operation in Europe in the frequency range 5945 MHz to 6425 MHz. The choice is to either leverage already defined n96 or introduce a new band. As argued in previous RAN4#98 meeting our opinion is that there is no need for a new band as we can utilize n96 with additional restrictions specific for Europe (EU) by the Harmonised Standard (HS) for access to radio spectrum in the 6 GHz range [2]. As a result, our preference is option 1 from the WF[4].

Proposal 1:	Introduce the 5945 MHz to 6425 MHz frequency range for unlicensed operation in Europe by adopting option 1 i.e. re-use already defined n96.

The needed additional restrictions for the UE and BS specification is discussed further in [5] and [6] respectively.

The reasoning for adopting option 1 is that design used for band n96 follows the approach, also adopted earlier for band n46, which is aligned to the European Harmonised Standard for access to radio spectrum in the 6 GHz range [2]. This implicitly means co-existence aspects out of the band as well as within, via e.g. WiFi channel alignment, are considered.

The difference from band n96, originally intended for US deployments, and the frequency range for EU is that band n96 starts at 5925 MHz while the EU range starts at 5945 MHz and that the US range goes to 7125 MHz while the EU range stops at 6425 MHz. Even though the starting frequency is different for the US and EU ranges the channel and synchronization raster are still aligned since the 20 MHz shift have already been accounted for in the design of band n96. The main difference is that for the US range will have additional available raster points corresponding to the upper range of band n96 (i.e. 6425 MHz – 7125 MHz).

Regarding the limitation of n96, for US operation only, it can be noted that South Korea [7] and Brazil [8] have adopted the same frequency range (i.e.  5925 – 7125MHz). This as wells as the fact that other countries are considering the same would imply that the restriction of n96 for US operation only should be revisited.

Proposal 2:	The restriction of n96, for US operation only, should be discussed by RAN4.

It should be emphasised that even by removing the regional restriction in the 3GPP specification for unlicensed operation in band n96 they are still enforced by NS signalling for the UE and regulatory compliance requirements for the BS. Further, there should be no doubt that if/when regulations are ready for licensed operation in either the same or parts of the same frequency range a band shall per normal RAN4 procedure be defined accordingly.

Observation 4: 	Enabling wider use of band n96 for unlicensed operation does not in any way preclude a future introduction of a licensed band covering either the entire or parts of the same frequency range.

#### LPI and VLP deployment

When reviving the latest version 0.0.12 of the Harmonised Standard for access to radio spectrum in the 6 GHz range approved March 12th, 2021 [2], it can be found that VLP (Very Low Power) deployment are still “To be further defined”. As a result, if is suggested to prioritize the introduction of LPI (Low Power Indoor) by RAN4 but still enable VLP by introducing the requirements already included in the Harmonised Standard.

Proposal 3:	Introduce LPI deployment to 3GPP specification with priority but also include VLP with available requirements.

Another topic to consider is the allowance of NB usage by the ECC Decision (20)01 which allows channels lower than 20 MHz with a maximum ERIP density of 10 dBm/MHz. These narrowband devices are also requiring to perform frequency hopping with at least 15 hop per channels to operate at a PSD value above 1 dBm/MHz.

Proposal 4:	RAN4 to investigate NB deployments in the 6GHz band.

# Conclusion

In this contribution, we discuss the 6 GHz band for unlicensed operation in Europe. We have made following observations and proposals:

Proposal 1:	Introduce the 5945 MHz to 6425 MHz frequency range for unlicensed operation in Europe by adopting option 1 i.e. re-use already defined n96.

Proposal 2:	The restriction of n96, for US operation only, should be discussed by RAN4.

Proposal 3:	Introduce LPI deployment to 3GPP specification with priority but also include VLP with available requirements.

Proposal 4:	RAN4 to investigate NB deployments in the 6GHz band.

# References

- RP-202592, WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Nokia Shanghai Bell

- Draft EN 303 687 v0.0.12, ETSI BRAN

- RP-200312 Revised WID on NR-based Access to Unlicensed Spectrum; rapporteur: Qualcomm

- R4-2103229, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Qualcomm, Charter Communications Inc.

- R4-2107198, On UE RF aspects for the lower 6GHz NR unlicensed operation, Nokia

- R4-2107199, On BS RF aspects for the lower 6GHz NR unlicensed operation, Nokia

- Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

- National Telecommunications Agency (ANATEL), ACT NO. 1306 OF 26 FEBRUARY 2021