3GPP TSG-RAN WG4 Meeting#98bis             	R4-2107204

E-meeting, Apr. 12th – 20th, 2021

Title:	TP to TR 37.717-21-11 Addition of DC_46-48_n66

Source:	Nokia, Verizon

Agenda item:	7.4.2

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-21-11 to introduce DC_46A-48A_n66A, DC_46C-48A_n66A, DC_46D-48A_n66A and DC_46E-48A_n66A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.x	DC_46-48_n66

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |
| NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations of DC_48A_n66, where:

- 2nd and 3rd IMD product caused by DC_48A_n66A may fall into own Rx of band 46.

### 5.x.3	∆TIB and ∆RIB values

### Values are reused from CA including same bands, CA_46-48-66, as given in 36.101.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_46-48_n66 | 46 | 0 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_46-48_n66 | 46 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |

### 5.x.4	Reference sensitivity exceptions

Based on co-existence analysis it is found that MSD is needed due to 2nd and 3rd IMD falling into own Rx band of 46. Therefor additional MSD is needed defined in Table 7.3B.2.3.5.2-1 of 38.101-3.

Table 5. x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_46A-48A_n66A5 DC_46C-48A_n66A5 DC_46D-48A_n66A5 DC_46E-48A_n66A5 | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD3 |
|  | 48 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + DFHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. |

*********************** End of the TP ***************************************************

# 3	References