3GPP TSG-RAN4 Meeting # 98bis-e	R4-2107226

# Electronic Meeting,  12nd– 20th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | <CR#> | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Tx signal quality requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 2021-4-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | EVM detailed procedure is missing in Annex D and E |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EVM detailed procedure is in Annex D and E |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EVM detailed procedure is missing in Annex D and E |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D and E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## < start of changes >

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

[3]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[6]	3GPP TS 38.133: “NR: Requirements for support of radio resource management”

[7]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[8]	3GPP TS 38.211: "NR; Physical channels and modulation”.

[9]	3GPP TS 38.212 "NR; Multiplexing and channel coding".

[10]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[11]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[15]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[16]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[17]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain".

[18]	ITU-R Recommendation M.1545: “Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications – 2000”

[19]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[20]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[21]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing".

[22]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing; Part 1: Conducted conformance testing".

[23]	3GPP TS 38.521-1: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone ".

[24]	3GPP TS 38.521-2: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone ".

## << Unchanged part is omitted>>

Annex D (normative):

IAB-MT Error Vector Magnitude (FR1)

The Annex B in TS 38.104 [2] applies to IAB-MT. Alternatively EVM procedure for CP-OFDM in Annex E in TS 38.521-1[23] applies to IAB-MT.

Annex E (normative):

IAB-MT Error Vector Magnitude (FR2)

The Annex C in TS 38.104 [2] applies to IAB-MT. Alternatively EVM procedure for CP-OFDM in Annex E in TS 38.521-2 [24] applies to IAB-MT.

##### << Unchanged part is omitted>>

## < end of changes >