3GPP TSG-RAN WG4 Meeting #98bis-e	R4- 2107268

Electronic Meeting, 12 – 20 April 2021

Source: 	Huawei, HiSilicon

Title:	Consideration on high order band combinations for NR PC2 inter-band CA

and SUL

Agenda Item:	7.22.2

Document for:	Approval

# 1	Introduction

- Specify the band-combination specific RF requirements for all listed power class 2 NR inter-band UL CA  and SUL configurations with power class 2 on NUL and power class 3 on SUL (xDL/yUL, x>2, y=1, 2).

- Analysing combinations that have self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

- Other additional impact to the requirements due to the high power on UL, if necessary.

Note: for a band combination, the common requirements are expected.

The power class 2 NR inter-band CA band combination configurations are defined in the table 1 below:

Table 1: Power class 2 NR Inter-band CA for x (x>2) bands DL with y (y=1, 2) bands UL

| NR CA configuration | Uplink CA configuration | contact name, company | Contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n78A | CA_n1A-n78A | Basaier Jialade China Unicom | basejld@chinaunicom.cn | Huawei, HiSilicon, ZTE | new |
| CA_n1A-n3A-n78A | CA_n3A-n78A | Basaier Jialade China Unicom | basejld@chinaunicom.cn | Huawei, HiSilicon, ZTE | new |

In RAN#91, the WI Power Class 2 UE for NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL was [1]. The WI objectives are reproduced below.

In the following, we provide further clarification on the WI and propose the general procedure for the work to be carried out.

# 2	Discussion

According to the WID [1], three types of band combinations are envisaged, i.e.,

- PC2 CA with more than two bands DL and one band UL

- PC2 CA with more than two bands DL and two bands UL

- PC2 CA with more than two bands DL and one SUL band combination

For all cases, the sources of self-interference should be similar to the same band combinations in PC3, except that the resultant MSD could be larger due to higher Tx power. Since there is effectively only one UL carrier transmitting in case 1 and 3, the MSD analysis should focus on harmonic interference, harmonic mixing and cross-band isolation. Additionally, MSD due to IMD from dual UL should be studied for case 2. Owing to the similarity, it’s beneficial to complete the same band combination in PC3 before proceeding to PC2.

Proposal 1: It is recommended to complete a band combination in PC3 before proceeding to PC2.

This basket WI allows multiple DL bands to be specified. However, as a general practice in 3GPP, it is recommended to complete the lower order fallback combinations first before proceeding to the high order ones.

Proposal 2: It is recommended to complete low order band combinations before proceeding to high order ones.

Recommended procedure:

- Request new band combinations on the 3GPP NR band and combo email reflector. An example spreadsheet is provided as a companion to this tdoc.

- Submit TPs to RAN4 meetings for approval. A TR skeleton is provided in [2], which may serve as a template for the TPs.

- After approval, the rapporteur adds the TPs to the TR and prepare relevant CRs for block approval.

The usual procedure from band combination request to TP approval should be followed, which is elaborated below.

Proposal 3: It is recommended to follow the above procedure for any work under this WI.

# 3	Conclusion

The new WI on PC2 NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL is introduced. And the following proposals are presented.

Proposal 1: It is recommended to complete a band combination in PC3 before proceeding to PC2.

Proposal 2: It is recommended to complete low order band combinations before proceeding to high order ones.

Proposal 3: It is recommended to follow the above procedure for any work under this WI.

# References

[1] RP-210543 New WID on Power Class 2 UE for NR inter-band CA and SUL configurations with x (x>2) bands DL and y (y=1, 2) bands UL, Huawei, HiSilicon, China Unicom, RAN#91, March 2021

[2] R4-2107276 Draft TR 38.xxx High power UE (power class 2) for NR inter-band CA and SUL with x (x>2) bands DL and y (y=1, 2) band UL, Huawei, HiSilicon, RAN4#98bis-e, April 2021