3GPP TSG-RAN WG4 Meeting #98-bis-e                                       R4-2107298 Electronic Meeting, Apr. 12-20, 2021

Source: 	Huawei, HiSilicon

Title: 	REFSENS analysis for FDD HPUE

Agenda Item:	9.4.3

Document for:	Approval

# Introduction

Issues on FDD HPUE have been discussed in last RAN4 meeting, and a WF listed some issues to be further studied [1].

This contribution provides our analysis on the REFSENS impact for FDD HPUE.

# Discussion

According to the WF, RAN4 need to study receiver sensitivity degradation due to high max output power and Tx/Rx isolation levels according to RF component performance in both n1 and n3 FDD bands. Since n3 has much closer Tx/Rx separation, the analysis is focused on n3, and with similar calculation, the analysis can be easily extended to n1. Existing REFSENS for Band n1 and n3 are listed in Table 1.

Table 1: REFSENS for Band n1 and n3

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 | -90.6 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 | -90.7 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 | -90.9 |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -82.3 |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -82.4 |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -82.6 |

As we discussed in [2], considering the maximum rating capability of existing RF components, the focus here is for 2Tx UE architecture. 40MHz is considered in the analysis.

The PA noise floor usually used in RAN4 is about -130dBm/MHz. Here we consider a pseudo worst case to use -120dBm/MHz Tx noise floor instead to see whether PC2 UE could cause REFSENS degradation. Duplexer Tx/Rx isolation can be easily reached for larger than 50dB according to the information of a commercial component as below.

Table 1: Tx/Rx isolation for a commercial quadplexer

The parameters to be used in the analysis are listed in Table 2.

Table 2: REFSENS impact analysis for 2Tx UE architecture

|  | Value | Unit |
| --- | --- | --- |
| REFSENS (40MHz) | -82.3 | dBm/40MHz |
| SNR | -1 | dB |
| Noise Floor | -81.3 | dBm/40MHz |
| Tx noise floor at 1880MHz | -120 | dBm/MHz |
| Dup Tx/Rx isolation | 50 | dB |
| 2Tx UE Tx noise floor | -120+3=-117 | dBm/MHz |
|  | -51 | dBm/40MHz |
| Rejected noise floor | -51-50=-101 | dBm/40MHz |
| total noise floor | -81.3+(-101)=-81.25 | dBm/40MHz |
| rise of noise floor | 0.05 | dB |

From the simple calculation in above table, we see that the noise floor caused by increased power for PC2 HPUE is negligible. For other channel BWs and the case for band n1, the results are similar.

Observation: For 2Tx PC2 UE, noise floor for Band n3 and n1 in the Rx band is negligible.

# Conclusion

This contribution provides analysis for the REFSENS degradation for the increase Tx noise floor at Rx band for band n3 and n1.

Observation: For 2Tx PC2 UE, noise floor for Band n3 and n1 in the Rx band is negligible.

Proposal 1: For specific band n3 and n1, it can be concluded that there is no obvious REFSENS degradation for PC2 FDD HPUE.

# References

[1] R4-2103273, WF on PC2 for NR FDD band, China Unicom

[2] R4-2107299, On feasibility of RF component to support FDD HPUE, Huawei, HiSilicon