3GPP TSG-RAN WG4 Meeting #98-bis-e                                       R4-2107300

Electronic Meeting, Apr. 12-20, 2021

Source: 	Huawei, HiSilicon

Title: 	System performance evaluation for FDD HPUE

Agenda Item:	9.4.5

Document for:	Approval

# Introduction

WF on simulation assumptions for system performance evaluation was agreed in last RAN4 meeting. In which both dynamic system level and Monte Carlo based simulation assumptions are included [1].

Initial system level simulation results were provided in [2]. This contribution provides our further results according to the updated assumptions in the WF.

# Discussion

## Simulation assumptions

Table 1 provides the updated simulation assumptions based on [1].

Table 2.1-1: Simulation assumptions

| Configuration parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| ISD | 500 m |
| Duplexing | FDD |
| Carrier frequency | 1.8 GHz, 2.1GHz |
| Modulation | Up to 64QAM |
| Numerology | 15 kHz |
| Simulation bandwidth | 40 MHz |
| Transmission scheme | SU-MIMO |
| Codebook | For 2Tx, codebook [1 1]T is used for transmit diversity |
| SU dimension | 1 layer |
| Antenna configuration at TRxP | 4Rx, (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) 32Rx, (M,N,P,Mg, Ng) = (8,8,2,1,1; 2,8) |
| Antenna configuration at UE | 1Tx, (M,N,P,Mg, Ng) = (1,1,1,1,1; 1,1), 2Tx, (M,N,P,Mg, Ng) = (1,1,2,1,1; 1,1), |
| UE maximal transmit power | For 1Tx, 23 dBm for each TXRU For 1Tx, 26 dBm for each TXRU (High power UE) For 2Tx, 23 dBm for each TXRU (High power UE) |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Power control parameter | P0=-60, alpha = 0.6 P0 = -76, alpha = 0.8 P0=-60, alpha = 0.8 |
| TRxP number per site | 3 |
| TRxP number | 21 |
| Channel model | UMa following TR 38.901 |
| Electronic tilt | 102° |
| Traffic model | FTP3, packet size: 100k/10k Byte, arrival rate: 1 packet / 200ms. |
| Uplink duty cycle | 50%（PC 2UE）, 100%（PC 3 UE） |

## Simulation results

In the simulation assumptions, some parameters have more than one set of values. For the different values, more simulation cases are performed.

Table 2 provides the simulation results.

Table 2.2-1: Simulation results for P0 = -76, alpha = 0.8

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/200ms | 150.11% | 100.03% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/200ms | 147.90% | 101.67% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/200ms | 150.02% | 107.12% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/200ms | 150.18% | 105.24% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 159.56% | 105.72% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/200ms | 157.07% | 105.72% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/200ms | 159.78% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/200ms | 159.77% | 108.69% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/1s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/1s | 151.88% | 100.82% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/1s | 156.11% | 100.53% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/1s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/1s | 152.96% | 102.23% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/1s | 155.44% | 100.15% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/1s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/1s | 161.03% | 106.45% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/1s | 160.06% | 106.45% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/1s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/1s | 161.34% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/1s | 160.23% | 108.69% |

Table 2.2-1: Simulation results for P0 = -60, alpha = 0.6

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/200ms | 148.14% | 100.03% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/200ms | 143.43% | 108.01% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/200ms | 151.49% | 100.97% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/200ms | 151.38% | 104.57% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 160.12% | 105.72% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/200ms | 157.12% | 105.72% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/200ms | 163.25% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/200ms | 163.12% | 108.69% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/1 s | 153.16% | 100.78% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/1 s | 151.74% | 104.03% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/1 s | 155.10% | 102.41% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/1 s | 154.49% | 102.41% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/1 s | 162.77% | 106.45% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/1 s | 161.15% | 106.45% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/1 s | 164.53% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/1 s | 164.49% | 108.69% |

Table 2.2-1: Simulation results for P0 = -60, alpha = 0.8

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/200ms | 132.77% | 100.02% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/200ms | 134.44% | 100.81% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/200ms | 127.14% | 100.52% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/200ms | 126.51% | 102.70% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 133.91% | 105.12% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/200ms | 132.87% | 105.12% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/200ms | 137.62% | 107.41% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/200ms | 136.81% | 107.41% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/1 s | 127.75% | 100.63% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/1 s | 128.25% | 101.69% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/1 s | 125.74% | 101.48% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/1 s | 124.84% | 105.37% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/1 s | 133.45% | 106.45% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/1 s | 133.46% | 106.45% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/1 s | 137.95% | 107.41% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/1 s | 137.70% | 107.41% |

Observation 1: From the simulation results, performance gain can be observed for both cell average and cell edge cases. And extremely obvious performance gain on cell average throughput is observed.

# Conclusion

This contribution provides simulation results on system performance evaluation for FDD HPUE according to the updated assumptions in last RAN4 meeting. Based on the results, we see obvious performance gain.

Observation 1: From the simulation results, performance gain can be observed for both cell average and cell edge cases. And extremely obvious performance gain on cell average throughput is observed.

Proposal 1: It is proposed not to spend more time to justify the obvious system performance gain for FDD HUPE and focus on other remaining issues.

# References

[1] R4-2103274, WF on simulation assumptions for system performance evaluation, China Unicom

[2] R4-2102392, System performance evaluation for FDD HPUE, Huawei, HiSilicon