3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2104411

E-meeting, 12th – 20th  April, 2021

Title:	TP to TR 37.717-21-11: DC_5A-30A_n2A

Source:	Nokia, AT&T

Agenda item:	7.4.2

Document for:	Approval

# 1	Introduction

This is a TP into TR 37.717-21-11 to introduce DC_5A-30A_n2A

*********************** Start of TP ***************************************

## 5.x	DC_5-30_n2

### 5.x.1	Operating bands for DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_5-30_n2 | CA_5-30 | n2 | No |

### 5.x.2	Configurations for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A | CA_5A-30A | n2 |

### 5.x.3	Co-existence studies

Based on co-existence studies of Band 5 + Band n2 captured in 37.716-11-11, no MSD is necessary

Based on co-existence studies of Band 30 + Band n2 captured in 37.716-11-11, MSD shall be considered since

-	4th order IMD generated by dual uplink of the two bands may fall into own Rx of band 5.

### 5.x.4	∆TIB and ∆RIB values

Based on E-UTRA CA_2-5-30 configuration, following relaxations are proposed

Table 5.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30_n2 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30_n2 | 5 | 0 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |

### 5.x.5	REFSENS requirements

As stated in 5.x.3 MSD requirement caused by 4th order IMD is needed and specified below accordingly which is based on similar case of  DC_1A_n8A-n40A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-30A_n2A | 5 | 835 | 5 | 25 | 880 | 8 | IMD4 |
|  | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | N/A |

*********************** End of TP ***************************************