3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2104422

E-meeting, 12th – 20th  April, 2021

Title:	TP to TR 38.717-04-01: CA_n25-n41-n66-n77

Source:	Nokia, T-Mobile USA

Agenda item:	7.10.2

Document for:	Approval

# 1	Introduction

This contribution is a TP into TR 38.717-04-01to introduce CA_n25-n41-n66-n77

******************************* Start of TP *************************************

## 5.x	CA_n25-n41-n66-n77

### 5.x.1	Operating bands for CA

Table 5.x.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n41-n66-n77 | n25 | 1850 MHz | - | 1915 MHz | 1930 MHz | - | 1995 MHz | FDD |
|  | n41 | 2496 MHz | - | 2690 MHz | 2496 MHz | - | 2690 MHz | TDD |
|  | n66 | 1710 MHz | - | 1780 MHz | 2110 MHz | - | 2200 MHz | FDD |
|  | n77 | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | TDD |

### 5.x.2	Channel bandwidths per operating band for CA

Table 5.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41C-n66A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41C bandwidth combination set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n25A-n41(2A)-n66A-n77A | - | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 | See CA_n41(2A) bandwidth combination set 1 in Table 5.5A.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

### 5.x.3	∆TIB,c and ∆RIB,c values

For CA_n25-n41-n66-n77 following TIB,c and RIB,c values specified based on CA_n25-n41-n66

Table 5.x.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n77 | n25 | 0.5 |
|  | n41 | 0.85/1.36 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.x.3-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n41-n66-n77 | n25 | 0.3 |
|  | n41 | 0.55/1.06 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

### 5.x.4	REFSENS requirements

There are no additional MSD requirements for this band combination.

******************************* End of TP *************************************