3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104545

Electronic meeting, 12th – 20th April, 2021

Source:	Ericsson

Title:	Discussion on remain issue of NR-U PDSCH demodulation requirements

Agenda item:	5.1.4.2

Document for:	Discussion

# Introduction

In RAN4#98-e meeting, companies got agreements on test scope and scenarios [1].

Agreements

A1	Use 20 MHz LBT BW (for channel sensing and clear channel assessment)

A2	Do not multiplex SSB and Data

A3	Define tests for TDD 30kHz only.

A4	Define tests with low delay spread and low doppler speed propagation channel.

A5	Q factor for SSB: NSSBQCL  = 8

A6	Use Type A PDSCH Mapping Type

A7	Do not use PDCCH DCI 2-0 in the PDSCH Tests

Remain issues

- Slot format

- TRS allocation

- COT configuration

- LBT failure probability

- Test metric

- Other parameters

In this contribution, open issues will be further discussed.

# Discussion

In general issue, slot format, TRS allocation and LBT probability issues are discussed [2].

- Slot format: Fixed frame period (FFP) and burst transmission model as figures below.

  - DL slot: both {2, 3, 5, 6} and {2, 4, 6, 7} slots in WF can fulfil the expectation.

But it should be noted that the maximum DL slot number could be 8 according to the definition of DL transmission burst and 4ms maximum burst length regulation in Japan. Then another option for DL slot format could be {2, 4, 6, 8} slots

  - UL slot: HARQ-ACK can be transmitted on the first UL slot in every FFP.

Based on DL slot format {2, 3, 5, 6} or {2, 4, 6, 7} slot listed in WF, a fixed start time of UL slot could be 4ms in each 5ms FFP, and the duration is 0.9ms considering Idle time at the end of FFP.

As mentioned above, if use longer DL burst format, such as {2, 4, 6, 8}, it would be better to adjust UL slot start time to 4.5ms to leave enough gap between DL and UL. It will be taken as one burst if the gap is less than 16us. The duration of UL could be 0.4ms.

- TRS allocation:

As we proposed in [2], no TRS configuration can be accepted by NR-U demodulation tests. In our simulation [3], we use Option 2 that no TRS are configured for simplify the simulation adaptation. But we also compared one case performance between with/without TRS, and the result shows almost no difference as expectation.

It can be note that TCI state on PDCCH/PDSCH would only have QCL information for SSB if no TRS is configured.

- DL COT configuration

According to the definition of COT [2], DL and UL will be taken as separate COT if the gap between them larger than 25us. In current assumptions, 1 slots gap at the worst case is agreed. Then maximum DL COT could be equal to 4ms maximum burst regulation.

The idle duration should be larger or equal to 5% of COT. For 4ms DL COT, the idle duration is 0.2ms which is less than 1 slot duration (0.5ms).

- UL COT configuration

If we use {2, 4, 6, 7} slot format, then UL COT could start from 4ms in each FFP and the duration could be 0.9ms. The corresponding idle duration is 0.1ms which is larger than 5% of UL COT.

If we use {2, 4, 6, 8} slot format, then UL COT could start from 4.5ms in each FFP and the duration could be 0.4ms.

- Idle duration after UL COT

Based on the 0.9ms or 0.4ms UL COT duration, the corresponding idle duration is 0.05ms or 0.02ms. But we can use fixed 0.1ms here to include the next DL burst transmission CCA.

- LBT probability

No clear performance difference between 0, 0.25 and 0.5 LBT failure probability based on normalized throughput test metric. Our simulation results also showed this trend. If the time consuming is concerned, then maybe probability as 0.25 could be accepted.

- Test metric

70% normalized throughput is agreed as test metric, but the definition of maximum throughput should be clarified. In LTE eLAA demodulation requirements, the nominal maximum throughput is used and “Fraction of nominal maximum throughput is calculated based on random transmission occasions of PDSCH” [4]. This definition could be reused, then only PDSCH transmissions are calculated when LBT is successful.

- Other parameters

  - K1 for HARQ-ACK: R15 configuration could be reused for fixed frame period structure and the minimum K1 is 2. All HARQ-ACK could be scheduled on the PUCCH in the first UL slot.

  - Bandwidth: It is feasible to define a single set of requirements for both Scenario A and Scenario C as discussed in [2]. Then {20, 40, 60, 80} MHz bandwidth requirements are necessary to cover all CA combination cases. For standalone deployment, only the largest supported bandwidth requirement need to be tested.

Regarding agreed assumptions in WF [1] and analysis above, the proposed simulation assumptions are as below tables.

Table 2-1	PDSCH demodulation parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | According to DL Transmission Model |
| LBT failure probability |  |  | 0.5 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDCCH configuration | TCI state |  | TCI state #0 |
| CSI-RS for tracking |  |  | Not configured |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |

Table 2-2	DL transmission model

| DL Transmission Model | Maximum DL COT Duration | ms | 4 |
| --- | --- | --- | --- |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | 0.5 |
|  | UL COT start time within each FFP | ms | Option 1: 4 for maximum DL burst length <=7 slots Option 2: 4.5 for maximum DL burst length is 8 slots |
|  | UL COT duration | ms | Option 1: 0.9 for maximum DL burst length <=7 slots Option 2: 0.4 for maximum DL burst length is 8 slots |
|  | Idle Time after UL COT | ms | 0.1 |
|  | Number of slots between PDSCH and corresponding HARQ-ACK information |  | 8 if mod(i,10)=0 7 if mod(i,10)=1 6 if mod(i,10)=2 5 if mod(i,10)=3 4 if mod(i,10)=4 3 if mod(i,10)=5 2 if mod(i,10)=6 |
|  | Duration of the Downlink burst | Slots | {2, 4, 6, 7(8)} |
|  | PDSCH Allocation in the last Slot of the Downlink burst | Symbols | 14 for 2 DL slots {6, 9, 12, 14} for more than 2 DL slots |
| Notes: - The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |

Table 2-3	requirement table template

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Slot Pattern | Propagation condition | Correlation matrix and antenna configuration |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | NR-U DL Transmission Model | 20, 40, 60, 80s/30 | 16 QAM, 0.48 | According to NR-U DL Transmission Model | TDLA30-10 | 2x2, ULA Low 2x4 ULA Low | 70 | [TBD] |

# Conclusion

Table 2-1	PDSCH demodulation parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active DL BWP index |  |  | 1 |
| Slot Pattern |  |  | According to DL Transmission Model |
| LBT failure probability |  |  | 0.5 |
| Common serving cell parameters | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | The first SSB |
|  | SSB Q factor |  | 8 |
| PDCCH configuration | TCI state |  | TCI state #0 |
| CSI-RS for tracking |  |  | Not configured |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | According to DL Transmission Model |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| RB Allocation |  |  | Full BW |
| MIMO Layers |  |  | 2 |
| Number of HARQ Processes |  |  | 8 |
| TX EVM |  |  | 6% |

Table 2-2	DL transmission model

| DL Transmission Model | Maximum DL COT Duration | ms | 4 |
| --- | --- | --- | --- |
|  | DL Transmission Model Period/ Fixed Frame Period (Note 1) | ms | 5 |
|  | Probability of LBT Failure pLBT |  | 0.5 |
|  | UL COT start time within each FFP | ms | Option 1: 4 for maximum DL burst length <=7 slots Option 2: 4.5 for maximum DL burst length is 8 slots |
|  | UL COT duration | ms | Option 1: 0.9 for maximum DL burst length <=7 slots Option 2: 0.4 for maximum DL burst length is 8 slots |
|  | Idle Time after UL COT | ms | 0.1 |
|  | Number of slots between PDSCH and corresponding HARQ-ACK information |  | 8 if mod(i,10)=0 7 if mod(i,10)=1 6 if mod(i,10)=2 5 if mod(i,10)=3 4 if mod(i,10)=4 3 if mod(i,10)=5 2 if mod(i,10)=6 |
|  | Duration of the Downlink burst | Slots | Option 1: {2, 4, 6, 7} Option 2: {2, 4 ,6, 8} |
|  | PDSCH Allocation in the last Slot of the Downlink burst | Symbols | 14 for 2 DL slots {6, 9, 12, 14} for more than 2 DL slots |
| Notes: - The Fixed Frame Period denomination applies only for ChannelAccessType-r16 = ‘semistatic’. For ChannelAccessType-r16 = ‘dynamic’ this parameter is identified only as DL Transmission Model Period. |  |  |  |

Table 2-3	requirement table template

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Slot Pattern | Propagation condition | Correlation matrix and antenna configuration |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | NR-U DL Transmission Model | 20, 40, 60, 80s/30 | 16 QAM, 0.48 | According to NR-U DL Transmission Model | TDLA30-10 | 2x2, ULA Low 2x4 ULA Low | 70 | [TBD] |

# References

[1]	R4-2103987, “Way Forward on NR-U UE demodulation requirements”, Qualcomm, RAN4#98-e

[2]	R4-2104544, “Discussion on general issue of NR-U UE and CSI demodulation requirements”, Ericsson, RAN4#98-bis-e

[3]	R4-2104546, “Simulation results for NR-U PDSCH demodulation”, Ericsson, RAN4#98-bis-e

[4]	TS36.104 v16.6.0