3GPP TSG-RAN WG4 Meeting #98-bis-e	                               R4-2104656

Electronic Meeting, 12 Apr – 20 Apr, 2021

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n8, n71, n25 for 35_45MHz channel bandwidth

Agenda Item:	7.25.3 UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

Introduction

In the RAN4#98e meeting, three channel locations (Worst, Middle and Best case) were proposed to consider REFSENS for n8/n71 at CBW 35MHz and n25 at CBW 45MHz [1].

We have provided the REFSENS estimation results for Worst case and Best case in [2], [3]. In this contribution, we provide the Tx noise measurement results and MSD estimation under Middle case.

Discussion

WF Review

In the WF [1], the following conditions were agreed.

◇WF R4-2103187

- For n8, n71, and n25 where UL BW < DL BW. Choose 1 of 3 options for the next meeting while considering test time, complexity, and coverage

  - Option 1: Worst case; UL BW closest to DL

  - Option 2: Middle case; keep existing duplex offset between center of RX and TX channel BWs

  - Option 3: Best case; UL BW furthest from DL

Power Amplifier Calibration and Test Waveforms

PA calibration conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test waveforms:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Measurement Results

The measurement results are shown in Table 2-1, Table 2-2 and Table 2-3.

Table 2-1: n8 RxBN measurement results

|  | UL/DL BW | Tx freq. | Rx freq. | Tx/Rx band gap | Tx RBs Allocation | RxBN |
| --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| Option1 | 20/35 | 905 | 942.5 | 10 | 20@86 | -14.0 |
| Option2 | 20/35 | 897.5 | 942.5 | 17.5 | 20@86 | -30.1 |
| Option3 | 20/35 | 890 | 942.5 | 25 | 20@86 | -37.9 |

Table 2-2: n71 RxBN measurement results

|  | UL/DL BW | Tx freq. | Rx freq. | Tx/Rx band gap | Tx RBs Allocation | RxBN |
| --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| Option1 | 20/35 | 673 | 634.5 | 11 | 20@0 | -14.1 |
| Option2 | 20/35 | 680.5 | 634.5 | 18.5 | 20@0 | -32.5 |
| Option3 | 20/35 | 688 | 634.5 | 26 | 20@0 | -37.8 |

Table 2-3: n25 RxBN measurement results

|  | UL/DL BW | Tx freq. | Rx freq. | Tx/Rx band gap | Tx RBs Allocation | RxBN |
| --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| Option1 | 40/45 | 1895 | 1972.5 | 35 | 40@176 | -27.8 |
| Option2 | 40/45 | 1882.5 | 1962.5 | 37.5 | 40@176 | -32.7 |
| Option3 | 40/45 | 1870 | 1952.5 | 40 | 40@176 | -34.6 |

MSD Estimation

The link-budget calculation assumptions and MSD estimations are shown in Table 3-1, Table 3-2 and Table 3-3.

Table 3-1 MSD Estimation for n8 35MHz

| Parameter | Unit | n8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Option 1 |  | Option 2 |  | Option 3 |  |
| UL/DL BW | MHz | 20/35 |  | 20/35 |  | 20/35 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -14.0 |  | -30.1 |  | -37.9 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  | 50.0 |  |
| DPX ATT RX band | dB | 30.0 |  | 30.0 |  | 30.0 |  |
| Div_filter_Tx rejection | dB | 45.0 |  | 45.0 |  | 45.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -60.0 | -60.0 | -76.1 | -76.1 | -83.9 | -83.9 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -87.0 | -105.0 | -87.0 | -105.0 | -87.0 | -105.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 |
| Total noise level at PRx/DRx ANT Port | dBm | -61.0 | -61.0 | -76.4 | -76.7 | -81.6 | -82.7 |
| Desense | dB | 24.7 |  | 9.2 |  | 3.5 |  |
| REFSENS | dBm | -64.0 |  | -79.5 |  | -85.2 |  |

Table 3-2 MSD Estimation for n71 35MHz

| Parameter | Unit | n71 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Option 1 |  | Option 2 |  | Option 3 |  |
| UL/DL BW | MHz | 20/35 |  | 20/35 |  | 20/35 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -14.1 |  | -32.3 |  | -37.8 |  |
| DPX_ISO_from Tx to Rx | dB | 55.0 |  | 55.0 |  | 55.0 |  |
| DPX ATT RX band | dB | 45.0 |  | 45.0 |  | 45.0 |  |
| Div_filter_Tx rejection | dB | 40.0 |  | 40.0 |  | 40.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -65.1 | -65.1 | -83.3 | -83.3 | -88.8 | -88.8 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -97.0 | -95.0 | -97.0 | -95.0 | -97.0 | -95.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 | -85.7 |
| Total noise level at PRx/DRx ANT Port | dBm | -66.1 | -66.1 | -82.2 | -82.1 | -84.8 | -84.6 |
| Desense | dB | 19.6 |  | 3.6 |  | 1.0 |  |
| REFSENS | dBm | -69.1 |  | -85.2 |  | -87.7 |  |

Table 3-3 MSD Estimation for n25 45MHz

| Parameter | Unit | n25 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Option 1 |  | Option 2 |  | Option 3 |  |
| UL/DL BW | MHz | 40/45 |  | 40/45 |  | 40/45 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -27.8 |  | -32.7 |  | -34.6 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  | 50.0 |  |
| DPX ATT RX band | dB | 40.0 |  | 40.0 |  | 40.0 |  |
| Div_filter_Tx rejection | dB | 40.0 |  | 40.0 |  | 40.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -73.8 | -73.8 | -78.7 | -78.7 | -80.6 | -80.6 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -87.0 | -95.0 | -87.0 | -95.0 | -87.0 | -95.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -84.6 | -84.6 | -84.6 | -84.6 | -84.6 | -84.6 |
| Total noise level at PRx/DRx ANT Port | dBm | -74.3 | -74.4 | -78.2 | -78.6 | -79.5 | -80.0 |
| Desense | dB | 10.2 |  | 6.1 |  | 4.8 |  |
| REFSENS | dBm | -77.4 |  | -81.5 |  | -82.8 |  |

Proposal 1:

Adopt the MSD estimation results shown in the Table 3-1, 3-2, 3-3 when the channel locations are decided.

Conclusion

In this contribution, we make proposal for REFSENS requirements of n8/n71 with 35MHz CBW and n25 with 45MHz CBW.

Proposal 1:

Adopt the REFSENS estimation results shown in the Table 3-1, 3-2, 3-3 when the channel locations are decided.

References

[1]		R4-2103187, WF on REFSENS for 35MHz and 45MHz, Qualcomm, Skyworks, […]

[2]		R4-2100703, REFSENS of n8 and n71 for 35MHz channel bandwidth, Murata Manufacturing Co Ltd. [3] 	R4-2100705, REFSENS of n25 for 45MHz channel bandwidth, Murata Manufacturing Co Ltd.