3GPP TSG-RAN WG4 Meeting #98-bis-e	                               R4-2104657

Electronic Meeting, 12 Apr – 20 Apr, 2021

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS of n5 for 25MHz channel bandwidth

Agenda Item:	7.24.2.1 Reference sensitivity [NR_bands_R17_BWs -Core]

Document for:	Approval

Introduction

The agreement could not be reached on REFSENS of n5 for 25MHz CBW in RAN4#98e meeting. It is encouraged that companies bring analysis and measurements results for evaluating REFSENS requirement [1].

In this contribution, we provide our analysis of Tx noise, measurement results and MSD estimation.

Discussion

WF Review

- WF [R4-2103185]

  - Companies encouraged to bring UL config proposals for REFSENS at next meeting

  - Companies encouraged to bring analysis/measurements for REFSENS at next meeting for 25MHz

  - Companies encouraged to evaluate TX/RX 20MHz/25MHz and 25MHz/25MHz options at next meeting

Analysis of Tx noise

When UL/DL = 25 / 25MHz, the IM3 of Tx signal and Tx image falls into the Rx band (Figure 1). As a result, Tx noise in the Rx band increases, eventually causing a large Desense. The legacy channel scaling method is not suitable for specifying the REFSENS of n5 with 25MHz CBW.

On the other hand, when UL/DL = 20 / 25MHz (Figure 2), the IM3 of Tx signal and Tx image shifts to a low frequency and come out from the Rx band. Desense will be improved.

Figure 1: PA output spectrum for UL/DL = 25/25MHz

Figure 2: PA output spectrum for UL/DL = 20/25MHz

Tx noise measurement and MSD Estimation

Power Amplifier Calibration and Test Waveforms

PA calibration conditions:

The power amplifier (PA) is calibrated to meet 1 dB MPR for Power Class 3 using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

Test waveforms:

- QPSK DFT-s-OFDM SCS 15kHz with the UL RBs be located as close as possible to the DL operating band but confined within the transmission bandwidth configuration for the CBW;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc;

Measurement Results

The Rx band Noise (RxBN) of PA was measured by UL/DL CBW =25/25MHz and UL/DL CBW =20/25MHz. The results are shown in Table 1.

Table 1: PA RxBN measurement results

|  | UL/DL BW | Tx freq. | Rx freq. | Tx/Rx band gap | Tx RBs Allocation | RxBN |
| --- | --- | --- | --- | --- | --- | --- |
|  | (MHz) | (MHz) | (MHz) | (MHz) | (-) | (dBm/BW) |
| Option1 | 25/25 | 836.5 | 881.5 | 20 | 20@113 | -19.3 |
| Option2a (worst case) | 20/25 | 839 | 881.5 | 20 | 20@86 | -34.1 |
| Option2b (middle case) | ↓ | 836.5 | ↓ | 22.5 | ↓ | -36.5 |
| Option2c (best case) | ↓ | 834 | ↓ | 25 | ↓ | -38.0 |

Observation 1:  With limiting the UL BW to 20MHz, RxBN will be greatly improved.

MSD Estimation

The link-budget calculation assumptions and MSD estimation are shown in Table 2.

Table 2: MSD Estimation for n5

| Parameter | Unit | n5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Option 1 |  | Option2a |  | Option 2b |  | Option 2c |  |
| UL/DL BW | MHz | 25/25 |  | 20/25 |  | 20/25 |  | 20/25 |  |
| Antenna isolation | dB | 10.0 |  | 10.0 |  | 10.0 |  | 10.0 |  |
| Front-end loss | dB | 4.0 |  | 4.0 |  | 4.0 |  | 4.0 |  |
| PA output power | dBm | 27.0 |  | 27.0 |  | 27.0 |  | 27.0 |  |
| PA RxBN | dBm | -19.3 |  | -34.1 |  | -36.5 |  | -38.0 |  |
| DPX_ISO_from Tx to Rx | dB | 50.0 |  | 50.0 |  | 50.0 |  | 50.0 |  |
| DPX ATT RX band | dB | 45.0 |  | 45.0 |  | 45.0 |  | 45.0 |  |
| Div_filter_Tx rejection | dB | 45.0 |  | 45.0 |  | 45.0 |  | 45.0 |  |
| Transceiver effective phase noise | dBc/Hz | -144.0 |  | -144.0 |  | -144.0 |  | -144.0 |  |
| Thermal noise at PRx/DRX ANT port | dBm/Hz | -164.0 |  | -164.0 |  | -164.0 |  | -164.0 |  |
| SNR requirement for QPSK | dB | -1.0 |  | -1.0 |  | -1.0 |  | -1.0 |  |
| TX noise power at PRX/DRx ANT port | dBm | -65.3 | -65.3 | -80.1 | -80.1 | -82.5 | -82.5 | -84.0 | -84.0 |
| TX IM2 due to wider CBW at PRx/DRx ANT port | dBm | -87.0 | -105.0 | -87.0 | -105.0 | -87.0 | -105.0 | -87.0 | -105.0 |
| Thermal noise at PRx/DRx ANT Port | dBm | -87.2 | -87.2 | -87.2 | -87.2 | -87.2 | -87.2 | -87.2 | -87.2 |
| Total noise level at PRx/DRx ANT Port | dBm | -66.2 | -66.3 | -79.7 | -80.3 | -81.2 | -82.2 | -82.0 | -83.3 |
| Desense | dB | 21.0 |  | 7.2 |  | 5.5 |  | 4.5 |  |
| REFSENS | dBm | -69.3 |  | -83.0 |  | -84.7 |  | -85.7 |  |

Observation 2:

There is a large Desense when UL/DL BW = 25/25MHz. For improving Desense, UL/DL BW = 20/25MHz operation should be applied like n8 and n71 with 35MHz CBW [2].

Proposal 1:

Adopt the MSD estimation results shown in Table 2 when UL/DL BW and channel allocation are decided.

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n5 | 15 | 25 | 25 | 20 | 20 | 20 |  |  |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 | 10 |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2:

Use the UL configuration as highlighted in Table 7.3.2-3 for n5 REFSENS requirement with 25MHz CBW.

Conclusion

In this contribution, we present PA Rx band noise of n25 with 25MHz CBW and propose the following reference sensitivity levels and UL configurations.

Observation 1:

With limiting the UL BW to 20MHz, RxBN will be greatly improved.

Observation 2:

There is a large Desense when UL/DL = 25/25MHz. For improving Desense, UL/DL = 20/25MHz operation should be applied like n8 and n71 with 35MHz CBW [2].

Proposal 1:

Adopt the MSD estimation results shown in the Table 2 when UL/DL BW and channel allocation are decided.

Proposal 2:

Use the UL configuration as highlighted in Table 7.3.2-3 for n5 REFSENS requirement with 25MHz CBW.

References

[1]		R4-2103185, Way forward on the introduction of 25 MHz in band n5, AT&T

[2]		R4-2103187, WF on REFSENS for 35MHz and 45MHz, Qualcomm, Skyworks, […]