3GPP TSG-RAN4 Meeting #98bis-e	R4-210662

# Electronic meeting, 12th – 20th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to 38.176-2: Introduction of CSI-RS performance tests and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Performance requirements and tests for CSI-RS shall be introduced for IAB-MT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performance requirements are intorduced. The test method aligns to the agreements on IAB-MT testing and the performance test requirements are simplified according to prior agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete IAB specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.x	IAB-MT Performance requirements

### 8.x.3	Performance requirements for CSI reporting

##### 8.x.3.1     Definition and applicability

The performance requirement of CSI reporting is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [11]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

Which specific test(s) are applicable to IAB-MT is based on the test applicability rules defined in clause 8.1.2.1.

##### 8.x.3.2     Minimum requirement

The minimum requirement for IAB-MT type 1-O is in TS 38.174 [x] clause xx.

The minimum requirement for IAB-MT type 2-O is in TS 38.174 [x] clause xx.

##### 8.x.3.3     Test purpose

The test shall verify the receiver's ability to report correct median CQI and expected BLER performance under AWGN conditions.

##### 8.x.3.4     Method of test

###### 8.x.3.4.1    Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

###### 8.x.3.4.2    Procedure

1)	Place the IAB-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the IAB-MT with the test system.

3)	Set the IAB-MT in the declared direction to be tested.

4)	Connect the IAB-MT tester generating the wanted signal and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.x.3.4.2-1.

Table 8.x.3.4.2-1: Test parameters for testing CQI reporting requirements

| Parameter |  |  |  |  | Unit |  | Value FR1 |  | Value FR2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  |  |  | Transmission scheme 1 |  | Transmission scheme 1 |  |  |
| Duplex mode |  |  |  |  |  |  | TDD |  | TDD |  |  |
| PTRS epre-Ratio |  |  |  |  |  |  | N/A |  | 0 |  |  |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  |  |  | RBs |  | 0 |  | 0 |  |  |
|  | Subcarrier spacing |  |  |  | kHz |  | 30 |  | 120 |  |  |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  |  | Normal |  | Normal |  |  |
|  | RB offset |  |  |  | RBs |  | 0 |  | 0 |  |  |
|  | Number of contiguous PRB |  |  |  | PRBs |  | 106 |  | 66 |  |  |
| Active DL BWP index |  |  |  |  |  |  | 1 |  | 1 |  |  |
| PDSCH configuration |  | Mapping type |  |  |  |  | Type A |  | Type A |  |  |
|  |  | k0 |  |  |  |  | 0 |  | 0 |  |  |
|  |  | Starting symbol (S) |  |  |  |  | 2 |  | 2 |  |  |
|  |  | Length (L) |  |  |  |  | 12 |  | 12 |  |  |
|  |  | PDSCH aggregation factor |  |  |  |  | 1 |  | 1 |  |  |
|  |  | PRB bundling type |  |  |  |  | Static |  | Static |  |  |
|  |  | PRB bundling size |  |  |  |  | 2 |  | 2 |  |  |
|  |  | Resource allocation type |  |  |  |  | Type 0 |  | type 0 |  |  |
|  |  | RBG size |  |  |  |  |  |  | Config 2 |  |  |
|  |  | VRB-to-PRB mapping type |  | Non-interleaved | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  | N/A |  |  |
|  |  | PDCCH & PDCCH DMRS Precoding configuration |  |  |  |  | Single Panel Type I, Random precoder chosen from precoder index 0 an 2, selection updated per slot |  |  |  |  |
| PDSCH DMRS configuration |  | DMRS Type |  |  |  |  | Type 1 |  | Type 1 |  |  |
|  |  | Number of additional DMRS |  |  |  |  | 1 |  | 1 |  |  |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  | 1 |  |  |
|  |  | DMRS ports indexes |  |  |  |  | {1000} |  | {1000} |  |  |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  |  |  |  | 2 |  | 2 |  |  |
| PTRS configuration |  | Frequency density (KPT-RS) |  |  |  |  | N/A |  | 2 |  |  |
|  |  | Time density (LPT-RS) |  |  |  |  | N/A |  | 1 |  |  |
|  |  | Resource Element Offset |  |  |  |  | N/A |  | 2 |  |  |
| NZP CSI-RS for CSI acquisition |  |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  | Start PRB 0 Number of PRB = BWP size |  |  |
| ZP CSI-RS for CSI acquisition |  |  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  | Start PRB 0 Number of PRB = BWP size |  |  |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 3: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [3] or 38.101-2 [4] for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the IAB-MT receiver is as specified in clause 8.x.3.1.5.1 and 8.x.3.1.5.2 for IAB type 1-O and IAB type 2-O respectively, and that the SNR at the IAB-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.x.3.4.2-2.

Table 8.x.3.4.2-2: AWGN power level at the IAB-MT input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| IAB-MT type 1-O | 30 | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
| IAB-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.53 in table 4.6-1 and clause 7.1. NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the IAB-MT, measure the median CQI and the BLER at (median CQI +1) and (median CQI -1).

###### 8.x.3.4.3    Test requirement

###### 8.x.3.4.3.1   Test requirement for IAB type 1-O

For the parameters specified in Table 8.x.3.4.3.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 8.x.3.4.3.1-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-4 |  |  |  |

###### 8.x.3.4.3.2   Test requirement for IAB type 2-O

For the parameters specified in Table 8.x.3.4.3.2-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified by the following:

a)	the reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time;

b)	if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 8.x.3.4.4.2-1 Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-1, TBS.1-2 |  |  |  |