3GPP TSG-RAN4 Meeting #98bis-e	R4-2104663

# Electronic meeting, 12th – 20th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to 38.176-1: IAB-MT performance tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2011-04-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Performance tests for the IAB-MT are needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Performance tests and performance requirements are introduced. The performance requirements are simplified according to the selected requirements and simplifications of previous agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete IAB-MT specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.x	IAB-MT Performance requirements

### 8.x.1	General

Unless otherwise stated, for a BS supporting more than 8 antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the performance requirement tests for 8 RX antennas shall apply, and the specific connectors used for testing are based on manufacturer declaration.

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

### 8.x.2	Performance requirements for IAB-MT demodulation

#### 8.x.2.1      Performance requirements for PDSCH

##### 8.x.2.1.1     Definition and applicability

The performance requirement of PDSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to IAB-MT is based on the test applicability rules defined in clause 8.1.2.1.

##### 8.x.2.1.2     Minimum requirement

The minimum requirement is in TS 38.174 [x] clause xx.

##### 8.x.2.1.3     Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

##### 8.x.2.1.4     Method of test

###### 8.x.2.1.4.1    Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

###### 8.x.2.1.4.2    Procedure

1)	Connect the IAB tester generating the wanted signal, multipath fading simulators and AWGN generators to all IAB-MT antenna connectors for diversity reception via a combining network as shown in annex D.x.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 8.x.2.1.4.2-2.

Table 8.x.2.1.4.2-2: Test parameters for testing PDSCH

| Parameter |  |  |  | Unit |  |  | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Duplex mode |  |  |  |  |  |  | TDD |  |
| Active BWP index |  |  |  |  |  |  | 1 |  |
| PDSCH transmission scheme |  |  |  |  |  |  | Transmission scheme 1 |  |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | RBs |  |  | 0 |  |
|  | Subcarrier spacing |  |  | kHz |  |  | 30 |  |
| DL BWP configuration #1 | Cyclic prefix |  |  |  |  |  | Normal |  |
|  | RB offset |  |  | RBs |  |  | 0 |  |
|  | Number of contiguous PRB |  |  | PRBs |  |  | 106 |  |
| PDSCH DMRS configuration | Antenna ports indexes |  |  |  |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |  |
|  | Position of the first DMRS for PDSCH mapping type A |  |  |  |  |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  |  |  |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DMRS Type |  |  |  |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  |  |  |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  |  |  | 1 |
| PDSCH configuration |  | Mapping type |  |  |  | Type A |  |  |
|  |  | k0 |  |  |  | 0 |  |  |
|  |  | Starting symbol (S) |  |  |  | 2 |  |  |
|  |  | Length (L) |  |  |  | Specific to each Reference channel |  |  |
|  |  | PDSCH aggregation factor |  |  |  | 1 |  |  |
|  |  | PRB bundling type |  |  |  | Static |  |  |
|  |  | PRB bundling size |  |  |  | 2 |  |  |
|  |  | Resource allocation type |  |  |  | Type 0 |  |  |
|  |  | RBG size |  |  |  | Config2 |  |  |
|  |  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |  |
|  |  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |  |
| PT-RS configuration |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Physical signals, channels mapping and precoding |  |  | As specified in Annex xx |
| Note 1: Point A coincides with minimum guard band as specified in Table xx from TS 38.174 [xx] for tested channel bandwidth and subcarrier spacing. |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in tables xx (as applicable) is achieved at the IAB-MT input.

6)	For each of the reference channels in table xx applicable for the IAB-MT, measure the throughput.

###### 8.x.2.1.4.3    2RX Test requirement

The throughput measured according to clause 8.x.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.x.2.4.3-1 and 8.x.2.1.4.3-2.

Table 8.x.2.4.3-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | TBC | 40 / 30 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | TBC |
| 2 | TBC | 40 / 30 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | TBC |

Table 5.2.2.2.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | TBC | 40 / 30 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | TBC |

###### 8.x.2.1.4.4    4RX Test requirement

The throughput measured according to clause 8.x.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.x.2.4.4-1, 8.x.2.1.4.4-2, 8.x.2.1.4.4-3 and 8.x.2.1.4.4-4.

Table 8.x.2.4.4-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | TBC | 40 / 30 | 256QAM, 0.82 | TDLA30-10 | 2x4, ULA Low | 70 | TBC |
| 2 | TBC | 40 / 30 | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 30 | TBC |

Table 8.x.2.4.4-2: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3 | TBC | 40 / 30 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | TBC |

Table 5.2.3.2.1-5: Minimum performance for Rank 3

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4 | TBC | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | TBC |

Table 5.2.3.2.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBC | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | TBC |

#### 8.x.2.2      Performance requirements for PDCCH

##### 8.x.2.2.1     Definition and applicability

The performance requirement of PDSCH is determined by a maximum allowed missed detection rate for a given SNR. The required missed detection rate is expressed for the FRCs listed in annex A.

Which specific test(s) are applicable to IAB-MT is based on the test applicability rules defined in clause 8.1.2.1.

##### 8.x.2.2.2     Minimum requirement

The minimum requirement is in TS 38.174 [x] clause xx.

##### 8.x.2.2.3     Test purpose

The test shall verify the receiver's ability to achieve missed detection rate under multipath fading propagation conditions for a given SNR.

##### 8.x.2.2.4     Method of test

###### 8.x.2.2.4.1    Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

###### 8.x.2.2.4.2    Procedure

1)	Connect the IAB tester generating the wanted signal, multipath fading simulators and AWGN generators to all IAB-MT antenna connectors for diversity reception via a combining network as shown in annex D.x.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 8.x.2.2.4.2-2.

Table 8.x.2.2.4.2-2: Test parameters for testing PDCCH

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 6 |  |  |
| Shift Index |  | 0 |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in tables xx (as applicable) is achieved at the IAB-MT input.

6)	For each of the reference channels in table xx applicable for the IAB-MT, measure the missed detection.

###### 8.x.2.2.4.3    2RX Test requirement

###### 8.x.2.2.4.3.1    1 TX antenna performance

For the parameters specified in Table 8.x.2.2.4.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.x.2.2.4.3.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 8.x.2.2.4.3.1-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | TBC | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 2 | 40 | 102 | 1 | 4 | TBC | TDLC300- 100 | 1x2 Low | 1 | 3.0 |

8.x.2.2.4.3.2    2 TX antenna performance

For the parameters specified in Table 8.x.2.2.4.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table .x.2.2.4.3.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | TBC | TDLC300-100 | 2x2 Low | 1 | -1.2 |

###### 8.x.2.2.4.4    4RX Test requirement

###### 8.x.2.2.4.4.1    1 TX antenna performance

For the parameters specified in Table 8.x.2.2.4.4.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.x.2.2.4.4.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 8.x.2.2.4.4.1-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 2 | TBC | TDLA30-10 | 1x4 Low | 1 | 2.1 |
| 2 | 40 | 102 | 1 | 4 | TBC | TDLC300-100 | 1x4 Low | 1 | -0.9 |

###### 8.x.2.2.4.4.2    2 TX antenna performance

For the parameters specified in Table 8.x.2.2.4.4.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.x.2.2.4.4.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 8.x.2.2.4.4.2-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 90 | 1 | 8 | TBC | TDLC300-100 | 2x4 Low | 1 | -4.3 |