3GPP TSG-WG RAN4 Meeting #98bis-e	R4-2104723

# Online, 12th – 20th April, 2021

Source: 				Nokia, Nokia Shanghai Bell, [Verizon]

Title: 				TP to TR 37.717-21-11 DC_13-46_n77

Agenda Item: 	7.4.2 [DC_R17_2BLTE_1BNR_3DL2UL]

Document for:	Approval

# Introduction

In this contribution, a text proposal to complete 3DL/2UL EN-DC configurations DC_13A-46A_n77A is provided.

# TP to TR 37.717-21-11

## 5.X	DC_13-46_n77

### 5.X.1	Operating bands for DC

Table 5.X.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_13A-46A_n77A | DC_13A_n77A |

### 5.X.2	Co-existence studies

For UE coexistence study of Band 13 + Band n77, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products are presented in Table 5.X.2-1.

Table 5.X.2-1: Harmonic and IMD analysis for DC_13_n77

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 777 | 787 | 3300 | 4200 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1554 | 1574 | 6600 | 8400 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2331 | 2361 | 9900 | 12600 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 2513 | 3423 | 4077 | 4987 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 2646 | 1726 | 5813 | 7623 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 4854 | 5774 | 7377 | 9187 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1869 | 939 | 9113 | 11823 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 5631 | 6561 | 10677 | 13387 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 6846 | 5026 | 8154 | 9974 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 16023 | 12413 | 152 | 1092 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 13977 | 17587 | 6408 | 7348 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 11046 | 8326 | 4239 | 6069 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 11454 | 14174 | 8931 | 10761 |

Based on co-existence study as presented in the table 5.X.2-1, own Rx impact is shown in the following.

- The 2nd order IMD generated by dual uplink of 13+n77 may fall into own Rx of band n77

- The 3rd order IMD generated by dual uplink of 13+n77 may fall into own Rx of band 46

- The 3rd and 4th order IMD generated by dual uplink of 13+n77 may fall into own Rx of band 46

- The 5th order IMD generated by dual uplink of 13+n77 may fall into own Rx of band 13, 46 and n77

### 5.X.3	∆TIB and ∆RIB values

The same relaxation values as DC_13_n77 is assumed for DC_13-46_n77.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-46_n77 | 13 | 0.5 |
|  | n77 | 0.8 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-46_n77 | 13 | 0 |
|  | 46 | 0 |
|  | n77 | 0 |

### 5.X.4	Reference sensitivity exceptions

The IMD issues specifc to 3DL/2UL is the IMD3/IMD4/IMD5 of band 13+n77 falling into band 46. The exclusion zone for IMD to band 46 is applied.