3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104977

Electronic Meeting, Apr. 12-20, 2021

Agenda item:	8.14.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Views on MMSE-IRC receiver for inter-cell interference test

Document for:	Discussion

1. Introduction

Rel.17 WI on NR demodulation performance was approved [1]. This contribution presents our views on MMSE-IRC receiver for inter-cell interference test for Rel.17 NR.

2. Discussion

In Rel.16 NR, RAN4 introduced the following UE demodulation requirements. In Rel.17 NR, RAN4 discusses to introduce the MMSE-IRC receiver for handling inter-cell and inter-user interference.

- CA normal demodulation - CA CQI reporting - CA/EN-DC power imbalance demodulation - PMI reporting for Tx ports larger than 8 and up to 32 - TDD LTE-NR co-existence demodulation requirements

2.1. MMSE-IRC receiver for inter-cell interference test

This WI has been under discussion since Rel.15 NR discussion and is discussed for extension of test coverage in Rel.17 NR. Therefore, it is straightforward to reuse the test parameters of Rel.15/16 PDSCH normal demodulation requirement as much as possible.

Proposal 1: For MMSE-IRC receiver for inter-cell interference test, the following test parameters are applied;

| Parameters | Value |
| --- | --- |
| Duplex mode/SCS | FDD 15kHz, TDD 30kHz |
| Channel Bandwidth | FDD 10MHz, TDD 40MHz |
| TDD pattern | 7D1S2U (S= 6D:4G:4U) |
| Antenna configuration | 2x2, 2x4 |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| PDCCH allocation | Symbol 0,1 |
| Test metric | SNR @70% of maximum throughput |

3. Conclusion

In this contribution, we present our views on MMSE-IRC receiver for inter-cell interference test. Our proposal is summarized below.

Proposal 1: For MMSE-IRC receiver for inter-cell interference test, the following test parameters are applied;

| Parameters | Value |
| --- | --- |
| Duplex mode/SCS | FDD 15kHz, TDD 30kHz |
| Channel Bandwidth | FDD 10MHz, TDD 40MHz |
| TDD pattern | 7D1S2U (S= 6D:4G:4U) |
| Antenna configuration | 2x2, 2x4 |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| PDCCH allocation | Symbol 0,1 |
| Test metric | SNR @70% of maximum throughput |

References

- RP‑210920, Further enhancement on NR demodulation performance, China Telecom