3GPP TSG-RAN WG4 Meeting #98-bis-e              	R4-2104982

E-Meeting, 12 April 2021 – 20 April 2021

Title:	Discussion on concurrent and independent MG

Source:	NEC

Agenda item:	8.5.2.2

Document for:	Approval

Introduction

In last meeting companies had different understanding on the definition of independent gaps and concurrent gaps.

In this contribution, we discuss the principle for defining requirements for multiple concurrent and independent MG patterns.

Discussion

In Rel-15/16 of NR, only one measurement gap can be configured within one measurement period per-UE or per-FR based on the per-UE and per-FR measurement gap capability. However the SMTC can be configured in a much flexible manner, e.g. with different periodicity and offset. Due to the restriction imposed by the single measurement gap per measurement period, flexibility of SMTC configuration cannot be fully utilised, there by resulting in measurement performance degradation in some scenarios. To solve this aspect, in Rel-17, measurement gap enhancements WI is introduced and one of the objective of the WI is described below.

Multiple concurrent and independent MG patterns [RAN4, RAN2]

  - RRM requirements for concurrent and independent MG patterns [RAN4]

    - Define requirements for UE maximum number of concurrent and independent MG patterns active at any time

    - Specification of requirements for multiple concurrent and independent MG patterns (MGL, MGRP)

    - Specification of requirements and UE behavior for proximity of MG instances in time, priority, and partial or full overlap of MG instances

    - Define the corresponding measurement requirements

  - Specification of applicability of multiple concurrent and independent gap patterns [RAN4]

  - Procedures and signaling for simultaneous RRC (re-)configuration of one or more gap patterns [RAN2]

    - Specification of protocol impacts for multiple concurrent and independent MG patterns based on RAN4 input

According to the objective of the WI, it was agreed to introduce multiple independent and concurrent measurement gap patterns per measurement period.

However, during last meeting companies have different understanding on the definition of the concurrent and independent MG.

Independent measurement gaps

In NR Rel-15 measurements are mainly mobility oriented.  With the addition of new features such as positioning services and NTN in Rel-16 and Rel-17, new types of measurements with different characteristics and requirements compared to mobility measurements are introduced. As we know, in Rel-15 only one measurement gap is allowed to be configured per measurement period. In Rel-16 during positioning measurements discussion, we discussed that configuring positioning measurements and RRM measurements in a single MG may require frequent MG reconfiguration through RRC, which may not be efficient. In Rel-17 preliminary discussions, performing NTN measurements and RRM measurements of TN simultaneously using a same measurement gap may be challenging for UE.

In last meeting discussion, Independent measurement gaps definition is understood differently by different companies. Following are the options discussed in last meeting [2].

- FFS definition of independent MG

  - Option 1: (configuration perspective) gaps are considered as independent gaps if at least one of the configurations in MGL, MGRP, time offset is different.

  - Option 2: (UE behavior perspective) gaps are considered as independent gaps if they can operate simultaneously without impacting the measurement performance requirements.

  - Other option is not precluded

From the discussion above we prefer 2nd method based definition. That means for example if a UE is configured with positioning measurements and RRM measurements, if UE can measure them simultaneously without having impact on the other measurement performance requirements, then these measurement gaps are called Independent measurement gaps.

Proposal 1: RAN4 to agree that measurement gaps are considered as independent if UE can measure on these gaps simultaneously without impacting the measurement performance requirements of each MG.

Concurrent measurement gaps

SMTC can be configured with multiple (two) SMTC periodicity and different offsets per periodicity. For a given SMTC periodicity, SMTC offset can be upto SMTC periodicity-1. For example for a SMTC periodicity of 40ms, offset of SMTC configuration can be 0 to 39. Theoretically speaking for a given SMTC period, based on {SMTC window, SMTC offset} multiple measurement gap pattern can be configured for UE. However in Rel-15 and 16 only one measurement gap is allowed at a time. This may be restrictive for some scenarios for configuring measurements of different periodicity requirements.

In this scenario to improve the MG configuration and thereby overall measurement performance and flexibility of the system, it is beneficial to allow more than one MG to be configured simultaneously within a measurement period. In our view these are called concurrent measurement gaps.

Following definition is discussed in last meeting [2].

- Concurrent MG definition

  - Concurrent MGs are multiple MGs that are configured for measurements during a common period of time

    - Exact definition of common period of time is FFS

    - UE behaviour for non-overlapping, partially or fully overlapped cases is irrelevant to the definition and will be discussed separately.

    - Note 1: current definition does not address pre-configured MG patterns and NCSG. FFS how to address pre-configured MG patterns and NCSG.

From the above discussion and the last meeting discussed definition we can derive concurrent MG definition as following.

  - Concurrent MGs are multiple MGs that are configured for measurements during a common period of time

    - Where common period of time is measurement period of UE

    - Any two measurement gaps configured as concurrent MG can have different MGL, MGRP, offset different values.

    - Note 1: UE behaviour for non-overlapping, partially or fully overlapped cases is irrelevant to the definition and will be discussed separately.

    - Note 2: current definition does not address pre-configured MG patterns and NCSG. FFS how to address pre-configured MG patterns and NCSG.

Proposal 2: RAN4 to agree that concurrent MGs are multiple MGs that are configured for measurements during a common period of time

  - Where common period of time is measurement period of UE

  - Any two measurement gaps configured as concurrent MG can have different MGL, MGRP, offset different values.

  - Note 1: UE behaviour for non-overlapping, partially or fully overlapped cases is irrelevant to the definition and will be discussed separately.

  - Note 2: current definition does not address pre-configured MG patterns and NCSG. FFS how to address pre-configured MG patterns and NCSG

Relation between Independent MG and concurrent MG

In our view there can be following relation between any two MG configured.

- Concurrent but not independent

- Concurrent and independent

- Independent but not concurrent

The UE behaviour may depends on the UE capability of what configuration UE can support. For example if UE indicate a capability saying it can support concurrent and independent measurement gaps, NW can configure accordingly the MG. To better utilize the functionality of these newly introduced MG, RAN4 to request other WG to define the UE capability related to what type of new MG configurations it can support.

Proposal 3: RAN4 to further study and request other WG to define new capability to indicate which of the following capability is supported by UE.

- Concurrent but not independent

- Concurrent and independent

- Independent but not concurrent

MG configuration overhead

Though it was agreed to introduce multiple measurement gap patterns in a measurement period, RAN4 should be careful in deciding the number of concurrent or independent measurement gap patterns that are allowed per measurement period. If too many measurement gap patterns are allowed, it will increase the measurement gap density in a measurement period and thereby resulting in higher data interruptions.

Another aspect RAN4 should consider while allowing the multiple MG patterns per measurement period is the cumulative MG length across the MG patterns per measurement period shall not exceed the current maximum MGL allowed for a MG pattern. That means currently, maximum MGL for a MG pattern is 20ms and the cumulative MG length across MG patterns in a measurement period shall not exceed 20ms. To understand this further, let use following example. In the example, if we assume 4 parallel independent MG patterns are allowed, the 4 MG patterns chosen should satisfy the criteria that total cumulative MG length across them cannot be more than 20ms. That means MG 0 (MGL=6ms), MG 1 (MGL=6ms), MG 4 (MGL=6ms) and MG 5 (MGL=6ms) should not be allowed to be configured as the parallel multiple MG patterns, as their resultant MGL is 24ms.

In our view, the above mentioned restriction is further justifiable due to the fact that when one MGL is configured, total interruption length due to MG is 20ms, where as in the given example, interruption due to multiple MG gaps is increased to 24ms, which is not desirable as this is not the intended goal for the introduction of multiple MG patterns.

Another aspect RAN4 should consider in designing multiple MG patterns per measurement period is the interruption periodicity due to introduction of multiple MG patterns. We can take the current interruption periodicity as the reference for designing this parameter. In current NR system, the minimum MGRP is 20ms, that means minimum interruption periodicity due to MG is 20ms. In our understanding we can consider this as the minimum interruption periodicity.  That means RAN4 should allow only one MG for each 20ms.

Based on the above analysis we make the following proposal.

Proposal 4:  When designing multiple MG patterns in a measurement period, RAN4 to agree that maximum of only one MG is allowed for every 20ms.

Proposal 5: RAN4 to agree the principle for deciding the number of MG patterns per measurement period is “total cumulative MGL across MG patterns in a measurement period shall be less than current maximum MGL of 20ms and there cannot be more than one MG for each 20ms period”.

- Conclusion

In this contribution, we discussed measurement gap enhancements related to concurrent and independent MG and made the following proposals:

Proposal 1: RAN4 to agree that measurement gaps are considered as independent if UE can measure on these gaps simultaneously without impacting the measurement performance requirements of each MG.

Proposal 2: RAN4 to agree that concurrent MGs are multiple MGs that are configured for measurements during a common period of time

  - Where common period of time is measurement period of UE

  - Any two measurement gaps configured as concurrent MG can have different MGL, MGRP, offset different values.

  - Note 1: UE behaviour for non-overlapping, partially or fully overlapped cases is irrelevant to the definition and will be discussed separately.

  - Note 2: current definition does not address pre-configured MG patterns and NCSG. FFS how to address pre-configured MG patterns and NCSG

Proposal 3: RAN4 to further study and request other WG to define new capability to indicate which of the following capability is supported by UE.

- Concurrent but not independent

- Concurrent and independent

- Independent but not concurrent

Proposal 4:  When designing multiple MG patterns in a measurement period, RAN4 to agree that maximum of only one MG is allowed for every 20ms.

Proposal 5: RAN4 to agree the principle for deciding the number of MG patterns per measurement period is “total cumulative MGL across MG patterns in a measurement period shall be less than current maximum MGL of 20ms and there cannot be more than one MG for each 20ms period”.

References

- RP-202119

- R4-2103676