3GPP TSG-RAN WG4 Meeting # 98-bis-e	R4-2105002

# Electronic Meeting, 12 - 20 April, 2021

Source:	LG Electronics

Title:	Discussion on contents and table format for V2X demodulation specification

Agenda item:	5.2.1.1

Document for:	Discussion

Introduction

In last meeting, there were much time to discuss detail contents and table format for test case, RMC, and resource pool configuration, and draft Big CR was endorsed during post-meeting email approval. In the draft Big CR, some tables format for test parameters and minimum performance was not aligned. In this contribution, we provide our views on these to align contents and table format for NR V2X test cases.

Discussion

Reference measurement channels

In [1], reference measurement channels have been defined for each test case. Based on companies’ views from e-mail discussion, we propose

- Proposal 1: Reference measurement channels are defined for different physical channels like LTE V2X

- Proposal 2: Add 2nd stage SCI configuration in PSSCH reference measurement channel table as Table 1

Table 1 Proposed RMC table

| Parameter | Unit |
| --- | --- |
| Reference channel |  |
| Channel bandwidth | MHz |
| Subcarrier spacing | kHz |
| Allocated resource blocks | RB |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |
| CP-OFDM symbols for slot without PSFCH |  |
| DMRS symbols for slot with PSFCH |  |
| DMRS symbols for slot without PSFCH |  |
| Modulation order |  |
| MCS index |  |
| Number of MIMO layers |  |
| Number of DMRS REs |  |
| 2nd stage SCI format 2-A configuration | Payloads |
|  | α |
|  | βoffset |
| Transport Block Size for slot with PSFCH | Bits |
| Transport Block Size for slot without PSFCH | Bits |
| Transport block CRC | Bits |
| Maximum number of HARQ transmissions |  |
| Binary Channel Bits for slots with PSFCH |  |
| Binary Channel Bits for slots without PSFCH | Bits |

Test parameters and minimum performance for test cases

To align table format for test parameter and minimum performance of NR demodulation specification, the table format for those in [1] needs to be revised. The information of bandwidth/SCS and propagation condition should be included in the table for minimum performance as following table. Depending on test cases, modulation format and code rate should also are included.

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSxxx BLER (%) | SNR (dB) |
| 1 | R.PSxxx.y-z.z | 20 / 30 | TDLA30-xxx | 1 | TBA |

- Proposal 3: Add the information for bandwidth/SCS and propagation condition in the table of minimum performance

Based on Proposal 2 and Proposal 3, the information for 2nd stage SCI configuration, bandwidth/SCS, and propagation condition should be removed in the table of test parameters.

Resource pool configuration

Resource pool configuration includes common resource parameters for all test cases, so single common resource pool configuration could be defined for all test cases. Following common resource pool configuration is proposed based on companies’ views from e-mail discussion.

- Proposal 4: Define single common resource pool configuration as shown in Table 2.

Table 2 Proposed common resource pool configuration

| Information Element | Value/Remark | Comment |
| --- | --- | --- |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |
| sl-LengthSymbols-r16 | sym14 |  |
| sl-StartSymbol-r16 | sym0 |  |
| } |  |  |
| sl-BWP-PoolConfigCommon-r16 SEQUENCE { |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SEQUENCE | 1 entry | SL-ResourcePool-r16 |
| sl-TxPoolSelectedNormal-r16 SEQUENCE { |  |  |
| sl-PoolToReleaseList-r16 |  |  |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SEQUENCE { | 1 entry | SL-ResourcePoolConfig-r16 |
| sl-ResourcePoolID-r16 |  |  |
| sl-ResourcePool-r16 { |  |  |
| sl-PSCCH-Config-r16 { |  |  |
| sl-TimeResourcePSCCH-r16 | n2 |  |
| sl-FreqResourcePSCCH-r16 | n10 |  |
| } |  |  |
| sl-PSFCH-Config-r16 { |  |  |
| sl-NumMuxCS-Pair-r16 | n1 |  |
| sl-PSFCH-HopID-r16 | 0 |  |
| sl-PSFCH-CandidateResourceType-r16 | startSubCh |  |
| } |  |  |
| sl-SyncAllowed-r16 | gnss | ENUMERATED {gnss, gnbEnb, ue } |
| sl-SubchannelSize-r16 | n10 |  |
| sl-TimeResource | 11111111 11111111 1111 | Indicates the time resource of resource pool within sl-Period. |
| sl-StartRB-Subchannel-r16 | 0 |  |
| sl-Additional-MCS-Table-r16 | Not presented |  |
| sl-PTRS-Config-r16 | Not presented |  |
| sl-X-Overhead-r16 | n0 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| sl-TxPoolSelectedNormal-r16 |  |  |
| sl-TxPoolExceptional-r16 |  |  |
| } |  |  |
| } |  |  |
| SL-ThresPSSCH-RSRP | 66 | Threshold to allow PSSCH transmission for PSFCH reception is infinity dBm. |

Conclusion

In this contribution, we provide the contents and table format for test case, RMC, and resource pool configuration. We propose based on companies’ views from e-mail discussion

- Proposal 1: Reference measurement channels are defined for different physical channels like LTE V2X

- Proposal 2: Add 2nd stage SCI configuration in PSSCH reference measurement channel table as Table 1

- Proposal 3: Add the information for bandwidth/SCS and propagation condition in the table of minimum performance

- Proposal 4: Define single common resource pool configuration as shown in Table 2.

Reference

[1] R4-2103976, “Draft Big CR: Introduction of Rel-16 NR V2X demodulation performance requirements,” LG Electronics