3GPP TSG-RAN WG4 Meeting # 98-bis-e 													R4-2105181

Electronic Meeting, Apr. 12-20, 2021

Agenda item:			7.20.1

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [98-bis-e][108] NR_PC2_CA_R17_2BDL_2BUL

Document for:	Information

Introduction

This email discussion thread is related to NR PC2 CA basket WI, and will focus on the topic of following aspects:

- Topic #1: draft TR and draft big CR

- Topic#2: UE RF requirements

  - Sub-topic 2-1: UE RF requirements for proposed CA

  - Sub-topic 2-2: MSD improvement

  - Sub-topic 2-3: Power class

Note that the tables for collecting comments for sub-topic issues are arranged just below each issue.

Topic #1: draft TR and big CR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2106277 | China Telecom | Abstract: This draft TR provides the draft TR v0.3.0, which was reserved for email approval and aims to reflect the TP approved in this meeting. |
| R4-2106278 | China Telecom | Draft CR to Introduce the completed combos to 38101-1 |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: draft TR and draft big CR

This sub-topic will discuss rapporteur input for draft TR and draft big CR.

Issue 1-1-1: draft TR

- Recommended WF

  - It is recommended for email approval for the draft TR of R4-2106277

| Company | Comments on Issue 1-1-1: draft TR |
| --- | --- |
| ZTE | We think there is no need to add a new table for OOB blocking for PC2 band combination, since the current table can be applied to both PC2 and PC3 for the same band combination. |
| China Telecom | We have no strong view on OOB blocking table. However, it may need another TP to remove the existing table for OOB blocking agenda in the draft TR. |

Issue 1-1-2: draft big CR

- Recommended WF

  - It is recommended to approve the draft big CR of R4-2106278 and the combos completed in this meeting could be captured into formal big CR in May RAN4 meeting

| Company | Comments on Issue 1-1-2: draft big CR |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: No concern on the draft big CR, the draft big CR R4-2106278 is recommended as endorsed Candidate options: Recommendations for 2nd round: The topic #1 is closed |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2106277 | for email approval |
| R4-2106278 | endorsed |

Discussion on 2nd round (if applicable)

The topic #1 is closed

Topic #2: UE RF requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations/Abstracts |
| --- | --- | --- |
| R4-2104973 | LG Electronics France | Proposal 1: For cross-band isolation issue of PC2 NR inter band CA UE, the proposed MSD values in Table 5 shall be considered in TS38.101-1. Proposal 2: For IMD problem by dual uplink transmission of PC2 NR inter band CA UE, the proposed MSD values in Table 9 shall be considered in TS38.101-1. |
| R4-2106279 | China Telecom | TP to 38.841: RF requirements for PC2 CA_n3A-n78A with up to 2 uplink |
| R4-2106561 | OPPO | Observation 1: 30dB is a big improvement in UE design which is quite challenging to be implemented and is unknown of the UE status on the market. Observation 2: Deploy a band combination only for a small amount of UEs may not wise considering the huge operation costs in the real NW. Proposal 1: It is encouraging proponents to share the statistical measurement data of UEs on the market to understand better on the value of this discussion before moving forward. |
| R4-2106904 | Apple | Remove power class 2 requirement for UL CA_n1A-n78A in Table 6.2A.1.3-1. |
| R4-2107315 | Huawei, HiSilicon | Proposal: It is proposed to consider the UE power class issue for PC2 CA band combination similar to that for PowerClassNRPart for EN-DC. |
| R4-2107333 | T-Mobile USA | TP for TR_38.841 PC2 CA_n25A-n41A |
| R4-2107334 | T-Mobile USA | TP for TR_38.841 PC2 CA_n41A-n66A |
| R4-2107335 | T-Mobile USA | TP for TR_38.841 PC2 CA_n41A-n71A |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: UE RF requirements for proposed CA

This sub-topic will discuss UE RF requirements for proposed combinations.

Issue 2-1-1: MSD analysis for PC2 NR inter-band CA

- Proposals (R4-2104973)

  - Proposal 1: For cross-band isolation issue of PC2 NR inter band CA UE, the proposed MSD values in Table 5 shall be considered in TS38.101-1.

Table 5 MSD due to cross band isolation for PC2 for CA band combinations

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n25 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |
| n41 | n66 | 5.3 | 5.3 | 5.3 | 5.3 |  |  |  |  |  |  |  |  |  |
| n78 | n40 | 6.5 | 6.5 | 6.5 | 6.5 |  |  |  |  |  |  |  |  |  |
| n77 | n41 | 6.5 | 6.5 | 6.5 | 6.5 |  |  |  |  |  |  |  |  |  |
| n41 | n77 |  | 10.5 | 10.5 | 10.5 | 9.5 | 8.6 | 8.3 | 7.2 | 6.3 | 6.0 | 5.7 | 5.6 | [5.6] |

  - Proposal 2: For IMD problem by dual uplink transmission of PC2 NR inter band CA UE, the proposed MSD values in Table 9 shall be considered in TS38.101-1.

- Table 9: Proposed MSD test configuration and results by IMD problems for PC2 NR inter-band CA UE

| CA bands | UL band | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n71A | n41 | IMD4 ¦3*fn71 -fn41¦ | 2614 | 5 | 25 | 2614 | 5 | N/A |
|  | n71 |  | 665 | 5 | 25 | 619 | 5 | 18.2 |
| CA_n71A-n77A | n71 | IMD5 ¦4*fn71 -fn77¦ | 681.5 | 5 | 25 | 635.5 | 5 | 12.2 |
|  | n77 |  | 3361.5 | 10 | 50 | 3582.5 | 10 | N/A |

- Recommended WF

  - Collect views on these  two proposals

| Company | Comments on Issue 2-1-1: MSD analysis for PC2 NR inter-band CA |
| --- | --- |
| Qualcomm | Based on 3GPP PC3 requirement only, and without consideration of MSD improvement for PC2, we expect the following MSD values at the minimum DL BW: Shouldn’t IMD4 desense n71 (FDD) and not n41 (TDD) since later has no 2 uplinks on at same time? For n3A_n78A, we need to account for PC2 harmonic mixing. |
| ZTE | Seems some values are missing for some CBW for some bands. For example, only MSD values for 5/10/15/20MHz are defined for band n41, however, some other CBW such as 30/40/50...MHz are supported in band n41. |
| LGE | The cross band isolation result will be consider to derive MSD requirements for each CA band combinations. The average manner can be reused to derive MSD requirements from interested companies’ MSD results. For the CA_n41A-n71A, sorry for error in Table 9. The MSD 18.2 dB is for reception in n71 by dual UL_CA_n41A-n71A. So, need to consider the revised Table 9 for both CA_n41-n71 and CA_n71A-n77A. |

Issue 2-1-2: TPs for approval

- Proposed TPs

  - R4-2106279, R4-2107333, R4-2107334, R4-2107335

- Recommended WF

  - Collect the comments for proposed TPs. If no comments for certain of TP’s, the TP’s will be recommended as approved.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2106279 | Qualcomm: Missing Harmonic Mixing MSD. Expect 5.7dB MSD for PC3 to 8.1dB MSD for PC2 |
|  | China Telecom: The IMD MSD in this TP is reused from ENDC. We will and encourage companies to provide harmonic mixing MSD in next meeting. With above clarification, can we approve this TP? |
|  | LGE: need to inform the MSD requirements are based on PC2 DC_3A_n78A UE. |
| R4-2107333 | Qualcomm: We expect 1.4dB MSD for PC2 from 0.6dB MSD for PC3 |
|  | LGE: need to merge MSD results from LGE (2.3dB), QC(1.4dB) and TM-US(1.1dB). And MSD will be derived as average manner. The expected MSD level is 1.6dB for PC2. |
| R4-2107334 | Qualcomm: We expect 6.2dB MSD for PC2 from 4.5dB MSD for PC3. |
|  | T-Mobile USA: Above Qualcomm said they expect 5.5 dB MSD from 3.5 dB MSD for PC3. 6.2 dB MSD from 4.5 dB MSD is for CA_n40A_n78A. |
|  | LGE: need to merge MSD results from LGE (5.3dB), QC(6.2dB) and TM-US(5.4dB). And MSD will be derived as average manner. The expected MSD level is 5.6dB for PC2. |
| R4-2107335 | Qualcomm: We expect 17dB MSD (IMD4) for PC2 given 11dB for PC3. |
|  | LGE: need to merge MSD results from LGE (18.2dB), QC(17.0dB) and TM-US(13.8dB). And MSD will be derived as average manner. The expected MSD level is 16.3dB for PC2. |

Issue 2-1-3: Draft CR for CA_n1A-n78A

- Proposal (R4-2106904): Remove power class 2 requirement for UL CA_n1A-n78A in Table 6.2A.1.3-1.

  - Because power class 2 for CA_n1A-n78A requirements have not yet been defined but already captured in Rel-17 TS 38.101-1 Table 6.2A.1.3-1.

- Recommended WF

  - Keep CA_n1A-n78A as it is. Because the requirements have been defined in Table 7.3A.5-1a

| Company | Comments on Issue 2-1-3: Draft CR for CA_n1A-n78A |
| --- | --- |
| ZTE | It seems PC2 n1-n78 was completed and introduced in Rel-16. |
| China Telecom | We share the same view as ZTE that PC2 n1-n78 has been completed. |
| CHTTL | To ZTE, maybe it’s Rel.17? |

Sub-topic 2-2: MSD improvement

Issue 2-2-1: MSD improvement

- Proposals (R4-2106561)

  - Proposal 1: It is encouraging proponents to share the statistical measurement data of UEs on the market to understand better on the value of this discussion before moving forward.

- Recommended WF

  - Collect views on the proposal(s)

| Company | Comments on Issue 2-2-1: MSD improvement |
| --- | --- |
| Qualcomm | We agree with the observation that 30 dB improvement is a lot to ask, not all UE’s will be able to see this much improvement for all combinations. However, even much smaller improvements (5 dB? 10 dB?, 15 dB?) may be valuable. Our general view is that the MSD’s specified are not usable in deployment and that devices are often capable of delivering much better performance. We are trying to narrow the gap between reality and 3GPP specifications. We would appreciate if OPPO (and any other vendor) could help by sharing data on actual commercial device performance and would be happy to work offline to anonymize the data. |
| ZTE | Agree. 30dBm MSD is too large and we believe there are rooms to improve them. But not sure how much the MSD is meaningful in reality? 10dB, 15dB? |
| T-Mobile USA | We support the effort to improve MSD. We’d appreciate more date from vendors. |
| LGE | As same approach in LTE/NR CA will be used to derive MSD for PC2. It means that vendor can provide their assumptions for PCB isolation and apply average manner to derive MSD. |
| OPPO | Agree. The real capability of UE should be clear before move forward on this topic. |
| Huawei | It is no surprise to see some UEs in the market perform better than the 3GPP minimum requirements. Typically high-end UEs are expected to perform better than low-end ones. As commented above, for combos that have up to 30 dB MSD in the current spec (e.g. due to IMD2), it’s very challenging to improve the MSD by 10 or 20 dB. Maybe only a small portion of UEs can achieve, which is not sufficient for an operator to deploy a network using those carrier frequency combinations. The economy of scale would be lost. The problem may be better solved by alternative frequency planning. |
| CHTTL | We support T-Mobile USA’s comment. Also it’s not ok for operator to deploy a network with very large MSD. |
| Apple | Our understanding is that there should be only one set of MSD requirements for each power class. Even if we could agree on improving the values, any improvement would have to affect this single set. This means that all UEs have to meet those requirements. The performance of a UE depends on many aspects. Form factor and integration plays a huge role as well as the individual component quality and performance e.g. duplexer. It is still unclear what we will do with the measurements, especially if the combined set of contributions provide all kinds of results due to the large variety of different implementation choices. Would a tightening mean that certain implementation choices are excluded if most of the contributions have better performance compared to a view performing worse. |

Sub-topic 2-3: Power class

Issue 2-3-1: Power class

- Proposals (R4-2107315)

  - Proposal: It is proposed to consider the UE power class issue for PC2 CA band combination similar to that for PowerClassNRPart for EN-DC.

- Recommended WF

  - Collect views on the proposal(s)

| Company | Comments on Issue 2-3-1: Power class |
| --- | --- |
| Qualcomm | We don’t see the need for this. For inter-band CA, the power class is signaled by the UE and the requirements are defined in the specifiation. If the UE is not able to meet these requirements, it should not signal the power class. Otherwise, configuring the UE to CA would imply that the UE can only support PC3 in one or both carriers which means a loss of coverage even if the allocation is only on the PCC. |
| ZTE | Share the same view as QC. |
| T-Mobile USA | Share the view with Qualcomm and ZTE. We don’t think this is needed. |
| OPPO | Not quite understand the reason of introducing this power class for CA, since in the ENDC UE needs to be tested with the NR band separately according to the NR power class, however, for CA the two bands are tested according to the total power class. Therefore, it seems this is not needed. |
| Huawei | Here’re some clarification on the problem the paper is trying to address. For example, in per-band capability report, a UE may report PC3 on band X and PC2 on band Y. The UE may only be equipped with 2TX (i.e. 23+23), since PC2 on band Y could be implemented by TxD. For CA_X_Y, the UE could also report PC2 for CA power class, since 23 dBm on band X + 23 dBm on band Y is 26 dBm in total. Note that the max power capability on band Y downgrades to PC3 from PC2 when CA is configured. If the network continue to assume PC2 on band Y, it would misinterpret PHR during per-cell power control. In summary, depending on UE implementation, the power class of a component band in a CA could be different from the power class of that band w/o CA. The network may not be aware of this change. Hence a solution is proposed to address the problem. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 2-1: UE RF requirements for proposed CA | Recommendations for 2nd round: Focus on the four revised TPs, they are R4-2106279, R4-2107333, R4-2107334, R4-2107335. The corresponding revised number will be allocated in chairman’s report. |
| Sub-topic 2-2: MSD improvement | Tentative agreements: No consensus on this sub-topic, it seems we have repeated the same situation for companies’ standpoints with last meeting. Recommendations for 2nd round: Continue discussion in 2nd round. |
| Sub-topic 2-3: Power class | Recommendations for 2nd round: Continue discussion in 2nd round. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2106279 | revised |
| R4-2107333 | revised |
| R4-2107334 | revised |
| R4-2107335 | revised |

Discussion on 2nd round (if applicable)

Summary for 2nd round

|  | Status summary |
| --- | --- |

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |