3GPP TSG-RAN WG4 Meeting # 98-bis-e										              R4-2105194

Electronic Meeting, Apr. 12-Apr. 20, 2021

Agenda item:			7.35

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [98-bis-e][121] Simultaneous_RxTx

Document for:	Information

# Introduction

This email thread discuss Rel-17 PC2 HPUE for NR sidelink enhancements.  The contributions are in agenda 7.35, which includes:

- Topic #1: Principles for simultaneous Rx/Tx capability

  - Issue 1-1: Applicability of simultaneous Rx/Tx capability for similar band combination

  - Issue 1-2: Rules to decide simultaneous Rx/Tx capability for a band combination

  - Issue 1-3: Specification impact

  - Issue 1-4: Release independent

- Topic #2: TR skeleton

# Topic #1: Issues related to PC2 HPUE for SL enhancements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2106386 | ZTE | Proposal 1: Same simultaneous Rx/Tx capability for NR DC band combination and the corresponding inter-band NR CA band combination. Proposal 2: Same simultaneous Rx/Tx capability for inter-band NR CA band combination and the corresponding inter-band ENDC band combination. FDD+TDD NR CA: Proposal 3. Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR1 FDD-TDD CA band combination. Proposal 4. Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR2 FDD-TDD CA band combination. TDD+TDD NR CA: Proposal 5. Simultaneous Rx/Tx capability should be discussed case by case for the FR1+FR1 TDD-TDD CA band combination. Proposal 6. Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR2 TDD-TDD CA band combination. Proposal 7. FFS on simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD CA band combination. SUL: Proposal 8. For SUL band combination, if SUL band is from FDD band, mandatory simultaneous Rx/Tx capability should be supported. If SUL band is from TDD band, simultaneous Rx/Tx capability should be discussed case by case. |
| R4-2106553 | OPPO | Observation 1: It was agreed TDD-FDD EN-DC band combinations which have corresponding LTE CA band combinations, the simultaneous TxRx capability will be same as LTE, but this capability was missing in NR spec. Observation 2: For the TDD-TDD or TDD-FDD band combinations that don’t have corresponding LTE CA band combinations, no clear definition of criteria that can be carried out in practice up to now. Observation 3: The MSD values or the harmonic / IMD orders are more clear and operable criteria to decide the mandatory or optional simultaneous TxRx capability. And they can be used to replace the vague criteria like “isolation” or “close to the other band”. Proposal 1: It is proposed to align the simultaneous TxRx capability in NR specification for the band combinations which have corresponding LTE CA band combinations. Proposal 2: It is proposed to define more operable criteria like MSD value based or the harmonic / IMD order based criteria for the simultaneous TxRx capability. Observation 4: Once criteria are defined it can be used either to decide the simultaneous TxRx capability or initial classify band combinations for further evaluation. Observation 5: Optional simultaneous TxRx can be assumed for the band combinations which doesn’t have explicit statement of this capability in Rel-15/Rel-16 spec. Proposal 3: It is proposed to specify/change the simultaneous TxRx capability only in the Rel-17 spec in this WI and keep Rel-15/Rel-16 simultaneous capability unchanged to avoid NBC issue. |
| R4-2107047 | CHTTL | Proposal 1: Simultaneous RxTx shall be mandatory for TDD-FDD combinations by following the updated general criteria as listed below: - The capability shall be mandatory if low-band (below 1GHz) is aggregated with high (i.e. 2.69GHz and above) and mid-band (1GHz to 2.69GHz) TDD cells. - The capability shall also be mandatory if mid-band (1GHz to 2.69GHz) FDD is aggregated with frequency above 3.3GHz high band TDD cells. - The capability shall also be mandatory if FR1 FDD band is aggregated with FR2 TDD bands. Proposal 2: For the higher order combinations (including 3 bands and above), the capability shall be mandatory if all of the two bands fallback combinations are mandatory simultaneous RxTx support. Proposal 3: Consider specify the text for the general criteria in the specification so that the combinations that satisfy the general criteria will be mandatory simultaneous RxTx support. |
| R4-2107310 | Huawei, HiSilicon | Based on the previous discussion both in RAN4 and RAN, at least the following aspects should be considered when we discuss the principles to identify the simultaneous Rx/Tx capability for a band combination. - How to avoid a combination does not indicate its simultaneous Rx/Tx capability accurately according to the existing RAN2 signaling design - Which rules could be used to just a band combination could support simultaneous Rx/Tx capability, e.g. frequency separation between operating bands - If performance degradation exceeds a threshold (TBA), e.g. MSD degradation, whether the band combination can be considered as not supporting simultaneous Rx/Tx capability - Whether the simultaneous Rx/Tx capability for the fall back mode can be inherited to the high order band combinations - Whether the simultaneous Rx/Tx capability for CA could be different from that of MR-DC or NR-DC for the same band combination - Whether by default FDD-TDD band combinations can be considered to support simultaneous Rx/Tx capability unless specific issues are identified for a band combination Proposal 1: It is proposed to consider the principles based on the aspects listed in the contribution, while other aspects are not precluded. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 1-1: Applicability of simultaneous Rx/Tx capability for similar band combination

Issue 1-1-1: Whether same simultaneous Rx/Tx capability for NR CA band combination can be applied for the corresponding inter-band NR CA band combination

- Option 1: Yes.

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-1-2: Whether same simultaneous Rx/Tx capability for inter-band NR CA band combination can be applied for the corresponding inter-band ENDC band combination

- Option 1: Yes.

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-1-3: For the higher order combinations (including 3 bands and above), the capability shall be mandatory if all of the two bands fallback combinations are mandatory simultaneous RxTx support

- Option 1: Yes.

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 1-2: Rules to decide simultaneous Rx/Tx capability for a band combination

Issue 1-2-1: Simultaneous Rx/Tx capability according to frequency separation

Issue 1-2-1a: The capability shall be mandatory if low-band (below 1GHz) is aggregated with high (i.e. 2.69GHz and above) and mid-band (1GHz to 2.69GHz) TDD cells

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-1b: The capability shall be mandatory if mid-band (1GHz to 2.69GHz) FDD is aggregated with frequency above 3.3GHz high band TDD cells

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-1c: The capability shall be mandatory if FR1 FDD band is aggregated with FR2 TDD bands

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-2: Simultaneous Rx/Tx capability according to MSD

Issue 1-2-2a: Whether MSD value/threshold or harmonic / IMD order can also be considered as criteria to decide simultaneous Rx/Tx capability

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-3: Simultaneous Rx/Tx capability according to duplex mode and frequency range

Issue 1-2-3a: Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR1 FDD-TDD CA band combination

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-3b: Simultaneous Rx/Tx capability should be discussed case by case for the FR1+FR1 TDD-TDD CA band combination

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-3c: Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR2 TDD-TDD CA band combination

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-3d: FFS on simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD CA band combination

- Option 1: Yes

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

Issue 1-2-3e: For SUL band combination, if SUL band is from FDD band, mandatory simultaneous Rx/Tx capability should be supported.  If SUL band is from TDD band, simultaneous Rx/Tx capability should be discussed case by case.

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 1-3: Specification impact

Issue 1-3-1: Specify the text for the general criteria in the specification so that the combinations that satisfy the general criteria will be mandatory simultaneous RxTx support

- Option 1: Yes.

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 1-4: Release independent

1-4-1: From which release the capability can be indicated for a band combination introduced in Rel-17

- Option 1: Rel-15.

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

1-4-2: Specify/change the simultaneous TxRx capability only in the Rel-17 spec in this WI and keep Rel-15/Rel-16 simultaneous capability unchanged to avoid NBC issue.

- Option 1: Yes

- Option 2: FFS

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

## Companies views’ collection for 1st round

### Open issues

| Issues | Company Comments |
| --- | --- |
| 1-1: Applicability of simultaneous Rx/Tx capability for similar band combination | Issue 1-1-1: Whether same simultaneous Rx/Tx capability for NR CA band combination can be applied for the corresponding inter-band NR CA band combination OPPO: The question here should be “LTE CA be applied to NR CA?”, if it is, then Yes, it was agreed in Rel-15, but unfortunately was never implemented in spec. CHTTL: prefer if it is mandatory simultaneous Rx/Tx capability support for inter-band LTE CA band combination, than it is also mandatory for the corresponding inter-band NR CA band combination ZTE: Not clear for the question. If it is clarified by OPPO, then yes. CMCC:Yes Qualcomm: We agree with Oppo’s clarification. LGE : we are same view with OPPO Vivo: share same view with oppo’s clarification. Xiaomi: it’s yes if it is like OPPO’s clarification, Apple: No, because whether the NR-DC configuration is synchronous or not can have an impact. Huawei: OK with the clarification by OPPO. For NR DC, need more consideration. |
|  | Issue 1-1-2: Whether same simultaneous Rx/Tx capability for inter-band NR CA band combination can be applied for the corresponding inter-band ENDC band combination OPPO: Yes. CHTTL: prefer if it is mandatory simultaneous Rx/Tx capability support for inter-band NR CA band combination, than it is also mandatory for the corresponding inter-band EN-DC band combination ZTE: Yes Qualcomm: Yes Apple: No, because whether the NR-DC configuration is synchronous or not can have an impact. CMCC: Yes SoftBank: Yes LGE: Yes Vivo: Yes NTT DOCOMO: Yes Skyworks: Yes Xiaomi: Yes Huawei: Yes |
|  | Issue 1-1-3: For the higher order combinations (including 3 bands and above), the capability shall be mandatory if all of the two bands fallback combinations are mandatory simultaneous RxTx support OPPO: Yes, if the UL only consider 2CC. CHTTL: Yes ZTE: Yes Qualcomm: Yes CMCC: Yes SoftBank: Yes LGE :Yes Vivo: Yes NTT DOCOMO: Yes Skyworks: Yes Xiaomi: propose higher order combination with 2 band ULs follows the same rules with their all two bands fallback combination, for example, CA_nX-nY-nZ, if fallback mode CA_nX-nY_UL_nX-nY supports simultaneous RxTx mandatorily, and fallback mode CA_nX-nZ_UL_nX-nZ doesn’t support simultaneous RxTx, then CA_nX-nY-nZ_UL_nX-nY supports simultaneous RxTx mandatorily, and CA_nX-nY-nZ_UL_nX-nZ doesn’t support simultaneous RxTx. So, we just need note whether the two bands combination supports simultaneous RxTx mandatorily. Huawei: Yes |
| 1-2: Rules to decide simultaneous Rx/Tx capability for a band combination | Issue 1-2-1: Simultaneous Rx/Tx capability according to frequency separation Issue 1-2-1a: The capability shall be mandatory if low-band (below 1GHz) is aggregated with high (i.e. 2.69GHz and above) and mid-band (1GHz to 2.69GHz) TDD cells OPPO: Yes, this was agreed in Rel-15 CHTTL: Yes this was agreed in Rel.15, (so actually the proposal is to extend to the next issue below. ZTE: Yes Qualcomm: This question is somewhat contradicting with the question on deciding by MSD. SoftBank: Yes LGE: Yes Vivo: Share Qualcomm’s view that the questions are not totally independent. NTT DOCOMO: Yes Skyworks: Yes Xiaomi: Yes Apple: Yes Huawei: Yes Issue 1-2-1b: The capability shall be mandatory if mid-band (1GHz to 2.69GHz) FDD is aggregated with frequency above 3.3GHz high band TDD cells OPPO: FFS. Discuss per band combination might be better. CHTTL: Yes, we hope to push the general rule further a little bit, as you can see the capability is mandatory for DC_7A_n78A, also for the FDD-TDD combinations that have larger frequency separation. Would like to know if there is any exceptional case that can not mandatory support? ZTE: Yes. Same views as CHTTL. We also would like to see some general rules. Need to understand the RF architecture to implement the FDD-TDD inter-band CA not supporting simultaneous Rx/Tx . Qualcomm: This question is somewhat contradicting with the question on deciding by MSD. WE should discuss case by case, harmonic mixing could be an issue. SoftBank: Yes LGE: it is up to band specific issues since there are different RF component characteristics. So it is quite difficult to make general rule. Vivo: Share Qualcomm’s view that there are other issues may need to be considered. NTT DOCOMO: Yes Skyworks: our understanding is that for FDD-TDD, simultaneous Tx/Rx is mandatory except for cases where we have close proximity or little filter rejection. We can not think of any example of FDD-TDD, but we have the example of SUL_n41-n97 this week where MSD may be high. If we assume a SUL aggressor band is similar case than an FDD aggressor, than is it correct to assume this SUL combination is similar case than FDD-TDD where simultaneous Tx/Rx needs to be discussed ? Xiaomi: FFS Apple: we prefer to discuss the capability related to these scenarios on a case by case basis Huawei: Yes. For band combinations with MSD, corresponding requirements can be specified. If MSD is too high for specific FDD-TDD band combinations, which can be treated separately. Issue 1-2-1c: The capability shall be mandatory if FR1 FDD band is aggregated with FR2 TDD bands OPPO: Yes. CHTTL: Yes ZTE: Yes CMCC:Yes Qualcomm: FR1 was extended to 7GHz, this needs some more analysis. SoftBank: Yes LGE: Yes NTT DOCOMO: Yes. And FR1 TDD+FR2 TDD should be mandatory as well. Xiaomi: Yes Apple: Yes Huawei: Yes |
|  | Issue 1-2-2: Simultaneous Rx/Tx capability according to MSD Issue 1-2-2a: Whether MSD value/threshold or harmonic / IMD order can also be considered as criteria to decide simultaneous Rx/Tx capability OPPO: Yes, clear criteria is good to decide the simultaneous RxTx capability and is easy to judge what the capability is when future more and more band combinations are introduced. Besides, when band combinations are introduced the MSD/IMD/harmonics will be analyzed, then based on the analyze results the simultaneous RxTx can be determined. CHTTL: No, would also like to know the proposal is for TDD-TDD or FDD-TDD. For FDD-TDD, please see our comment on Issue 1-2-3a. Also it already implies simultaneous RxTx is supported when specifying MSD, otherwise there is no MSD….ZTE: No. Procedure looks a little weird. We think MSD analysis depends on the simultaneous Rx/Tx capability, seems not feasible vice verse. If simultaneous Rx/Tx capability is no supported, then no further actions for MSD analysis at all. Qualcomm: We agree with this criteria, if MSD is too high then it doesn’t make sense to support simultaneous Rx/Tx. LGE: there are to candidate option for support simultaneous Rx/Tx capability. One is define MSD without any freq. separation. 2nd one is that the specific freq. separation is guaranteed, then MSD do not define or allow small MSD values. Another option is only allow freq. separation without MSD. Vivo: Yes, prefer this one over the speration based solution. Skyworks: Same view as Qualcomm,. Question may also consider front end architecture constaraints as agreed for example in DC_42_n79 Xiaomi: how much is too high the MSD, it is hard to form a criterion. Apple: the capability depends on scenario, requirements, and UE implementation; just an MSD threshold does not capture the complete picture. Huawei: Yes. For band combinations which have MSD issues, threshold could be considered as judgement to decide if the combination can support simultaneous Rx/Tx capability. Whether the threshold can be accepted for certain combination, views from proponents/operators can be taken into consideration. |
|  | Issue 1-2-3: Simultaneous Rx/Tx capability according to duplex mode and frequency range Issue 1-2-3a: Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR1 FDD-TDD CA band combination OPPO: No, should be per band combination specific rather than duplex mode. CHTTL Yes, we support this proposal. Also this should also expand to the FDD-TDD EN-DC cases. We are not sure what excalty means and how the UE will support if an FDD-TDD combination can not support simultaneous Rx/Tx ? As the FDD part is already simultaneous Rx/Tx. Would like to know if there is any answer from RAN4. ZTE. Yes. Same views as CHTTL. ENDC and NR CA should kept consistency. For TDD-FDD NR CA, if simultaneous Rx/Tx is not supported, does it mean TDD band cannot work, or UL and DL for FDD cannot work at the same time...Need to understand the RF architecture to implement the FDD-TDD inter-band CA not supporting simultaneous Rx/Tx. Qualcomm: No, should be per band combination. this question is related to Issue 1-2-2 SoftBank: Yes. Our understanding is that the coexistence study in each band combination was basically conducted under the condition of simultaneous Rx/Tx. If not, the limitation is clarified in the TP. LGE: No, RAN4 shall treat case by case for the impact in both FDD DL band and TDD band such as Band 7 and Band 38 or Band 1 and Band 34. Vivo: No, a case by case determination is more preferred. NTT DOCOMO: We would like to set mandatory as default, i.e., unless otherwise stated. But we can discuss its optionally for each band combination if problem is identified. Xaiomi: No, hope discuss case by case, and this question is related to Issue 1-2-2 Apple: This question seems to have already been addressed in Issue 1-2-1; in the scenarios according to Issues 1-2-1a and 1-2-1c, a rule can be derived. But in the case of Issue 1-2-1b, we prefer to define the capability on a case by case basis. Huawei: Yes. We think by default the capability can be assumed for TDD-FDD band combinations. If some band combinations have MSD issue, corresponding MSD requirements can be specified. Unless the MSD is too high to be acceptable for specific FDD-TDD band combination, which can be indicated separately. Issue 1-2-3b: Simultaneous Rx/Tx capability should be discussed case by case for the FR1+FR1 TDD-TDD CA band combination OPPO: Ok. ZTE. Yes. Case by case. Qualcomm: ok LGE: Ok. Vivo：ok Xiaomi: Yes Apple: We should clarify this issue only applies to inter-band TDD CA; we agree this should be discussed case by case. Huawei: Yes Issue 1-2-3c: Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR2 TDD-TDD CA band combination OPPO: OK. ZTE. Yes. Qualcomm: TDD bands are higher, going up to 7GHz now, this needs more analysis. SoftBank: Yes. LGE: Ok. Vivo: Ok for current bands, more study as Qualcomm’s may also useful. Xiaomi: Yes Apple: Agree Issue 1-2-3d: FFS on simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD CA band combination OPPO: OK. ZTE. Yes. FFS Qualcomm: should be discussed case by case. LGE: Ok. Xiaomi: Yes, FFS Apple: We don’t think is a feasible scenario to consider and recommend not persuing. Huawei: Yes Issue 1-2-3e: For SUL band combination, if SUL band is from FDD band, mandatory simultaneous Rx/Tx capability should be supported. If SUL band is from TDD band, simultaneous Rx/Tx capability should be discussed case by case. OPPO: For the FDD band if it is low band then ok, otherwise, should be per band combination discuss ZTE. Yes. Same with FDD-TDD/TDD-TDD NR CA. CMCC: We are concerned that if SUL TDD +NR TDD does not support simultaneous Rx/Tx, it means that this is a frequency division and half duplex mode of operation band combination, and this low spectrum utilization is somewhat inconsistent with the operator's deployment request. We would like to see that some companies can put forward a construction method that makes SUL TDD+NR TDD mandatory to support simultaneous Rx/Tx. Qualcomm: we cannot agree with this blanket statement, should be discussed case by case. Skyworks: we need to analyse case by case based on MSD and front-end architectures. Xaiomi: No, hope discuss case by case Huawei: Same consideration as FDD-TDD CA, mandatory capability can be considered for SUL band combinations. Also agree with CMCC, if MSD is needed, the corresponding requirements can be specified. |
| 1-3: Specification impact | Issue 1-3-1: Specify the text for the general criteria in the specification so that the combinations that satisfy the general criteria will be mandatory simultaneous RxTx support OPPO: OK in general and wording can be discussed. CHTTL: Support in general, we hope with the general wording in the spec can reduce the effort on maintaining notes, since the general criteria needs to be discussed first, the wording can be discussed later on. ZTE: Support. Same view as CHTTL Qualcomm: This can be documented in some TR but does not belong in the specifications. SoftBank: Yes. LGE: we are not sure how to define the general criteria in TR. Maybe RAN4 can try to make consensus. Vivo：No strong view. Slight prefer in a TR since may not that necessary for a TS. Xiaomi: we should first discuss Issue 1-1 and Issue 1-2, then come back to discuss this issue. Apple: regardless of the general rules we might use to derive the requirements, the specification should explicitly identify each band combination for which the UE shall signal mandatory simultaneous RxTx support. Huawei: The agreed principles need to be captured. Whether TR or TS can be further discussed when we have more analysis for the band combinations and have a better understanding and alignment of the principles. |
| 1-4: Release independent | Issue 1-4-1: From which release the capability can be indicated for a band combination introduced in Rel-17 OPPO: Yes, Rel-17 ZTE: Not sure from Rel-15 is still possible. Qualcomm: Rel.17 SoftBank: No. It seems that this may have contradiction of the release independence manner. If Rel-16 UE support the band combination specified in Rel-17 based on the release independence, this UE automatically support simultaneous TxRx capability optional regardless of the general rules if it is from Rel-17. LGE: Yes, from Rel-17 Vivo：Prefer Rel-17. NTT DOCOMO: We are not sure if we understand this question clearly, but the simultaneous RxTx capability was introduced in Rel-15, so the capability can be indicated for all band combination introduced in Rel-15 and later if the band combination has a requirement for simultaneous RxTx such as MSD if needed. Xiaomi: prefer Rel-17 Apple: the capability signaling should match the band combination release Huawei: The capability is introduced in Rel-15. For the band combinations introduced in Rel-17, we think the capability can be supported with the release independent manner. |
|  | Issue 1-4-2: Specify/change the simultaneous TxRx capability only in the Rel-17 spec in this WI and keep Rel-15/Rel-16 simultaneous capability unchanged to avoid NBC issue. OPPO: Yes. CHTTL: still we hope it is possible to have a chance to modify the R15/R16 spec as correction. Currently maybe the note is missing for some combinations. ZTE: Same view as CHTTL Qualcomm: Yes under the condition that we still discuss case by case whether to change support to mandatory. SoftBank: No. Same view as CHTTL. LGE: Yes NTT DOCOMO: Same view with CHTTL. Xiaomi: Yes Apple: We agree: this work item does not have the scope to impact previous (and closed) releases. Huawei: Prefer to discuss case by case, and ok with the possibility to change early release spec to address operators’ concern. |
| Others |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF ahe ssignment.

|  | Status summary |
| --- | --- |
| Topic#1 | Issue 1-1-1: Whether same simultaneous Rx/Tx capability for LTE CA band combination can be applied for the corresponding inter-band NR CA band combination Tentative agreements: Yes. Simultaneous Rx/Tx capability for NR CA band combination can be applied for the corresponding inter-band NR CA band combination. Candidate options: Recommendations for 2nd round: Companies can further provide comments for the issue below: Issue 1-1-1a: Whether same simultaneous Rx/Tx capability for NR DC band combination can be applied for the corresponding inter-band NR CA band combination Issue 1-1-2: Whether same simultaneous Rx/Tx capability for inter-band NR CA band combination can be applied for the corresponding inter-band ENDC band combination Tentative agreements: Candidate options: Option 1: Yes, same simultaneous Rx/Tx capability for inter-band NR CA band combination can be applied for the corresponding inter-band ENDC band combination. Option 2: whether the NR-DC configuration is synchronous or not can have an impact on the capability as that for CA Recommendations for 2nd round: One company has concern on NR-DC. Check in the 2nd round whether option 1 is agreeable. Issue 1-1-3: For the higher order combinations (including 3 bands and above), the capability shall be mandatory if all of the two bands fallback combinations are mandatory simultaneous RxTx support Tentative agreements: Yes. For the higher order combinations (including 3 bands and above), the capability shall be mandatory if all of the two bands fallback combinations are mandatory simultaneous RxTx support. Candidate options: Recommendations for 2nd round: |
| Topic#2 | Issue 1-2-1: Simultaneous Rx/Tx capability according to frequency separation Issue 1-2-1a: The capability shall be mandatory if low-band (below 1GHz) is aggregated with high (i.e. 2.69GHz and above) and mid-band (1GHz to 2.69GHz) TDD cells Tentative agreements: Candidate options: Option 1: Yes. The capability shall be mandatory if low-band (below 1GHz) is aggregated with high (i.e. 2.69GHz and above) and mid-band (1GHz to 2.69GHz) TDD cells. Option 2: The frequency separation can be considered together with MSD rule Recommendations for 2nd round: Check in the 2nd round whether option 1 is agreeable as the frequency separation is large enough. Issue 1-2-1b: The capability shall be mandatory if mid-band (1GHz to 2.69GHz) FDD is aggregated with frequency above 3.3GHz high band TDD cells Tentative agreements: Candidate options: Option 1: study case by case Option 2: Yes. Simultaneous capability shall be mandatory for the band combination in mid-band (1GHz to 2.69GHz) aggregated with frequency above 3.3GHz high band TDD cells. Option 3: Yes. On top of mandatory capability, for the band combination has MSD issue, specify the corresponding MSD requirements. Recommendations for 2nd round: Further discuss the options. Issue 1-2-1c: The capability shall be mandatory if FR1 FDD band is aggregated with FR2 TDD bands Tentative agreements: Candidate options: Option 1: The capability shall be mandatory if FR1 FDD band is aggregated with FR2 TDD bands Option 2: More analysis is needed as FR1 is extended to 7GHz Recommendations for 2nd round: Check if option 1 is agreeable in 2nd round. Issue 1-2-2: Simultaneous Rx/Tx capability according to MSD Issue 1-2-2a: Whether MSD value/threshold or harmonic / IMD order can also be considered as criteria to decide simultaneous Rx/Tx capability Tentative agreements: Candidate options: Option 1: MSD degradation can be considered as a rule to decide whether simultaneous Rx/Tx capability can be supported. Option 2: MSD can be considered together with frequency separation. For some mandatory support band combinations, MSD may need to be defined. Recommendations for 2nd round: Further discuss for the two options. Other options are not precluded. Issue 1-2-3: Simultaneous Rx/Tx capability according to duplex mode and frequency range Issue 1-2-3a: Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR1 FDD-TDD CA band combination Tentative agreements: Candidate options: Option 1: Study case by case. Can be considered together with MSD rule. Option 2:Mandatory support Recommendations for 2nd round: Further discuss for the two options. Issue 1-2-3b: Simultaneous Rx/Tx capability should be discussed case by case for the FR1+FR1 TDD-TDD CA band combination Tentative agreements: It is agreed that simultaneous Rx/Tx capability should be discussed case by case for the FR1+FR1 TDD-TDD CA band combination. Candidate options: Recommendations for 2nd round: Issue 1-2-3c: Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR2 TDD-TDD CA band combination Tentative agreements: Candidate options: Option 1: Yes. Mandatory simultaneous Rx/Tx capability should be supported for all of the FR1+FR2 TDD-TDD CA band combination. Option 2: More analysis is needed. Recommendations for 2nd round: Further check in 2nd round if option 1 is agreeable. Issue 1-2-3d: FFS on simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD CA band combination Tentative agreements: Candidate options: Option 1: Study case by case Option 2: Not a valid scenario Recommendations for 2nd round: Further discuss for the two options. Issue 1-2-3e: For SUL band combination, if SUL band is from FDD band, mandatory simultaneous Rx/Tx capability should be supported. If SUL band is from TDD band, simultaneous Rx/Tx capability should be discussed case by case. Tentative agreements: Candidate options: Option 1: Study case by case Option 2: Mandatory support Recommendations for 2nd round: Further discuss for the two options. |
| Topic#3 | Issue 1-3-1: Specify the text for the general criteria in the specification so that the combinations that satisfy the general criteria will be mandatory simultaneous RxTx support Tentative agreements: The general criteria can be documented in somewhere. Candidate options: Option 1: Documented in TR Option 2: Documented in TS if needed. Recommendations for 2nd round: Based on the tentative agreement, further discuss for the two options. |
| Topic#4 | Issue 1-4-1: From which release the capability indicated for a band combination introduced in Rel-17 Tentative agreements: Candidate options: Option 1: Rel-17 Option 2: Follow the normal release independent manner Recommendations for 2nd round: Further discuss for the two options. Issue 1-4-2: Specify/change the simultaneous TxRx capability only in the Rel-17 spec in this WI and keep Rel-15/Rel-16 simultaneous capability unchanged to avoid NBC issue. Tentative agreements: Candidate options: Option 1: Spec impact for Rel-17 only Option 2: Could allow changes for Rel-15/Rel-16 specs. Recommendations for 2nd round: Further discuss for the two options. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on Simultaneous Rx/Tx | Huawei, HiSilicon |
| #2 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Topic #2: TR skeleton

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2107311 | Huawei, HiSilicon | TR skeleton for simultaneous Rx/Tx band combinations |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 2-1: TR skeleton revision

Suggestions for the improvement of TR skeleton

Moderator’s recommendation:

- Recommended WF

  - Comments and suggestions are collected in 1st round discussion

## Companies views’ collection for 1st round

### Open issues

| Issues | Company Comments |
| --- | --- |
| 2-1: TR skeleton revision | Suggestions for the improvement of TR skeleton Company A |
| Others |  |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#2 | Issue 2-1: TR skeleton revision Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2107311 | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” No comments in 1st round discussion. TR skeleton is agreeable. |

## Discussion on 2nd round (if applicable)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on Simultaneous Rx/Tx | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2106386 | Discussion on Simultaneous RxTx | ZTE | Noted |  |
| R4-2106553 | R17 Discussion on simultaneous TxRx | OPPO | Noted |  |
| R4-2107047 | Discussion on general principle for simultaneous Rx/Tx band combinations for CA, SUL, MR-DC and NR-DC | CHTTL | Noted |  |
| R4-2107310 | On principles for deciding simultaneous Rx/Tx capability | Huawei, HiSilicon | Noted |  |
| R4-2107311 | TR skeleton for simultaneous Rx/Tx band combinations | Huawei, HiSilicon | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents