3GPP TSG RAN WG4 Meeting #98BIS-e                   	R4-2105405

Electronic meeting, 12th – 20th April, 2021

Agenda Item:	8.10.5, 8.10.6

Source:	LG Electronics

Title:	TP on coexistence evaluation and MPR/A-MPR simulation assumptions for left over issues

Document for:	Approval

- Introduction

In the last RAN4 #98 meetings and e-mail discussion, RAN4 agreed the general MPR/A-MPR simulation assumptions for leftover issues for 5G V2X service.

In this paper, we propose to capture MPR/A-MPR simulation assumptions for left over issues such as PC2 V2X operation in ITS spectrum and intra-ban con-current operation in licensed band.

****************** Start of the TP in TR38.xxx ************************

5	Leftover RF requirements

5.1	Power class 2 sidelink UE

5.1.1	Coexistence evaluation for PC2 SL UE in licensed band

In this section, the adjacent channel system coexistence evaluation for PC2 NR V2X operation was studied for NR V2X services.  The operating scenarios include the case where the carrier is deployed for NR V2X service in licensed spectrum. For the licensed carrier, the coexistence evaluation include LTE or NR Uu on the adjacent carrier of NR V2X.

5.1.1.1 Coexistence evaluation scenarios

The adjacent channel coexistence evaluation scenarios for PC2 NR V2X service are shown in Table 5.1.1.1-1

Table 5.1.1.1-1: The adjacent channel coexistence scenarios for PC2 NR V2X service

| NR V2X operating frequency |  | Deployment scenarios (Aggressor-to-Victim) |
| --- | --- | --- |
| FR1 | Scenario A: V2X service at licensed band where only NR SL is supported. (TDD: 2.6GHz) (2nd priority) | - Case1: PC2 NR V2X UE-to- PC2 NR V2X UE - Case2: PC2 NR V2X UE-to- PC3 NR V2X UE |
|  | Scenario B: V2X service at licensed bands where NR SL and NR Uu are supported. (TDD: 2.6GHz) (1st priority) | - Case3: : PC2 NR V2X UE-to-NR Uu BS - Case4: NR Uu UE-to- PC2 NR V2X UE |

Basic simulation parameters are below

- Deployment scenarios: Urban Manhattan grid model

- Simulation Block Size :

  - Urban : Manhattan grid model: 3*433m, 3*250m

- RAN1 dependent parameter

  - For licensed band, NR SL operation in Uplink band in FDD, UL opportunity in TDD is considered.

  - For SINR calculation in partial overlapping between aggressor and victim, worst case SINR should be considered.

The details of the deployment scenarios are presented in the following clauses.

5.1.1.2 Coexistence simulations assumptions

5.1.1.2.1 Layout model

RAN4 reuse the Manhattan Grid model based on the network layout model for licensed band as shown in section 5.2.1 and section 5.2.2 in TR38.886.

5.1.1.2.2 Simulation parameters

In Table 5.1.1.2.2-1 and Table 5.1.1.2.2-2, RAN4 provide detail simulation parameters for PC2 coexistence evaluation in licensed band.

Table 5.1.1.2.2-1: Simulation parameters in licensed band for scenarios A

| Parameter | Value |  |
| --- | --- | --- |
|  | NR V2X UE (Aggressor) | NR V2X UE (Victim) |
| Tx power | 26dBm | 23dBm or 26dBm |
| Channel Bandwidth | 20MHz | 20MHz |
| Packet size | 1) 14 PRB (190 byte packet) for 15kHz SCS 2) Other options are not precluded | 1) 14 PRB (190 byte packet) for 15kHz SCS 2)Other options are not precluded |
| Traffic model | 1 transmission every 100ms - 100ms message generation period - Time instance of message generation is randomized among vehicles |  |
| Noise figure | 9dB | 9dB |
| Antenna pattern | Omni-directional with gain of 0 dBi |  |
| Sidelink Power control | The worst case of no power control is used |  |
| SINR-to-BLER mapping | As per link level performance model in TR 38.xxx Table A-x for 2.6GHz | As per link level performance model in TR 38.xxx Table A-x for 2.6GHz |

Table 5.1.1.2.2-2: Simulation parameters in licensed band for scenarios B

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  |  | NR UE |  | NR BS | NR V2X UE |
| Max Tx power |  | 23dBm or 26dBm |  | NA | 26dBm |
| Channel Bandwidth |  | 20MHz |  | 20MHz | 20MHz |
| Packet size |  | 1) [32] PRB for 15kHz SCS 2) Other options are not precluded |  |  | 1) 14 PRB (190 byte packet) for 15kHz SCS 2) Other options are not precluded |
| Traffic model |  | Full buffer |  | Full buffer | Reference table 5.2.1.2-1 |
| Noise figure |  | N/A |  | 5dB | 9dB |
| Antenna pattern |  | Omni-directional with gain of 0 dBi |  | Antenna pattern for FR1 Macro BS from TR 38.828 | Omni-directional with gain of 0 dBi |
| SINR-to-BLER mapping for NR V2X |  | NA |  | NA | As per link level performance model in TR 38.xxx Table A-x for 2.6GHz |
| SINR-to-rate mapping for NR |  | NA |  | As per link level performance model in TR 36.942 (Table A.2). α, attenuation = 0.4, SNIRMIN, dB = -10, SNIRMAX, dB = 22 (subclause 5.2.3.6 from TR 38.828). | NA |

        - ACLR and ACS

RAN4 only consider 1step ACLR/ACS model to derive the PC2 coxistence evaluation in licensed band

Table 5.1.1.2.3-1: ACLR and ACS in licensed band for scenarios A

| Parameter | Value |  |
| --- | --- | --- |
|  |  |  | PC2 NR V2X UE (Aggressor) |  |  | NR V2X UE (Victim PC3/PC2) |  |
| ACLR |  | 31+XdB |  |  | 30 + X dB 31 + X dB |  |  |
| ACS |  |  | 27+XdB |  |  | 27+ X dB |  |

Table 5.1.1.2.3-2: ACLR and ACS in licensed band for scenarios B

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | NR UE (PC3/PC2) | NR BS | NR V2X UE (PC2) |
| ACLR | 30 dB/31 dB | NA | 31 + X dB |
| ACS | NA | 46 dB | 27 + X dB |

        - Power control

For V2X operating scenarios A, RAN4 make consensus to reuse the OLPC in TR36.786 or no power control is considered.

For V2X operating scenarios B, the power control mechanism which was specified in clause 5.2.3.4 in TR38.886 for FR1 TDD band can be reused.

5.1.1.3 Coexistence results

5.1.1.4 Conclusion of Coexistence evaluations

5.1.2	PC2 NR V2X UE RF requirements for single carrier

5.1.2.1	Maximum output power for NR V2X UE

The following V2X UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 8.1-1: V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| n47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| NOTE 1: NR Band n47 is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

5.1.2.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.1.2.2-1 and Table 5.1.2.2-2.

Table 5.1.2.2-1: MPR simulation assumption for PC2 NR V2X

| parameter | Assumption |
| --- | --- |
| center frequency | 5.9GHz |
| Bandwidth | 10/20/30/40MHz |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver [31dBc] ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

For PC2 NR V2X, simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.1.2.2-2 can be assumed based on current RAN1’s agreement.

Table 5.1.2.2-2: PC2 V2X UE’ MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216 |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for PC2 V2X UE, RAN4 assumed as follow

Table 5.1.2.2-3: PC2 V2X UE’ MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - 1 RB per user - All users have the same power per RB - Total power of all users equals 26dBm for PC2 - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed - MPR will be derived by non-contiguous PSFCH RB allocation (N>1) - At least, the worst cases with possible RBstart and Ngap need to be checked. ( Ngap = RBend – RBstart ) - For example: The worst case N gap is (106-1 =105*15kHz*12=) 18.9MHz for 20MHz, 15kHz SCS - IMD problem by dual PSFCH in SEM/SE region shall be considered to derive MPR level according to all supporting CBW and SCS. - N (Number of users) is up to 5 and RBs except for RBstart and RBend can be inserted between RBstart and RBend randomly. - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision. |

For S-SSB transmission for PC2 V2X UE, RAN4 assumed as follow

Table 5.1.2.2-4: PC2 V2X UE’ MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | RBstart: All the possible cases LCRB: 11 RB |

<Unchanged sections are omitted>

5.2	Intra-band V2X operation in a licensed band

5.2.1	Intra-band V2X operation scenarios and basic assumptions

5.2.2	Coexistence evaluation

5.2.2.1 Coexistence evaluation scenarios

5.2.2.2 Coexistence simulations assumptions

5.2.2.3 Coexistence results

5.2.2.4 Conclusion of Coexistence evaluations

5.2.3	NR V2X intra-band UE RF requirements for TDM operation

5.2.4	NR V2X intra-band con-current UE RF requirements with adjacent channel for FDM operation

5.2.4.1 Tx requirements for NR V2X intra-band contiguous con-current operation

5.2.4.1.1	Maximum output power

The following NR V2X intra-band contiguous con-current operating UE’ Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 5.2.4.1.1-1: intra-band contiguous con-current V2X UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |  |  |
| V2X_n79B |  |  |  |  | 23 | ±2 |  |  |
| V2X_n79C |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: the intra-band contiguous con-current NR V2X operating band is used for NR V2X and NR Uu Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

5.2.4.1.2	UE maximum output power reduction

The following assumption can serve as a starting point for MPR simulation assumptions as shown in Table 5.2.4.1.2-1 and Table 5.2.4.1.2-2.

Table 5.2.4.1.2-1: MPR simulation assumption for NR V2X intra-band contiguous con-current operation

| parameter | Assumption |
| --- | --- |
| center frequency | 4.5GHz (n79) |
| Bandwidth | 10/20/30/40MHz for NR SL 10/20/40/60/80MHz for NR Uu |
| Maximum output power | 26 dBm |
| numerology | 15 kHz/30kHz/60kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for NR SL CP-OFDM or DFT-S-OFDM for NR Uu |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 45dBc or 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. |

For NR V2X intra-band contiguous con-current operation, simultaneous transmission of PUSCH/PUCCH and NR V2X’s PSCCH and PSSCH in the same sub-frame is supported. The following constraints in Table 5.2.4.1.2-2 can be assumed based on current RAN1’s agreement.

Table 5.2.4.1.2-2: NR V2X intra-band contiguous con-current operation UE’ MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes for NR SL | • Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation for NR SL | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216. No restriction of LCRB for NR Uu |
| Regarding PSCCH / PSSCH multiplexing for NR SL |  |
| PSCCH size for NR SL | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH for NR SL | 0dB |
| NOTE 1: If there is not indicate the detail parameters, then follow TR38.886 for NR SL operation. Also follow TS38.211/TS38.212/TS38.101-1 for NR Uu operation. |  |

For simultaneous transmission of PSFCH transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

For simultaneous transmission of S-SSB transmission for V2X UE and PUSCH/PUCCH for NR Uu, RAN4 need further discussion how to apply the MPR requirements.

<End of Changes>