3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106030

# E-meeting, 12th April – 20th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on General section of NR V2X requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Definition of general section for V2X requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes in comparison to R4-2103976 - Update of Test list for applicability section - Remove “Symbols for all unused REs” from Table 11.1.1.2-1 because OCNG is not required for V2X requiremnets. - Add resource pool configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR V2X requirements are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.1.2, 11.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

# 11	V2X requirements

This clause contains the performance requirements for the sidelink physical channels specified for V2X Sidelink Communication.

## 11.1	Demodulation performance requirements (Conducted requirements)

### 11.1.1	General

#### 11.1.1.1	Applicability of requirements

##### 11.1.1.1.1	General

The minimum performance requirements are applicable to all V2X operating bands defined in TS 38.101-1[6] Clause 5.2E.

The minimum performance requirements in Clause 11.1 are mandatory for UE supporting NR SL operation (sl-Reception-r16), except test cases listed in Clauses 11.1.1.1.2.

##### 11.1.1.1.2	Applicability of requirements for mandatory UE V2X features with capability signalling

The performance requirements in Table 11.1.1.1.2-1 shall apply for V2X UEs which support mandatory UE features with capability signalling only.

Table 11.1.1.1.2-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of synchronization sources for NR sidelink (sync-Sidelink-r16) | FR1 | PSBCH | Clause 11.1.4.1.1 |  |
| Supports of PSFCH format 0 (psfch-FormatZeroSidelink-r16) | FR1 | PSSCH | Clause 11.1.2.1.1 Clause 11.1.6.1.1 Clause 11.1.7.1.1 |  |
|  |  | PSCCH | Clause 11.1.3.1.1 |  |
|  |  | PSFCH | Clause 11.1.5.1.1 Clause 11.1.9.1.1 |  |

#### 11.1.1.2	Common test parameters

Parameters specified in Table 11.1.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 11.1.1.2-1: Common test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| SL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
| PT-RS configuration |  |  | PT-RS is not configured |
| Resource pool configuration | PSCCH Time resource | Symbols | 2 |
|  | PSCCH Frequency resource | PRBs | 10 |
|  | PSFCH number of cyclic shift pairs |  | n1 |
|  | PSFCH hopping ID |  | 0 |
|  | PSFCH candidate resource type |  | allocSubCH |
|  | Set of PRBs for PSFCH transmission |  | ones(1,100) for 40 MHz and ones(1,50) for 20 MHz |
|  | PSSCH RSRP threshold |  | 66 (infinity dBm) |
|  | Synchronization reference |  | GNSS |
|  | Subchannel size | PRBs | 10 |
|  | Number of sub-channels |  | 5 for 20 MHz and 10 for 40 MHz |
|  | Start PRB for first sub-channel |  | 0 |
|  | Time resource bitmap |  | ones(1, 160) |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. |  |  |  |

END OF CHANGE