3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2106295

# Electronic Meeting, 25th Jan- 5th Feb., 2021

Title: 	on HPUE for V2X RF requirements

Source: 	Xiaomi

Agenda item:	8.10.6.1

Document for:	Approval

Background

The HPUE for V2X has been discussed in RAN4#98-e and a WF[1] has been agreed. Also a new suffix G has been introduced for UE supporting TxD has been endorsed in [2]. To further develop the HPUE V2X RF requirement, we give further analysis about the specification structure as well as specific requirements in this paper.

Discussion

During the RAN4#98-e meeting, the WF [1] has been agreed and corresponding HPUE for SL RF requirements are summarized from the WF and shown below as issue 1-4 to issue 1-7:

  - Issue 1-4: TxD for NR V2X

    - Agreement

It is agreed to introduce V2X TxD requirements after the NR Uu TxD requirements are defined, and it will be captured in TS 38.101-1 from Rel-16.

Considering the TxD feature which has been agreed as a new feature in [2], so UL MIMO, V2X, TxD are all UE features with specific requirements defined.

Look back to TS 36.101, the LTE SL with TxD features only consist of below different requirements as:

1, Table 6.2.2G-3: V2X UE Power Class for Transmit Diversity scheme

2, Table 6.2.5G-2: PCMAX,c tolerance in Transmit Diversity scheme for PC2 V2X UE

3, Table 6.2.5G-3: PCMAX,c tolerance in Transmit Diversity scheme for PC3 V2X UE

Similar approach can be followed for NR SL with TxD feature from LTE that to specify: power class and PCMAX,c tolerance for V2X UE with TxD feature.

Furthermore, the UE power class of SL-MIMO has been defined in TS 38.101-1. Hence with the above LTE and NR approach, it is proposed that define specific SL-MIMO and NR SL with TxD requirements respectively under the V2X suffix E.

Proposal 1: To define the SL-MIMO and NR SL with TXD requirements respectively under the V2X suffix E.

For the MPR requirement, it has been agreed in RAN4#97-e the WF[3] that the MPR requirement for TX diversity and UL MIMO should be the same for the same power class. We believe similar agreement can be reached also for V2X UE.

Proposal 2: MPR for V2X TXD and SL-MIMO should be the same.

Conclusions

In this contribution, we give discussion on remaining issue on V2X HPUE and the observations are shown as below：

Proposal 1: To define the SL-MIMO and NR SL with TXD requirements respectively under the V2X suffix E.

Proposal 2: MPR for V2X TXD and SL-MIMO should be the same.

Reference

- R4-2103249	Way forward on issues related to PC2 NR V2X, Huawei

- R4-2103156	CR for TS 38.101-1: TxD requirements, Huawei, HiSilicon

- R4-2016830	Way Forward on NR TxD & Power Class, VIVO