3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106526

Electronic Meeting, Apr. 12 – 20, 2021

Agenda item:	5.7.2.2.2

Source: 	OPPO

Title: 	Simulation results for CSI-RS RSRQ accuracy requirements

Document for:	Discussion

Introduction

In RAN4 #98-e meeting, the impacts of negative and positive timing offset on the CSI-RS based RSRQ measurement were discussed [1] and the simulation assumptions were updated accordingly [2]. This contribution will provide our simulation results for CSI-RS RSRQ based on the following conditions:

- CSI-RS configuration: <X=1,D=3,N=1,NA,noCDM> with bandwidth=48RBs

- Number of samples: 5

- Carrier frequency: 4GHz for FR1 and 30GHz for FR2

- SNR configuration: 6dB for cell 1 and 1dB for cell 2 (the Es/Iot for cell 2 is -5.97dB)

- Propagation condition: AWGN for FR1 and TDL-A with 30ns delay spread for FR2

- Relative delay for cell 2: 0, ±CP/2, ±0.9*CP, ±CP

Discussion

Table 1-3 provide the delta CSI-RS RSRQ of cell 2 for 15kHz, 30kHz and 120kHz subcarrier spacing, respectively. And our observations drawn from the simulation results are given below.

Observation 1: The absolute measurement error of CSI-RS RSRQ with negative timing offset is worse than that of CSI-RS RSRQ with positive timing offset.

Observation 2: The absolute measurement error of CSI-RS RSRQ can be within ±2.0dB for timing offset within [-0.5*CP, +0.5*CP], and ±3.0dB for timing offset within [-CP, +CP].

Observation 3: The relative measurement error of CSI-RS RSRQ can be within ±1.5dB for timing offset within [-0.5*CP, +0.5*CP], and ±2.0dB for timing offset within [-CP, +CP].

Table 1: SCS=15 kHz, AWGN channel @ FR1

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.43 | -0.182 | 0.823 | 1.248 | 1.005 | 1.248 |
| 0.5*CP | -1.518 | -0.236 | 0.722 | 1.282 | 0.958 | 1.282 |
| 0.9*CP | -1.751 | -0.441 | 0.487 | 1.31 | 0.928 | 1.31 |
| CP | -1.66 | -0.429 | 0.557 | 1.231 | 0.986 | 1.231 |
| -0.5*CP | -1.972 | -0.578 | 0.533 | 1.394 | 1.111 | 1.394 |
| -0.9*CP | -2.483 | -0.972 | 0.201 | 1.511 | 1.173 | 1.511 |
| -CP | -2.805 | -1.055 | 0.131 | 1.75 | 1.186 | 1.75 |

Table 2: SCS=30 kHz, AWGN channel @ FR1

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.412 | -0.187 | 0.813 | 1.225 | 1 | 1.225 |
| 0.5*CP | -1.513 | -0.219 | 0.738 | 1.294 | 0.957 | 1.294 |
| 0.9*CP | -1.689 | -0.408 | 0.543 | 1.281 | 0.951 | 1.281 |
| CP | -1.677 | -0.395 | 0.52 | 1.282 | 0.915 | 1.282 |
| -0.5*CP | -1.941 | -0.531 | 0.542 | 1.41 | 1.073 | 1.41 |
| -0.9*CP | -2.451 | -0.985 | 0.195 | 1.466 | 1.18 | 1.466 |
| -CP | -2.626 | -1.072 | 0.085 | 1.554 | 1.157 | 1.554 |

Table 3: SCS=120 kHz, TDL-A channel @ FR2

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.435 | -0.155 | 0.725 | 1.28 | 0.88 | 1.28 |
| 0.5*CP | -1.582 | -0.236 | 0.88 | 1.346 | 1.116 | 1.346 |
| 0.9*CP | -1.692 | -0.419 | 0.476 | 1.273 | 0.895 | 1.273 |
| CP | -1.763 | -0.491 | 0.389 | 1.272 | 0.88 | 1.272 |
| -0.5*CP | -1.588 | -0.379 | 0.468 | 1.209 | 0.847 | 1.209 |
| -0.9*CP | -2.153 | -0.698 | 0.408 | 1.455 | 1.106 | 1.455 |
| -CP | -2.079 | -0.803 | 0.292 | 1.276 | 1.095 | 1.276 |

Conclusion

This paper provides our simulation results for CSI-RS RSRQ measurements and the following observations.

Observation 1: The absolute measurement error of CSI-RS RSRQ with negative timing offset is worse than that of CSI-RS RSRQ with positive timing offset.

Observation 2: The absolute measurement error of CSI-RS RSRQ can be within ±2.0dB for timing offset within [-0.5*CP, +0.5*CP], and ±3.0dB for timing offset within [-CP, +CP].

Observation 3: The relative measurement error of CSI-RS RSRQ can be within ±1.5dB for timing offset within [-0.5*CP, +0.5*CP], and ±2.0dB for timing offset within [-CP, +CP].

- References

- R4-2104089, WF on performance requirements of CSI-RS based L3 measurement, CATT, OPPO

- R4-2103725, “Updated link-level simulation assumptions for CSI-RS based L3 measurements”, CATT