3GPP TSG-RAN WG4 Meeting # 98bis-e                            R4-2106639

Electronic Meeting, 12– 20 April, 2021

Source: 	Huawei, HiSilicon, Telecom Italia

Title: 	TP for TR 37.717-11-11: DC_20_n257

Agenda Item:	7.3.3

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #91e meeting. This contribution provides a TP for TR 37.717-11-11 to finish the UE RF requirements for the band combination.

- References

[1]	RP-210204, “Revised WID: Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL)”, CHTTL

- Text Proposal

<TP for TR 37.717-11-11>

6.2.x	DC_20_n257

6.2.x.1	Configuration for DC

Table 6.2.x.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A_n257A DC_20A_n257B DC_20A_n257C DC_20A_n257D DC_20A_n257E DC_20A_n257F DC_20A_n257G DC_20A_n257H DC_20A_n257I DC_20A_n257J DC_20A_n257K DC_20A_n257L DC_20A_n257M | DC_20A_n257A |

#### 6.2.x.2	Maximum output power for DC

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

#### 6.2.x.3	Spurious emission band UE co-existence for DC

General requirements specified in clause 6.5B.3.4 from TS 38.101-3 apply.

#### 6.2.x.4	MSD analysis for DC

There is no MSD requirement for these band combinations.

#### 6.2.x.5	∆TIB and ∆RIB values

For DC_20_n257, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.2.x.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n257 | 20 | 0 |
|  | n257 | 0 |

Table 6.2.x.5-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n257 | 20 | 0 |
|  | n257 | 0 |

<End of TP >