3GPP TSG-RAN WG4 Meeting # 98bis-e                            R4-2106671

Electronic Meeting, 12– 20 April, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion and draft Reply LS on Half-duplex FDD switching time for RedCap UE

Agenda Item:	12.1

Document for:	Approval

Introduction

In this meeting, RAN4 received the LS [1] on Half-duplex FDD switching time for RedCap UE. The LS and actions are shown as below:

RAN1 discussed the RedCap WI objective on “HD-FDD type A with the minimum specification impact” and agreed the following as a working assumption for RedCap UEs:

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

RAN1 would like to inform RAN4 about the above working assumption and ask for RAN4’s feedback, if any.

For reference, Table 4.3.2-3 in TS 38.211 (V16.4.0) looks like this:

Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx

| Transition time | FR1 | FR2 |
| --- | --- | --- |
| NTx-Rx | 25600 | 13792 |
| NRx-Tx | 25600 | 13792 |

In this paper, we’d like to provide some analysis and feedback to RAN1. A draft reply LS are attached in the Annex.

Discussion

Figure 1 The architecture for HD-FDD RedCap UE

Based on the analysis [2], RAN4 send an LS [3] to RAN1 about the Maximum allowed UE Transition Times from Rx to Tx and from Tx to Rx. It’s specified based on the TAoffset value. For HD-FDD RedCap UE, the possible architecture is shown as figure 1, which is similar with the TDD band architecture. For variable duplex HD-FDD bands (SUL+SDL) n91, n92, n93, n94 and non-simultaneous RxTx SUL band combinations, the possible architecture is shown as figure 2.

Figure 2 The RedCap UE architecture for variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations

Observation 1: The RedCap UE architectures are same among HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations.

Thus, in order to reduce the unnecessary interoperation work, it’s proposed to include these cases, e.g. HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations when RAN4 reply this LS.

Proposal 1: It’s proposed to include these cases, e.g. HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations when RAN4 reply this LS.

Generally, the LOs for Tx and Rx can be set separately. The analysis [2] and TAoffset values can be reused for RedCap UE’s transition time. These cases’ mechanism for UL-DL/DL-UL transition is same with TDD bands. Therefore, RAN4 confirms RAN1’s working assumption about RedCap UE’s transition time for HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations.

Proposal 2: RAN4 confirms RAN1’s working assumption about RedCap UE’s transition time for HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations.

Summary

Based on the analysis and discussion above, all the observations and proposals are listed below:

Observation 1: The RedCap UE architectures are same among HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations.

Proposal 1: It’s proposed to include these cases, e.g. HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations when RAN4 reply this LS.

Proposal 2: RAN4 confirms RAN1’s working assumption about RedCap UE’s transition time for HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations.

- References

- R1-2102146, “LS on Half-duplex FDD switching time for RedCap UE”, RAN1

- R4-1802447, “UE RX/TX Transition Times”, Qualcomm Incorporated

- R4-1805766, LS on UE Rx/Tx Transition Times, RAN4

- Annex

3GPP TSG-RAN WG4 Meeting # 98bis-e                            R4-210xxxx

Electronic Meeting, 12– 20 April, 2021

Title:	Reply LS on Half-duplex FDD switching time for RedCap UE

Response to:	LS R1-2102146 on Half-duplex FDD switching time for RedCap UE

Release:	Release 17

Work Item:	Support of reduced capability NR devices (NR_redcap)

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact person:	Zhang Peng

Zhangpeng169@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

RAN4 thanks RAN1 LS on Half-duplex FDD switching time for RedCap UE. RAN4 has discussed it and made the following agreement and response:

The RedCap UE architectures are same among HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations. Meanwhile, these cases’ mechanism for UL-DL/DL-UL transition is similar with TDD bands. RAN4 confirms that RAN1’s working assumptions about RedCap UE’s transition time are applied for HD-FDD bands, variable duplex HD-FDD bands and non-simultaneous RxTx SUL band combinations.

- 2	Actions

To TSG RAN WG5

ACTION: 	RAN4 respectfully asks RAN5 to take into account the above information in the future.

- 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting#99e 	 	 19th – 27th May 2021 	 Electronic Meeting

TSG-RAN4 Meeting#100e 	 	 23th – 27th August 2021	 Electronic Meeting