3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106739

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson, Bell Mobility

Title:	TP for TR 37.717-21-11: Including band combinations DC_7-13_n25

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_7A-13A_n25A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_7-13_n25

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-13_n25 | CA_7-13 | n25 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-13A_n25A | DC_7A_n25A DC_13A_n25A | CA_7A-13A | n25A |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A | CA_7A-7A-13A | n25A |
| DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A | CA_7C-13A | n25A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_7-13_n25, the TIB,c and RIB,c values are reused from DC_7-28_n2 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-13_n25 DC_7-7-13_n25 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-13_n25 DC_7-7-13_n25 | 7 | 0 |
|  | 13 | 0 |
|  | n25 | 0 |

### 5.1.x.4	REFSENS requirements

There are no IMD impact from UL 7_n25 affecting DL band 13.

There are IMD2 impact from UL 13_n25 affecting DL band 7. The MSD value is derived from DC_7A-28A_n2A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A-13A_n25A DC_7A-7A-13A_n25A DC_7C-13A_n25A | 7 | 2542 | 10 | 50 | 2662 | 27.6 | IMD2 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-202613, Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon