3GPP TSG-RAN WG4 Meeting #98bis-e	R4-2106787

# Electronic Meeting, Apr.12-20,2021

Title: 	Simulation results on NR-U PUSCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.1.4.4.2

Document for:	Discussion

Background

In this paper, we provide our simulation results for PUSCH with interlace structure based on simulation assumptions derived from WF [1].

Simulation results

The summary of simulation results are shown in Table 2-1:

Table 2-1: Simulation results for PUSCH with interlace structure

| Bandwidth (MHz) |  | 20 |  |
| --- | --- | --- | --- |
| SCS (kHz) |  | 15 | 30 |
| SNR@70% of maxTP(dB) | DMRS Type A | 10.31 | 10.42 |
|  | DMRS Type B | 10.29 | 10.38 |

Corresponding simulation assumptions are shown in Table 2-2:

Table 2-2: Simulation assumptions for NR-U PUSCH with interlace structure

| Parameters | Value |
| --- | --- |
| Transform precoding | Disabled |
| Antenna configuration | 1T2R Low |
| SCS and BW | 15kHz: 20MHz; 30kHz: 20MHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30kHz SCS 3D1S1U S=10D:2G:2U for 15kHz SCS |
| Interlace structure | First single interlace per slot |
| DM-RS type | type 1 with single-symbol |
| Number of DMRS symbols | dmrs-AdditionalPosition ‘pos1’ (1+1) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | Type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 |
| Guard band | Not configured |
| PT-RS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV sequence | {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |

Conclusion

In this paper, we provide our simulation results on performance for PUSCH with interlace structure.

Reference

[1]   R4-2103806 Way forward on NR-U BS demodulation requirements for general part and PUSCH, Huawei, HiSilicon