3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106798

# Electronic Meeting, Apr. 12-20, 2021

Title: 	Simulation results on PSCCH requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.2.1.2.2

Document for:	Discussion

Background

At last meeting, simulation assumptions for PSCCH were discussed. In this paper, we provide our simulation results based on simulation assumptions from [1]

Simulation results

Simulation results for PSSCH are shown in Figure 2-1 and Table 2-1. Corresponding simulation assumptions are shown in Table 5-1.

Figure 2-1: Simulation results for PSCCH

Table 2-1 Summary of PSCCH simulation results

|  | SNR@1% BLER |  |
| --- | --- | --- |
|  | Ideal results | Impairment results |
| 26bits | 3.16 | 4.66 |

Conclusion

In this contribution, we provide our simulation results on PSCCH test.

Reference

[1]   R4-2103991 Updated simulation assumptions for V2X single link test cases

Appendix

Table 5-1: Simulation assumption for PSCCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-1400 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 3 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | 26 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 540 |
| Antenna configuration |  | 1x2 Low |
| Performance metric (Note 1) |  | SNR@1% BLER of PSCCH |
| Note 1: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 2: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. |  |  |