3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106805

# Electronic Meeting, Apr.12-20,2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0174 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4: Introduction of PSFCH decoding capability test for NR V2X. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-3-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN4 has agreed to introduce PSFCH decoding capability test for NR V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the requirments and test setup for NR V2X PSFCH decoding capability test Added the related resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for NR V2X will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.8, A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

11.1.9	PSFCH decoding capability test

11.1.9.1	2RX requirements

The purpose of this test is to verify the maximum number of PSFCHs received by UE per slot in group cast scenario by using ACK/NACK feedback mode. In each slot, the tested UE transmits one PSSCH that occupies all the sub channels  to a group of receiving UEs. All slots are divided into two parts. i.e. Slots for PSFCH transmission and slots for DTX occasions. In slots for PSFCH transmission, all receiving UEs transmit related ACKs or NACKs randomly to tested UE and related test parameters are specified in Table 11.1.9.1-1. In slots for DTX occasions, all receiving UEs transmit nothing to the tested UE.

Table 11.1.9.1-1: Test parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| SCS |  | kHz | 30 |
| Resource pool configuration |  |  | Configuration#3-V2X as specified in Table A.7-9 |
| HARQ-ACK information |  |  | ACK or NACK |
| Source ID of tested UE |  |  | 0 |
| Sidelink UE i, 0 ≤ i ≤ N-1(Note 3) | Sidelink transmissions for |  | PSFCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | Member ID(Note 4) |  | i |
|  | PSFCH resource allocation(Note 5) |  | N UEs transmit PSFCHs one by one on each RB with CS pair index 0. i.e. UE 0 transmits PSFCH on RB 0, UE 1 transmits PSFCH on RB 1,…, UE (N-1) transmits PSFCH on RB N-1 |
|  | PSFCH periodicity | Slots | 1 |
| Note 1: Time offset of received signal by Sidelink UE with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: N equals to the number of PSFCH(s) resources that UE can receive in a slot as specified in Clause 4.2.16.1.6 of TS 38.306[14]. Note 4: Member ID is an identifier uniquely identifying a member Note 5: All PSFCHs in a slot are corresponding to one PSSCH that occupies all sub channels. |  |  |  |

11.1.9.1.1	Minimum requirements

11.1.9.1.1.1	DTX to ACK requirements

The DTX to ACK probability, i.e. the probability that ACK is detected when nothing was sent:

ProbPSFCH DTX→Ack bits= #(false ACK bits)#PSFCH DTX

Where:

-	#(false ACK bits) denotes the number of detected ACK bits on DTX occasions.

-	#(PSFCH DTX) denotes the number of DTX occasions on the PSFCH resources as specified in Table 11.1.9.1-1.

The DTX to ACK probability shall not exceed 1%:

ProbPSFCH DTX→Ack bits ≤ 10-2

11.1.5.1.1.1	ACK miss requirements

The ACK missed detection probability is the probability of not detecting an ACK when an ACK was sent.

The ACK miss probability shall not exceed 1%.

<End of change>

<Start of change>

Table A.7-9 V2X sidelink communication resource pool for PSFCH decoding capability test (Configuration #3-V2X)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| SL-ResourcePool-r16 | sl-SyncAllowed-r16 |  | gnss-Sync-r16 |
|  | sl-SubchannelSize-r16 |  | n10 |
|  | sl-TimeResource-r16 |  | 160 |
|  | sl-StartRB-Subchannel-r16 |  | 0 |
|  | sl-NumSubchannel-r16 |  | 10 |
|  | sl-RB-Number-r16 |  | 106 |
|  | sl-PSFCH-Config-r16 | sl-PSFCH-Period-r16 | sl1 |
|  |  | sl-PSFCH-RB-Set-r16 | 100 |
|  |  | sl-NumMuxCS-Pair-r16 | n1 |
|  |  | sl-MinTimeGapPSFCH-r16 | sl2 |
|  |  | sl-PSFCH-HopID-r16 | 0 |
|  |  | sl-PSFCH-CandidateResourceType-r16 | allocSubCH |

<End of change>