3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106945

Electronic Meeting, 12th – 20th April, 2021

Title: 	Discussion on FR2 inter-band UL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.3.7.2

Document for:	Discussion

Introduction

In this contribution, we provide further discussion on the RRM impacts due to FR2 inter-band UL CA.

Discussion

In last RAN4 meeting, the RRM requirements for inter-band UL CA were discussed as below:

- MTTD requirements for inter-band UL CA

- Interruption requirements due to UL carrier RRC reconfiguration

- DL interruption at UE switching between two uplink carriers

- DL interruption at NR SRS carrier based switching

The following agreements have been achieved in [1].

- Issue 1-4-1: How to determine MTTD for CBM? FFS: - Option 1: 3.5 µs on condition of UE capability indication - Option 2: If CBM based FR2 inter-band UL CA would be introduced in Rel-17, then RAN4 needs to study the MTTD requirement applicable for CBM based FR2 inter-band CA, and it is based on the conclusion of MRTD requirements for CBM UE. - Issue 1-5-1: How to determine MTTD in case of IBM? - Agreement: For IBM capable UE, the Rel16 MTTD requirements for FR2 inter-band CA can be applied in Rel-17 and no additional discussion is required in Rel17. - Issue 2-1-1: interruption requirements due to UL carrier RRC reconfiguration FFS: - Option 1: The existing interruption and delay requirements for UL carrier RRC reconfiguration can be applied when new inter-band UL CA configurations are introduced - Option 2: More discussion is needed. - Option 3: RRM discussion should be hold until we have conclusion of FR2 inter-band UL CA in RF session. - Issue 2-1-2: DL interruption at UE switching between two uplink carriers FFS: - Option 1: The existing interruption requirements for UE switching between two uplink carriers are not applicable for FR2 inter-band UL CA - Option 2: More discussion is needed. - Option 3: RRM discussion should be hold until we have conclusion of FR2 inter-band UL CA in RF session. - Issue 2-1-3: DL interruption at NR SRS carrier based switching FFS: - Option 1: RAN4 investigates the interruption requirements for NR SRS carrier based switching applicable for inter-band SRS carrier switching in FR2. - Option 2: More discussion is needed. - Option 3: RRM discussion should be hold until we have conclusion of FR2 inter-band UL CA in RF session.

According to the objects of NR RF Enhancements for FR2 WID, RAN4 need to define requirements for FR2 inter-band CA based on IBM. Since the feasibility of FR2 inter-band CA based on CBM is still under discussion in RF session, it is suggested to start the discussion in RRM session after the feasibility is confirmed in RF session.

Proposal 1: It is suggested to start the discussion on RRM requirements for FR2 inter-band CA based on CBM after the feasibility is confirmed in RF session.

In this section, RRM requirements for FR2 inter-band UL CA are discussed based on IBM type UE.

MTTD for inter-band NR CA

For IBM type UE, it has been agreed that 8.5us MTTD requirement for FR2 inter-band UL CA defined in Rel-16 can be applied in Rel-17.

UL carrier RRC reconfiguration delay and interruption

In R15/R16, the interruption due to UL carrier RRC reconfiguration is allowed on the serving cells within the same FR as the reconfigured UL carrier. The interruption length requirements depends on the assumption of RF implementation. For IBM UE, the UE assumptions in R17 follow the agreements in R16. The maximum UL carrier RRC reconfiguration delay is defined as the maximum RRC procedure delay which is independent of CA configurations. Hence, the UL carrier RRC reconfiguration delay requirements in Rel-17 shall be same as in Rel-16.

Proposal 2: For IBM type UE, the existing interruption and delay requirements for UL carrier RRC reconfiguration in Rel-16 can be applied in Rel-17.

DL Interruptions at UE switching between two uplink carriers

The interruption requirements for UE switching between two uplink carriers are applied for an uplink band pair of an inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, where one uplink carrier capable of one transmit antenna connector and another uplink carrier capable of two transmit antenna connectors. It is common understanding that carriers with one or more transmit antenna connectors only belong to FR1. The capability uplinkTxSwitchingPeriod is also FR1-only. Hence, the existing interruption requirements for UE switching between two uplink carriers are only applied for FR1 inter-band UL CA.

Proposal 3: The Rel-16 interruption requirement for UE switching between two uplink carriers can be applied in Rel-17 since it is only applicable in FR1. There is no impact due to introducing FR2 inter-band UL CA.

DL interruptions at NR SRS carrier based switching

In TS38.133, the existing interruption requirements for NR SRS carrier based switching are defined to allow interruptions on active serving cells, which are applicable for intra-band SRS carrier switching in FR1, intra-band SRS carrier switching in FR2, inter-band SRS carrier switching in FR1 and inter-band SRS carrier switching between FR1 and FR2. For FR2 inter-band UL CA, the carrier of SCell not configured for PUCCH/PUSCH transmission to/from which SRS carrier based switching is in FR2, and the interrupted serving cell is also in FR2. The aggressor cell and the victim cell can be on FR2 intra-band carriers or on FR2 inter-band carriers. However, the case of inter-band SRS carrier switching in FR2 is missing for the current NR SRS carrier based switching requirements. Hence, RAN4 needs to investigate the interruption time for inter-band SRS carrier switching in FR2.

Proposal 4: RAN4 investigates the interruption requirements for NR SRS carrier based switching applicable for inter-band SRS carrier switching in FR2.

Conclusions

This contribution provides the discussion on FR2 inter-band UL CA enhancement. The following are provided:

Proposal 1: It is suggested to start the discussion on RRM requirements for FR2 inter-band CA based on CBM after the feasibility is confirmed in RF session.

Proposal 2: For IBM type UE, the existing interruption and delay requirements for UL carrier RRC reconfiguration in Rel-16 can be applied in Rel-17.

Proposal 3: The Rel-16 interruption requirement for UE switching between two uplink carriers can be applied in Rel-17 since it is only applicable in FR1. There is no impact due to introducing FR2 inter-band UL CA.

Proposal 4: RAN4 investigates the interruption requirements for NR SRS carrier based switching applicable for inter-band SRS carrier switching in FR2.

Reference

- R4-2104095, “WF on RRM requirements for FR2 Inter-band DL CA and UL CA”, Nokia, Nokia Shanghai Bell