3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2107003

Electronic Meeting, 12 – 20 April, 2021

Title: 	Discussion on CSSF, measurement capability and MG for PRS measurement

Source: 	Huawei, HiSilicon

Agenda item:	5.5.1.4

Document for:	Discussion

Introduction

The generic RRM requirements for UE PRS measurements have been discussed in RAN4#98-e, and the conclusions are captured in the WF [1]. Based on our understanding, there are still some remaining open issues:

- CSSF

  - How to determine a PFL is a candidate for a MG occasion

  - Selection of one PFL for CSSF calculation

  - CSSF for RRM measurements on a carrier frequency with PRS measurements

  - Definition of long periodicity measurement

  - Determination of parameter Ri

- Applicability condition considering UE capability

- Use of MG pattern #24 and #25 for LTE RRM measurement

- Terminology

In this paper we will provide our views on the remaining generic open issues on UE PRS measurement requirements.

Discussion

CSSF

Candidate PFL for a MG occasion

- FFS Whether a PLF is considered as candidate for a MG occasion when part but not all resources on that PFL are within the MGL of the MG occasion - Option 1 - PFL is counted as candidate for a MG occasion if at least one PRS resource on that PFL is fully covered by the MGL excluding RF switching time. - Option 2 - Yes, a PFL is counted as candidate for a MG occasion if a sufficient number of PRS symbols are contained within MGL excluding RF switching time

UE is required to measure all the configured PRS resources subject to its measurement capability. In this sense, as long as one PRS resource on that PFL is fully covered by the MGL excluding RF switching time, UE would have to measure the PFL, which means the PFL would be a candidate for this MG occasion.

Proposal 1: A PFL is counted as candidate for a MG occasion if at least one PRS resource on that PFL is fully covered by the MGL excluding RF switching time.

Selection of one PFL for CSSF calculation

- FFS for the sum-based approach, Selection of one PFL in CSSF calculation for PRS measurement period - Option 1 - Only one PRS frequency layer with a short periodicity would compete for MG with other gap-based RRM measurements at a time - Selection of the one PRS frequency layer for measurement is up to UE implementation - Option 2 - For PFLs that do not satisfy the long periodicity condition, CSSF would be calculated by counting only one PFL at a time. - For RRM frequency layers, N intermediate CSSF values would be calculated, when N is the number of PFLs and each intermediate CSSF value accounts for only one of the PFLs. FFS: The CSSF value for a RRM frequency layer could be the highest among the N intermediate CSSF values or chosen depending on with PFL is being processed at the time. - Option 3 - CSSF should be defined on per MG occasion basis, i.e. a PRS frequency layer is counted as candidate for a MG occasion if at least one PRS resource occasion is fully covered by the MGL excluding RF switching time. - Count only a single PLF for each MG occasion.

It was agreed in RAN4#98-e that when multiple PFLs are measured, the total measurement period is the sum of measurement periods of each individual PFL, and correspondingly, for CSSF calculation, only one PFL is counted per MG occasion. The remaining open issue is which PFL is assumed.

In our view, there is no need to specify the selection of a PFL for a MG occasion. For each MG occasion, the total number of candidates is same no matter which PFL is selected. We understand that there might be a difference in the final CSSF depending on UE is assumed to measure PLF#1 or PFL#2 for a time period, but as the final CSSF is calculated as the max number of candidates among all MG occasions, the difference would be rather small and occur in limited scenarios.

On the other hand, the requirements should not restrict UE to do parallel or sequential measurement for multiple PFLs, nor restrict UE to measure any particular PFL for a MG occasion. Therefore, we suggest that the CSSF is calculated in Rel-15 approach, and any PFL is counted as a candidate for a MG occasion as long as it fulfils the condition in Proposal 1.

Proposal 2: CSSF is derived in Rel-15 approach, and any PFL is counted as a candidate for a MG occasion as long as it fulfils the condition in Proposal 1.

CSSF for RRM measurements on a carrier frequency with PRS measurements

- FFS For the sum-based approach, CSSF for RRM measurements on a carrier frequency with PRS measurements - Option 1: Rel-15 approach

This issue is not fully clear to us. In our view, RRM and PRS measurement are always performed on different frequency layers, i.e. there is no “RRM measurements on a carrier frequency with PRS measurements”.

Proposal 3: PRS frequency layer and SSB frequency layer are always handled as separated frequency layers in CSSF calculation.

Definition of long periodicity measurement

- FFS: Definition of long periodicity measurement - Option 1 - Tavailable,i≥320ms . - Option 2 - max(Tprs * X * dl-prs-MutingBitRepetitionFactor) >=320ms, where X is the length of NR-MutingPattern-r16 for mutingOption1-r16r - Option 3 - LCMover all kTPRS, mutingk≥160 ms where k is the PRS resource set index within a PFL, TPRS, mutingk=TPRSk∙Xk∙dl-PRS-MultiBitRepetitionFactork+1, TPRSk is the PRS resource periodicity and Xk is the number of consecutive zeros in NR-MutingPattern-r16 for mutingOption1-r16 - Option 4 PRS periodicity (ms); DL-PRS-MutingPattern configuration. >160; With or without muting. 160; With muting.

We support option 1 as the criterion for defining long periodicity measurement.

- Option 2 and option 3 are accounting for resource muting explicitly. However, even if muting would be considered in the PRS measurement requirements (which we support), it should have already been accounted in Tavailable,i, so no need to separately define the muting impact in CSSF.

- Option 4 is not complete, e.g. what happens to PRS periodicity < 160ms. Also, muting option 2 should have no impact on the periodicity of the available resources.

In RAN4#98-e, one concern was raised by some companies, which is about the case where a PLF is categorized as long periodicity measurement while there are resources with small periodicity. This is shown in Figure 1, where PRS resource #1 is configured with 320ms period and offset 0, PRS resource #2 with 80ms period and offset 40ms, and PRS resource #3 with 160ms period and offset 80ms.

Figure 1: Example of PRS resources with different periodicities and offsets

As long periodicity is defined on per PFL not per resource level, having different resource periodicities for the same frequency layer will complicate the CSSF design, especially when some resources are considered for long periodicity measurement while others are not.

- If UE measures all resources of this PFL as long periodicity, then this PFL would take many MG occasions due to resource #2 and #3, and this is not aligned with Rel-15 CSSF principle that only long periodicity measurement is prioritized.

- If UE measures only resource #1 of this PFL as long periodicity, it will have no opportunity to measure resource #2 or resource #3.

Therefore, we suggest measurement requirements apply provided that the resource periodicities after muting are either <= 160ms for all PRS resources on the PFL, or > 160ms for all PRS resources on the PFL.

Proposal 4a: Measurement of PFL i is defined as long periodicity measurement if Tavailable,i ≥320ms.

Proposal 4b: Measurement requirements apply provided that the resource periodicities after muting are either <= 160ms for all PRS resources on the PFL, or > 160ms for all PRS resources on the PFL.

Determination of parameter Ri

- As for counting the number of actually available MGs for short-periodicity PRS layer i (the denominator of Ri), the candidate MG #j should be excluded under the following conditions: - Case-1: when MG #j is within the processing time of any long-periodicity PRS in another MG #j-n, as illustrated in Figure 1, or - Case-2: when any long-periodicity PRS in another MG #j+n is within the processing time of PRS layer i in MG #j, as illustrated in Figure 2, or - Case-3: when MG #j contains any long-periodicity PRS, which is already captured in the spec above

In CSSF calculation, there is scaling factor Ri which represents the loss of MG occasions for frequency layer i due to long periodicity measurement. In other words, it is derived based on the number of MG occasions not to be taken by long periodicity measurement.

In RAN4#98-e, some companies proposed above to determine the MG occasions to be taken by long periodicity measurement. Case-1 and case-2 consider the MG occasions that fall in the processing time of a long periodicity measurement. In our view, they are not necessary since RAN4 has agreed that when multiple PFLs are measured, the total measurement period is the sum of measurement periods of each individual PFL, which means when defining the CSSF for a PFL, there is no need to consider other PFLs.

Proposal 5a: An MG occasion is assumed to be taken by long periodicity measurement if any PFL categorized as long periodicity measurement is a candidate for this MG occasion.

Proposal 5b: No other MG occasion is assumed to be taken by long periodicity measurement.

Applicability condition considering UE capability

Time span of PRS resource instance > N

- FFS: time span of PRS resource instance > N - Option 1 - The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance is greater than UE reported capability N - Option 2 - at least UE should be able to measure one PRS resource without repetition - Option 3 - No clarification is needed

We support option 1, as an instance of a single PRS resource should be buffered and processed by UE with one PRS occasion/period, and coherent combining of a PRS resource across more than one PRS occasions/periods is not typical UE implementation.

Option 2 is a bit unclear to us. UE should measure each configured PRS based on the configuration, and that means UE should measure all the repetitions of a resource if configured. Option 2 seems to indicate that UE can determine how many repetitions of a resource it would measure, but should at least be able to measure a PRS resource without repetition. To us this is a new UE behaviour and requirement, and we do not think it can be accommodated in Rel-16.

Option 3 does not address the UE buffering and processing capability issue.

Time span of PRS resource instance > MGL

- FFS: time span of PRS resource instance > MGL - Option 1 - The measurement requirements do not apply for a PRS resource, if the time span of a DL PRS resource instance is greater than the configured measurement gap length - Option 2 - When time span of PRS resource instance > MGL, the PRS within the useful part of MG can be assumed - Option 3 - The requirements apply , provided a sufficient number of PRS symbols are available within MGL excluding the RF switching time. The PRS symbols beyond MGL are not considered for PRS measurements

We support option 1 for the same reason as mentioned in section 2.2.1.

Option 2 and option 3 are assuming that UE can choose to measure part of symbols for a PRS resource. To us this is a new UE behaviour and requirement, and we do not think it can be accommodated in Rel-16.

Time span of PRS resource instance being across two sampling duration of N within duration Lprs

- FFS: time span of PRS resource instance being across two sampling duration of N within duration Lprs - Option 1 - The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lprs - Option 2 - UE may need to combine the PRS resource in the two sampling periods, or overlapped sampling window can be used if the issue exists - Option 3 - No clarification is needed

The issue is shown in Figure 2. The duration of each single resource is no larger than N, but the aggregate duration Lprs is larger than N. In this case, the measurement requirement would apply for resource 1 and resource 3, and the scaling factor in the measurement period LPRS,iN is 2.

Figure 2: sampling when configured PRS occasion duration K is larger than reported capability N

As resource 2 is across two sampling duration of N, it cannot be measured in one PRS sample. For the same reason that an instance of a single PRS resource should be buffered and processed by UE with one PRS occasion/period, the measurement requirement cannot apply for this resource, either. Therefore, we support option 1.

Option 2 and option 3 are assuming UE to coherently combine sampling data across two PRS period for one resource, which is not typical UE implementation.

General applicability due to UE processing and measurement limitations

- General applicability due to UE processing and measurement limitations - Option 1 - The measurement requirements do not apply to any instance of a PRS resource that cannot be measured and processed in its entirety due to limitations imposed by either the UE PRS processing capability {N, T} or the configured measurement gap pattern. - Option 2 - No further clarification is needed

Based on the analysis in section 2.2.1 ~ 2.2.3, option 1 is a good generic principle which can cover all 3 cases, so we also support option 1. However, for the spec we prefer to capture the exact cases to avoid any potential ambiguity.

Proposal 6: The measurement requirements do not apply for a PRS resource if

- the time span of the PRS resource instance is greater than UE reported capability N, or

- the time span of a DL PRS resource instance is greater than the configured measurement gap length, or

- the PRS resource is across two sampling duration of N within duration Lprs

MG pattern #24 and #25 for LTE RRM measurement

Use of MG pattern #25 for LTE

- FFS whether MG pattern #25 is an applicable pattern for LTE measurements - Option 1 - Remove MG pattern #25 as an applicable pattern for LTE measurement - Option 2 - No change is needed to the current specification

The MG applicability is defined in Table 9.1.2-2 and Table 9.1.2-3 of 38.133. For legacy MG patterns, i.e. #0~#23, the applicability for LTE measurement are defined as follows.

- MG pattern #0,1,2,3 can be used for LTE measurement in all cases

- MG pattern #4,6,7,8,10 can be used for LTE measurement when the configured MO includes botj LTE and NR target carriers

- MG pattern #5,9,11 cannot be used for LTE measurement (reason is that they are with 160ms MGRP while for LTE we do not have requirements defined with 160ms MGRP)

Following the applicability for legacy MG patterns, MG pattern #25, which is also with 160ms MGRP, should be applicable for LTE measurement.

In RAN4#98-e, some companies concerned that removing MG pattern #25 for LTE measurement would mean the pattern is practically unusable. In our view, when NW configures LTE RRM measurement then this pattern cannot be used, but it can still be used if the RRM measurements are only on NR carriers. This is also same as MG pattern #5,9,11.

Proposal 7: Remove MG pattern #25 as an applicable pattern for LTE measurement.

Measurement window

- FFS Measurement window when MG pattern #24 is used for LTE measurement - Option 1 - When MG pattern #24 is used for LTE measurement, the measurement window is defined as the first 5ms after the RF re-tuning time - Option 2 - no need to limit this to 5 ms

In previous meetings, we proposed to define the 5ms measurement window when MG pattern #24 with 10ms MGL is used for LTE measurement. The main motivation is to make sure that UE is not required to measure the whole 9ms effective time, but we realize that this is not required by existing requirements.

It is noted that the measurement window is defined in 36.133 for MG pattern #2,3,10 and #6,7,8. The reason is that these MG patterns have MGL <5ms, so NW and UE need to align where the LTE PSS/SSS should be located. However, this is not necessary for MG pattern #24 since there will always be a PSS/SSS occasion within the 9ms effective time.

Based on above analysis, we think there is no need to limit the measurement window for MG pattern #24 when used for LTE measurement.

Proposal 8: No need to limit the measurement window for MG pattern #24 when used for LTE measurement.

Terminology

- FFS which term should be used in the specification - Option 1 - Positioning frequency layer - Option 2 - PRS frequency layer - Option 3 - Frequency layer (with NR PRS measurement) - Other options not precluded - Companies are also encouraged to check specifications of other WGs

In our view, option 1, i.e. positioning frequency layer, is a well defined term which are also used by RAN1 and RAN2, e.g. following are excerpts from 38.214 and 37.355. RAN4 should use the same term.

5.1.6.5 PRS reception procedure The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer. The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource. A positioning frequency layer is configured by NR-DL-PRS-PositioningFrequencyLayer, consists of one or more DL PRS resource sets and it is defined by: - dl-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing. The supported values of dl-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211].

nr-DL-PRS-PositioningFrequencyLayer This field specifies the Positioning Frequency Layer for the nr-DL-PRS-AssistanceDataPerFreq field.. nr-DL-PRS-AssistanceDataPerFreq This field specifies the DL-PRS Resources for the TRPs within the Positioning Frequency Layer.. … dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.. dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer.. dl-PRS-PointA This field specifies the absolute frequency of the reference resource block for the DL-PRS. Its lowest subcarrier is also known as DL-PRS Point A. A single DL-PRS Point A for DL-PRS Resource allocation is provided per Positioning Frequency Layer. All DL-PRS Resources belonging to the same DL-PRS Resource Set have the same DL-PRS Point A.. dl-PRS-CombSizeN This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size N..

Proposal 9: Adopt the term “Positioning frequency layer” in RAN4 spec.

Conclusions

In this paper we provided our views on the remaining generic open issues on UE PRS measurement requirements.

Proposal 1: A PFL is counted as candidate for a MG occasion if at least one PRS resource on that PFL is fully covered by the MGL excluding RF switching time.

Proposal 2: CSSF is derived in Rel-15 approach, and any PFL is counted as a candidate for a MG occasion as long as it fulfils the condition in Proposal 1.

Proposal 3: PRS frequency layer and SSB frequency layer are always handled as separated frequency layers in CSSF calculation.

Proposal 4a: Measurement of PFL i is defined as long periodicity measurement if Tavailable,i ≥320ms.

Proposal 4b: Measurement requirements apply provided that the resource periodicities after muting are either <= 160ms for all PRS resources on the PFL, or > 160ms for all PRS resources on the PFL.

Proposal 5a: An MG occasion is assumed to be taken by long periodicity measurement if any PFL categorized as long periodicity measurement is a candidate for this MG occasion.

Proposal 5b: No other MG occasion is assumed to be taken by long periodicity measurement.

Proposal 6: The measurement requirements do not apply for a PRS resource if

- the time span of the PRS resource instance is greater than UE reported capability N, or

- the time span of a DL PRS resource instance is greater than the configured measurement gap length, or

- the PRS resource is across two sampling duration of N within duration Lprs

Proposal 7: Remove MG pattern #25 as an applicable pattern for LTE measurement.

Proposal 8: No need to limit the measurement window for MG pattern #24 when used for LTE measurement.

Proposal 9: Adopt the term “Positioning frequency layer” in RAN4 spec.

Reference

- R4-2104076, WF on UE PRS measurement requirements, Huawei, HiSilicon