3GPP TSG-RAN WG4 #98bis-e	R4-2107211

e-Meeting, Apr 12 – 20, 2020

Agenda item:	8.12.4.1

Source: 	Qualcomm Incorporated

Title: 	60 GHz UE TX EVM

Document for:	Discussion

# Scope

In this paper we present TX EVM measured on a sample 60 GHz transmitter chain for a UE.

# EVM measurement

We performed measurements on a transmitter chain for UE 60 GHz design. The result measured is for a single chain. That single chain would be used to drive an antenna element in the antenna array.

The measurement was done with a 400 MHz CP-OFDM signal at 60.5 GHz and 120 SCS subcarrier spacing. Phase noise degradation is not included in the measurement result as the LO was provided by a very low phase noise signal generator.

We can make a comparison between this measurement of a typical (as opposed to min performance) transmitter and the FR2 EVM spec. The 16QAM FR2 spec is 12.5%. From the figure 12.5% is ~ -6 dBm.

Note that -6 dBm TX power would result in approximately 18.5 dBm EIRP in the best direction … boresight. Note this is based on this typical measurement and an assumption of 16 transmitter chains driving 16 antenna elements and including losses.

# Conclusion

We have provided EVM measurement for 400 MHz 16-QAM CP-OFDM.