3GPP TSG-RAN WG4 Meeting RAN4#98-bis-e				R4-2107369 Online

April 12 – April 20, 2021

Agenda Item:	5.8.1.1

Source:	Lenovo, Motorola Mobility

Title:	On the EVM Definition for Transmit Diversity and Single Layer Transmission

Document for:  Approval

- Introduction

In the way forward [1] from RAN4#95-e, there was agreement on the EVM definition for transparent transmit diversity.  However, the relationship between the proposed EVM definition and the resulting noise floor at the gNB receiver is not clear. Since the purpose of the EVM requirement is to set a lower bound on the link performance due to transmitter impairments, this relationship must be understood. For multi-antenna transmission, the EVM should be defined as

EVM=100∙1SNR

where SNR is measured at the output of an ideal (noiseless) receiver.

For single antenna transmission and reception, the EVM at the transmit antenna connector and the EVM at the output of the single antenna gNB receiver are the same for any 1x1 propagation channel h since the gNB receiver can simply invert the channel using a zero-forcing equalizer.  Note that a zero-forcing equalizer is used in the definition of EVM for a single antenna transmitter in Appendix F of both TS 36.101 and TS 38.101. Note also that no receiver noise is considered in the definition of EVM for single antenna transmission.

When multiple transmit antennas are used for transparent transmit diversity or for the transmission of a single MIMO layer, the relationship between the EVM values at the UE antenna connectors and the resulting lower bound on the noise floor at the output of a noiseless gNB receiver must be evaluated in order to properly define EVM for the link.  In order to perform this evaluation, it is necessary to assume a particular receiver type. Furthermore, the EVM definition should have the following properties:

- The EVM definition should correspond to the link SNR that can be achieved at a noiseless receiver

- The EVM definition should be traceable to a receiver implementation to ensure that the corresponding link SNR is achievable

- The EVM definition should be independent of the propagation channel between the transmitter and the receiver

In previous contributions [2-5], the EVM for transparent transmit diversity and single layer-transmission was evaluated for both the linear unbiased MMSE receiver and the a zero-forcing receiver.  Based on a review of these results, we propose that the zero-forcing receiver be used to define the EVM for transparent transmit diversity and for single layer transmission using multiple antennas as in Proposal 2 from [5].

- Alternative Receiver Types for Defining EVM

# For this case in which there is transmitter noise but no receiver noise, there seem to be only two receiver types for which the SNR at the output of the receiver is independent of the 2x2 channel H between the transmitter and the receiver and these are i) the linear unbiased MMSE receiver, and ii) a zero-forcing receiver in which the channel is inverted.  It should be noted that the normalized spatial matched filter (the pseudo-inverse) cannot be used since the resulting EVM definition would be channel dependent.

# Linear Unbiased MMSE Receiver

In [2], the signal-to-noise ratio was evaluated for an antenna port comprised of two antennas for the case of a noiseless gNB receiver having two receive antennas and employing an unbiased linear MMSE receiver.  The resulting EVM definition was shown to be independent of the 2x2 propagation channel H between the transmitter and receiver so long as the channel H is invertible.

In [3], it was shown that the signal-to-noise ratio at the output of the linear unbiased MMSE receiver is given by

SNR= 11HΣ'-111

where

Σ'=n' n'H

and n' is the vector of the transmitter noise measured by the test equipment.  From this signal-to-noise ratio, the corresponding EVM is given as

EVM=100∙ 11HΣ'-111-12

There were concerns expressed with this proposal with respect to the ability of the test equipment to measure the covariance of the transmitter noise Σ'.  Subsequently in [3] it was shown that in the worst-case in which the transmitter noise is completely correlated so that there is no gain from noise averaging, the EVM is given by

EVM=minEVM1,EVM2.

Concerns were expressed over this EVM definition because it is independent of the power transmitted on the two antennas.

The Zero-Forcing Receiver

For single antenna transmission, a linear zero-forcing equalizer is used to define and measure EVM.  Similarly, for uplink MIMO, it has been proposed that a linear zero-forcing MIMO receiver be used to define and measure EVM.  For this reason, it seems reasonable to consider using a zero-forcing receiver to define EVM for transparent transmit diversity.  However, there are some technical problems with this approach because the zero-forcing receiver is not uniquely defined when two transmit and two receive antennas are used for single layer transmission and reception. For example, the receiver could throw away the output of the second receive antenna and use a zero-forcing equalizer on the first antenna, or alternatively, throw away the output of the first receive antenna and use a zero-forcing equalizer on the second antenna. Either of these approaches would yield an unbiased estimate of the data symbol and thus would technically be “zero-forcing.”

In [4], we proposed that a particular zero-forcing receiver be used to define EVM for transparent transmit diversity and for single layer transmission from two antennas in which the propagation channel H is inverted. The signal-to-noise ratio and the corresponding EVM for this receiver are derived in the Appendix. For this receiver, the signal-to-noise ratio at the output of the receiver is given by

SNR=w04σ12+w14σ22+2 w02w12Reε-1.

The corresponding EVM is

EVM=100 w04σ12+w14σ22+2 w02w12Reε

where the covariance of the transmitter noise is given by

Σ'=En'n'H=σ12εε*σ22

If it is assumed that the transmitter noise is completely correlated (the worst case), then it can be shown that

EVM≤P1 ∙EVM1+P2 ∙EVM2P1+P2

where EVM1 and EVM2 denote the EVM values measured at the first and second antenna connectors and P1 and P2 denote the power measured at the first and second antenna connectors.

It can be noted that this last EVM definition is conservative in two respects:

- the transmitter noise is assumed to be completely correlated with ρ=1 (the worst case);

- the EVM will always be less for an unbiased MMSE receiver than for a zero-forcing receiver since

minEVM1, EVM2≤P1 ∙EVM1+P2 ∙EVM2P1+P2

As a final observation, it should be noted that the zero-forcing receiver used in the Appendix to derive the EVM cannot actually be implemented by the test equipment or by the gNB receiver since there will be no per antenna reference symbols transmitted with transparent transmit diversity. However, as noted above, the performance of the linear unbiased MMSE receiver is always superior to that of the zero-forcing receiver and the linear unbiased MMSE can be implemented without per antenna reference symbols

Proposal:  The EVM for transparent transmit diversity is defined as

EVM=P1 ∙EVM1+P2 ∙EVM2P1+P2

where EVM1 and EVM2 denote the EVM measured at the first and second antenna connectors and P1 and P2 denote the power measured at the first and second antenna connectors.

The EVM definition in the Way Forward [1] is given by

EVMWF=(P1∙EVM12+ P2∙EVM22)/(P1+P2)

In Figure 1 below, the ratio of the proposed EVM to EVMWF is shown as a function of the fraction of the total power transmitted from the first antenna and the ratio β=EVM2EVM1 of the EVM for the second antenna to the EVM of the first antenna. From the figure, it can be observed that the value of the proposed EVM is always less than value of the EVM in the way forward.

Figure 1: Ratio (linear) of proposed EVM to EVMWF

- Summary

As noted in the introduction, the EVM definition for transparent transmit diversity should have the following properties:

- The EVM definition should correspond to the link SNR that can be achieved at a noiseless receiver

- The EVM definition should be traceable to a receiver implementation to ensure that the corresponding link SNR is achievable

- The EVM definition should be independent of the propagation channel between the transmitter and the receiver

Additionally, to maintain consistency with the EVM definition for single antenna transmission and with the proposed use of a zero-forcing equalizer to define EVM for uplink MIMO, a zero-forcing receiver should be used to define EVM for transparent transmit diversity as in the following proposal.

Proposal:  The EVM for transparent transmit diversity is defined as

EVM=P1 ∙EVM1+P2 ∙EVM2P1+P2

where EVM1 and EVM2 denote the EVM measured at the first and second antenna connectors and P1 and P2 denote the power measured at the first and second antenna connectors.

It can be noted that this definition is conservative in that it assumes that the transmitter noise is fully correlated (worst case) and also because the receiver the performance of the unbiased linear MMSE receive is always superior to that of the the zero-forcing receiver used to derive the proposed EVM definition.

# References

- R4-2008465, ‘WF on Enabling Transparent TxD in Rel-16’, Qualcomm, 3GPP TSG-RAN WG4 Meeting #95-e.

- R4-2011519, ‘Further Considerations on the EVM Definition for Antenna Ports Including Transparent Transmit Diversity’, Lenovo, Motorola Mobility,3GPP TSG-RAN WG4 Meeting #96-e.

- R4-2016288, ‘On the EVM Definition for Transmit Diversity,’ Lenovo, Motorola Mobility, 3GPP TSG-RAN WG4 Meeting #97-e.

- R4-2008276, ‘Considerations on the EVM Definition for an Antenna Port or a Single MIMO Layer’, Motorola, 3GPP TSG-RAN WG4 Meeting #95-e.

- R4-2102917, ‘On the EVM Definition for Transmit Diversity and Antenna Ports’, Lenovo, Motorola Mobility, 3GPP TSG-RAN WG4 Meeting #98-e.

Appendix:  EVM Evaluation for a Zero-Forcing Receiver

We consider a “zero-forcing” receiver in which the propagation channel H is inverted. For a two-layer MIMO transmission, the received signal is given by

y=HW x+n

where the 2x1 vector y denotes the received signal, the 2x2 matrix H denotes the propagation channel, the two columns of the 2x2 matrix W denote the precoding vectors for the two layers, the 2x1 vector x denotes the data symbols, and the 2x1 vector n denotes the transmitter noise. If per-layer reference symbols (DMRS) are transmitted along with the data, then the gNB uses these to estimate the product of the channel and the precoding matrix HW.  With the estimate HW, the zero-forcing MIMO receiver is given by

x= HW-1y .

For single layer transmission, the transmitted signal is given by

y=H w  x+n

where w is the 2x1 precoding vector for the single layer, and x denotes the transmitted data symbol. If only per-layer DMRS are transmitted for channel estimation, then the receiver can only estimate the product Hw, and thus it is not possible for the receiver to “zero-force” by inverting the propagation channel H.

We consider the case in which per antenna reference symbols are transmitted so that H can be estimated.  However, it should be noted that for transparent transmit diversity, per antenna reference symbols will not be transmitted.  However, the signal-to-noise ratio can still be computed for this receiver, even if it cannot be implemented.  With per-antenna reference symbols, the receiver can estimate the data as

wH H-1 y=wH H-1H w x+n≈x+wH n= x+v,

where w can be estimated as

w=H-1H w .

Since v=wHn , we have

Ev2=wHEn nH w=wH  Σ w

=wHw100w2Σ'w100w2H w ,

where the matrix

w100w2

is used to scale from the transmitter noise n' measured by the test equipment to the transmitter noise n at the input to the test equipment.

In order to evaluate Ev2, let

Σ'=En'n'H=σ12εε*σ22 ,

where

EVM1=100σ12    and   EVM1=100σ22  .

We then have

Ev2=wHw000w1σ12εε*σ22w000w1Hw

=w04σ12+w14σ22+2 w02w12Reε ,

The EVM is then given by

EVM=100 Ev2=100 w04σ12+w14σ22+2 w02w12Reε

=w04 EVM12+w14 EVM22+2 ∙104 w02w12 Reε

≤w04 EVM12+w14 EVM22+2 ∙104 w02w12 ε .

Let the correlation coefficient be defined as
ρ=εσ1σ2

where ρ≤1, and let P1 and P2 denote the power measured by the test equipment on the first and second antenna.  With these definitions, we have

EVM≤w04 EVM12+w14 EVM22+2 ρ w02w12 EVM1EVM2

If the transmitter noise is completely correlated so that ρ=1, then we have

EVM≤w02 EVM1+w12 EVM2

Noting that

w02+w12=1,

and furthermore, that

w02=P1P1+P2     and      w12=P2P1+P2 ,

we have

EVM≤1P1+P2 P12 EVM12+P22 EVM22+2 ρ P1 P2 EVM1EVM2

The worst case EVM occurs when ρ=1 and is given by

EVMwc=P1 ∙EVM1+P2 ∙EVM2P1+P2 .