3GPP TSG-RAN WG4 Meeting # 99-e                               R4-2107815

Electronic Meeting, 19– 27 May, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A_47A

Agenda Item:	8.26.2

Document for:	Approval

Introduction

In last meeting, companies discussed the band combinations V2X_n79A-n47A and V2X_n79A_47A. And MSD due to cross band isolation may be observed between these two bands. In this paper, we’d like to further analysis the MSD issue for these band combinations.

Discussion

For V2X_n79A-n47A and V2X_n79A_47A, the Tx leakage from band n79 may fall into Rx of band n47/47. We use antenna topology as RF front-end architecture in our analysis, as shown in figure 1. A separate antenna architecture is assumed for these two band combinations. Table 1 summarize the MSD analysis results for these band combinations.

Figure 1 The RF front-end architecture

Table 1 MSD calculation for victim Rx of band n47

| Item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n79 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| n79 PA output noise power at B41 freq | dBm/Hz | 40MHz UL | -121 | -121 |
| Antenna isolation Primary <-> Diversity |  | 10 |  |  |
| n41 Rx bandwidth | MHz | 9 |  |  |
| n79 PA output noise power |  |  | -51.45 | -51.45 |
| n79 SAW TX to COM port on n47 RX freq | dB | 30 |  |  |
| n79 PA noise power at ant port |  |  | -91.45 | -91.45 |
| noise figure dB | dB | 13 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -161 |  |  |
| Thermal noise, dBm |  |  | -91.45 | -91.45 |
| totle noise level at ANT port, dBm |  |  | -88.44 | -88.44 |
| SNR requirement for QPSK |  | -0.5 |  |  |
| REFSENSE (referred to antenna)(10MHz BW) |  |  | -88.94 | -88.94 |
| combined REFSENS(10MHz BW), dBm |  |  | -91.95 |  |
| Margin | dB | 2.5 |  |  |
| combined REFSENS(10MHz BW)after considering Margin, dBm |  |  | -89.45 |  |
| MSD | dB |  | 3.04 |  |

- Text Proposal

---Start of changes1---

6.2.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below.

Table 6.2.4.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

The REFSENS exception for band n79 from the aggressor n47 is FFS.

---End of changes1---

---Start of changes2---

6.3.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below.

Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X

| UL band | DL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | 47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

The REFSENS exception for band n79 from the aggressor 47 is FFS.

---End of changes2---